3GPP TSG-RAN WG4 Meeting #112	R4-2413138

# Maastricht, Netherlands, 19 – 23 August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2482 | rev | - | Current version: | 17.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (Power_Limit_CA_DC-Core) CR to TS 38.101-1: DL CA HPUE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Power_Limit_CA_DC-Core |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN4 specifications include notes for band combinations where HPUE operation is applicable. This results in lower output power even if UE has the necessary HW to support higher output power, but specification process is not fully complete yet. This CR enables using HPUE for inter-band CA with single CC UL configured also for cases when HPUE applicability note is not present. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new general clause to CA to specify HPUE for interband CA with single UL CC configured, including how this is indicated to network. Add an exception to refsens to test exceptions with power level indicated by powerClass for interband CA with single CC UL when UE has indicated higher power class than powerClass for single UL CC using ue-PowerClassPerBandPerBC-r17. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | HPUE is not possible by UE indication for 1UL CC with DL CA for cases where HPUE note is not present in clause 5.5A. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.0 (new clause), 7.3A.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change 1>

## 6.2A	Transmitter power for CA

### 6.2A.0	General

Maximum output power for inter-band CA with single configured UL CC is not bounded by the power class for the band combination indicated by powerClass and specified in clause 6.2A.1. powerClass is ignored in setting the PCMAX_H for inter-band CA with single CC UL when UE indicates higher power class than powerClass for a band using IE ue-PowerClassPerBandPerBC-r17. In such scenario the PC2 and PC1.5 applicability note in clause 5.5A is not required to be in place and UE shall meet the requirements for power class indicated for the band by ue-PowerClassPerBandPerBC-r17.

NOTE:	The non-CA and CA configurations for which minimum requirements have been evaluated for Power Class 2 or Power Class 1.5 operation are indicated in clause 5.5A.

### 6.2A.1	UE maximum output power for CA

<End of change 1>

<Start of change 2>

#### 7.3A.2.3	Reference sensitivity power level for Inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 with parameters specified in  Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, Table 7.3.2-3, and in Table 7.3F.2-1, Table 7.3F.2-2, Table 7.3F.2-3 for inter-band CA with one shared spectrum channel access band, modified in accordance with clause 7.3A.3.2. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active. Exceptions to reference sensitivity are allowed in accordance with clause 7.3A.4, 7.3A.5 and 7.3A.6. When UE indicates higher power class than powerClass for a band combination of a single UL CC and DL CA configuration using ue-PowerClassPerBandPerBC-r17, and this higher power class is not applicable according to clause 5.5A.3, reference sensitivity including exceptions in accordance with clause 7.3A.4 and 7.3A.6 shall be tested with output power limited to power class indicated by powerClass.

For the combination of intra-band and inter-band carrier aggregation, the intra-band CA relaxation, ΔRIBC and ΔRIBNC, are also applied according to the clause 7.3A.2.1 and 7.3A.2.2.

#### 7.3A.2.4	Void

<End of change 2>