# 3GPP TSG-RAN4 Meeting #112	R4-2413140

# Maastricht, Netherlands, August 19 – 23, 2024

Agenda item:		10.2

Source: 		Qualcomm Incorporated

Title: 		Support for HPUE with DL CA with single UL CC configured

Document for:		Approval

## 1	Introduction

As of today, RAN4 specifications explicitly indicate when HPUE is applicable for a band, as well as when HPUE is applicable to be used in CA configuration.

UEs having HPUE(=>26dBm) hardware support need to reduce power back to PC3(23dBm) in CA operation in case the HPUE basket work has not been completed. This may happen in DL CA with single UL carrier when simply configuring a second DL carrier can result in an output power drop of at least 3dB. This is illustrated in Figure 1.

Figure 1: In case HPUE is not explicitly specified for CA, UE needs to reduce output power and leave it’s HW capabilities unused.

## 2	Discussion

Given that the issue exists already in closed releases, the best approach is to give an allowance for UE to exceed CA power class, and allow them to indicate higher output power up to the level supported for single carrier operation. When formulated as an allowance UE can take full advantage of the higher output power capability, but also, the divergence from existing RAN4 HPUE introduction as well as regular UE power class indication is minimized this way.

Network needs to be made aware of UE power capabilities. Existing UE capability ue-PowerClassPerBandPerBC can be used to indicate maximum power for each individual band in a CA configuration. Therefore, UE can use this capability to unambiguously inform network of the used maximum output power without impact to single carrier (capability ue-PowerClass) or CA (capability powerClass).

Proposal 1: Allow UE to maintain HPUE maximum transmit power capability in inter-band DL CA with single configured UL carrier even when HPUE requirements are not yet completed for the CA configuration. Allow UE to indicate its power class for single uplink CC in the DL CA configuration without restrictions to network using ue-PowerClassPerBandPerBC.

One of the debated aspects in RAN4 have been the relationship of this allowance and MSD specifications, i.e. potential UE receiver desensitization due to higher output power. MSD requirements are based on an artificial worst case reference point to set minimum UE RF performance based on a specific channel bandwidth, center frequency and RB allocation, while in field operation different parameters can be used. It is rare that such worst case spectrum allocation is held by an operator and on top of that worst case scheduling occurs. The specifications should not penalize the performance of all the CA combinations due to some corner cases.

If there is still a concern on DL performance, network has means to limit the maximum output power used by configuring Pmax on the CC transmitting UL

Therefore, it is sufficient to maintain the existing MSD requirements as a reference point and specify that if HPUE requirements are not yet defined for the higher power classes, MSD shall be verified at default power class power level (e.g PC3).

Alternative approach to MSD would be to define formulas which derive PC2 MSD based from PC3 MSD. This approach is not preferred as it might completely miss scenarios where e.g. cross-band isolation MSD is not defined for PC3 but could be needed for PC2.

Proposal 2: If HPUE requirements for single UL with DL CA are not specified, MSD is verified at default power class output power level.

Envisioned changes to RAN4 specifications are provided for reference in Annex.

## 3	Conclusion

Based on current RAN4 specifications UEs are not allowed to utilize their real maximum output power capability but instead need to artificially reduce output power back to default power class even when only single UL carrier with DL CA is used. This results in reduced maximum output power and therefore lost coverage and UL performance. Following proposals are made to allow UEs to take advantage of their higher output power capability even if the 3GPP process for the CA configuration is not yet fully completed.

Proposal 1: Allow UE to maintain HPUE maximum transmit power capability in inter-band DL CA with single configured UL carrier even when HPUE requirements are not yet completed for the CA configuration. Allow UE to indicate its power class for single uplink CC in the DL CA configuration without restrictions to network using ue-PowerClassPerBandPerBC.

Proposal 2: If HPUE requirements for single UL with DL CA are not specified, MSD is verified at default power class output power level.

Changes to RAN4 specifications are provided in companion CR.