3GPP TSG-RAN WG4 Meeting #112	R4-2413141

# Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:		8.25.2.2

Source: 		Qualcomm Inc.

Title: 		NR NTN UL capacity enhancements

Document for:		Approval

## 1	Introduction

Work item for the third phase of NR NTN has been approved in [1]. The work item includes RAN4 for multiple objectives, including UL capacity/cell throughput enhancement for FR1-NTN, with a sub-objective to update RF requirements accordingly, if needed. In this contribution RAN4 RF impact is discussion

## 2	Discussion

RAN1 has already been working on the work item for a few meetings and reached initial agreements on the scope of the capacity enhancements. Relevant agreements from RAN1#117 have been reproduced below:

Agreement

For the normative phase, at least one of the OCC techniques will be specified:

- Inter-slot time-domain OCC with PUSCH repetition Type A with OCC length 2 or 4

- Inter-symbol(s) time domain OCC with OCC length 2 or 4

- Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 or 4

- FFS Combination of OCC techniques including multiplexing of 8 UEs

- FFS Use of OCC techniques with TBoMS

- FFS Backward compatibility with non-Rel-19 UEs

Furthermore, the work item itself rules out enhancements to PRACH and PUSCH DMRS, therefore it is sufficient to focus on PUSCH.

Proposal 1: RAN4 needs to evaluate whether there is any requirement impact from the orthogonal cover code (OCC) schemes.

The basic operating principle of symbol- and slot-level orthogonal cover code (OCC) schemes are that UE data symbols are repeated in consecutive symbols or slots multiplied by the OCC code. This allows placing multiple UEs on the same time-frequency resource, as SAN receiver can still differentiate the UEs thanks to the OCC. This is equivalent to receiver seeing a linear combination of the UE signals (e.g. sig1 + sig2 in symbol#1 and sig1 - sig2 in symbol#2) and solving for sig1 and sig2. This is illustrated in Figure 1. Capacity enhancement is taking place because multiple UEs can use the same subcarrier, while without the OCC scheme only single user would occupy the subcarrier all the time even in the case where the amount of data being sent would be minuscule.

Figure 1 Symbol-level OCC for two UEs

Adding the symbol- or slot-level OCC scheme as part of the signal generation in UE transmitter does not alter the signal characteristics. That is, e.g. peak-to-average ratio stays unchanged if the symbol is multiplied by a constant. Applying the OCC can be considered equivalent to changing the order of the constellation points. Using BPSK as an example for simplicity, if the original data is mapped to symbols +1 +1, after the repetition and applying OCC of [1, -1] the outcome is +1 -1 +1 -1. This is no different from having different random data from the start. Therefore, it can be concluded that the symbol- and slot level OCC schemes do not impact waveform characteristics, and therefore they do not impact UE emissions either.

Given that the waveform characteristics do not change, it can be also considered that signal quality (EVM) is sufficiently tested with current requirements.

Proposal 2: There is no UE RF requirement impact from symbol- or slot-level OCC schemes.

The third OCC-scheme under discussion is a comb-based OCC applied to subcarrier, which effectively distributes the data the non-contiguous subcarriers with constant frequency spacing, as shown in Figure 2.

Figure 2: pre-DFT-s OCC for 2 UEs

This scheme enables capacity enhancement by allowing to schedule multiple UEs within the same resource block, instead of reserving the whole resource block for a single UE. As seen in Figure 2, the OCC coefficients keep the total power constant, i.e. transmitted PSD is boosted for the active subcarriers.

The waveform type is similar to PUCCH format 4, which also constitutes from a frequency domain comb. Due to the similarity it is expected that requirement impacts would be small, if there would be any.

Observation 1: Waveform from pre-DFT-s OCC-scheme is similar to PUCCH format 4, and therefore specification impact is expected to be minor, if any at all.

Initial evaluations were conducted, and so far we have observed that placing the highest PSD 3-subcarrier comb transmission at the worst possible position from emissions perspective, i.e. at the outer edge of an RB placed at the outer edge (RB#0) of the channel, and SEM mask was met in this case. 6-subcarrier comb resulted in same outcome.

Observation 2: UE can meet SEM mask with MPR specified in TS 38.101-5 with 3-subcarrier and 6-subcarrier comb-transmissions at worst position at outer edge of RF channel (RB#0)

## 3	Conclusion

In this contribution NR NTN UL capacity enhancements were discussed. Following observations and proposals were made

Proposal 1: RAN4 needs to evaluate whether there is any requirement impact from the orthogonal cover code (OCC) schemes.

Proposal 2: There is no UE RF requirement impact from symbol- or slot-level OCC schemes.

Observation 1: Waveform from pre-DFT-s OCC-scheme is similar to PUCCH format 4, and therefore specification impact is expected to be minor, if any at all.

Observation 2: UE can meet SEM mask with MPR specified in TS 38.101-5 with 3-subcarrier and 6-subcarrier comb-transmissions at worst position at outer edge of RF channel (RB#0)

## 4	References

[1]  RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT