3GPP TSG-RAN WG4 Meeting #112	R4-2413143

# Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:		8.26.2

Source: 		Qualcomm Inc.

Title: 		NTN IoT UL capacity enhancements

Document for:		Approval

## 1	Introduction

Work item for the third phase of NTN IoT has been approved in [1]. The work item includes RAN4 for the objective to support capacity enhancements for uplink, with a sub-objective to update RF requirements accordingly, if needed. In this contribution RAN4 RF impact is discussion

## 2	Discussion

When the work item description is examined in detail, it can be observed that UE RF specification is not included in the list of impacted specifications. Therefore, it is not fully clear whether the discussion should be fully focused on potential impact on SAN, or also UE RF is considered. SAN RF specification 36.108 is listed in the WI, but only for performance part. In this contribution we have considered UE RF.

Observation 1: UE RF specification 36.102 is not listed as an impacted specification in the WID, neither for core nor performance part

RAN1 has already been working on the work item for a few meetings and reached initial agreements on the scope of the capacity enhancements. Relevant agreements from RAN1#117 have been reproduced below:

Agreement

For 3.75kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued.

For 15kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued.

Furthermore, RAN1 has also discussed potential updates on NPRACH, but no conclusions have been made. Currently it is unclear whether any enhancement to NPRACH will be made and RAN4 should not work to study NPRACH before RAN1 work has progressed.

Proposal 1: RAN4 shall not work on NPRACH before further progress in RAN1 is reached.

Therefore, the discussion can be focused on NPUSCH.

The basic operating principle of symbol- and slot-level orthogonal cover code (OCC) schemes are that UE data symbols are repeated in consecutive symbols or slots multiplied by the OCC code. This allows placing multiple UEs on the same time-frequency resource, as SAN receiver can still differentiate the UEs thanks to the OCC. This is equivalent to receiver seeing a linear combination of the UE signals (e.g. sig1 + sig2 in symbol#1 and sig1 - sig2 in symbol#2) and solving for sig1 and sig2. This is illustrated in Figure 1. Capacity enhancement is taking place because multiple UEs can use the same subcarrier, while without the OCC scheme only single user would occupy the subcarrier all the time even in the case where the amount of data being sent would be minuscule.

Figure 1 Symbol-level OCC for two UEs

Adding the symbol- or slot-level OCC scheme as part of the signal generation in UE transmitter does not alter the signal characteristics. That is, e.g. peak-to-average ratio stays unchanged if the symbol is multiplied by a constant. Applying the OCC can be considered equivalent to changing the order of the constellation points. Using BPSK as an example for simplicity, if the original data is mapped to symbols +1 +1, after the repetition and applying OCC of [1, -1] the outcome is +1 -1 +1 -1. This is no different from having different random data from the start. Therefore, it can be concluded that the symbol- and slot level OCC schemes do not impact waveform characteristics, and therefore they do not impact UE emissions either.

Given that the waveform characteristics do not change, it can be also considered that signal quality (EVM) is sufficiently tested with current requirements.

Proposal 2: There is no UE RF requirement impact from symbol- or slot-level OCC schemes for NPUSCH

## 3	Conclusion

In this contribution NTN IoT UL capacity enhancements were discussed. Following observations and proposals were made

Observation 1: UE RF specification 36.102 is not listed as an impacted specification in the WID, neither for core nor performance part

Proposal 1: RAN4 shall not work on NPRACH before further progress in RAN1 is reached.

Proposal 2: There is no UE RF requirement impact from symbol- or slot-level OCC schemes for NPUSCH

## 4	References

[1]  	RP-241624, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc. (Rapporteur)