3GPP TSG-RAN WG4 Meeting #112	R4-2413144

# Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:		8.8.2.2

Source: 		Qualcomm Inc.

Title: 		Support for NTN high power UE

Document for:		Approval

## 1	Introduction

The work item for enhanced requirements and conductive test methodology for NR NTN and IoT NTN has been approved [1]. One of the objectives is high power UE operation for NTN. In this contribution required work and specification impacts other than the co-existence study are discussed.

## 2	Discussion

One of the most important aspects of any new UE RF work is to understand what constraints are being set by regulatory requirements. The work item justification describes in high level, that EU and US regulations typically allow higher output powers than what 3GPP is looking at. SAR requirements have been identified needing a solution similar to TN.

In addition to these, work item objectives also explicitly list MPR and A-MPR for example bands as well as Rx RSD as necessary requirements.

Observation 1: Based on WID discussions on SAR, MPR and A-MPR and RSD are expected.

In order to do MPR and A-MPR evaluations it is necessary to consider regulatory requirements. During rel-17 and rel-18 there have been some gaps in this domain, but higher UE output power makes it even more important to address necessary regulatory requirements.

In Europe, harmonized standards are the primary reference document for bringing devices to market. During the work in rel-17 and rel-18, there have been LS exchange between 3GPP and ETSI TC SES, i.e. the technical committee with ETSI which is responsible to satellite connectivity. In RAN4#108bis, RAN4 received a reply LS from ETSI TC SES confirming that 3GPP NTN technology have not been considered in TC SES and updates may be needed.

In another LS [3] received in this meeting, it is mentioned that ETSI EN 301 681 – covering frequencies aligned with bands 255 and 253 – considers equipment which is typically for professional and governmental use with possible safety of life or critical mission requirements. This gives a background on why some requirements in harmonized standard are very stringent.

On the other hand, the reply in [2] also states that harmonized standards are based on CEPT regulatory framework or ITU recommendations.

The discussion to capture 3GPP NTN and necessary European regulatory requirements in ETSI TC SES have been started, and TC SES meeting in June 2024 discussed approval of two new work items for creation of new harmonized standards: one for NR NTN and another or IoT NTN. These WIs are not yet officially approved, and reaching a status of stable draft is expected to take place in mid 2025.

The timeline is challenging from 3GPP perspective, as time will be running out of rel-19 to perform proper studies considering appropriate regulatory requirements in ECC/CEPT. Therefore, 3GPP should do their own study on the regulatory requirements. Harmonized standards, and especially the originally regulatory sources from where the requirements in harmonized standards may be originating would need to be studied.

Completing the co-existence study in RAN4 will take few meetings, and conclusions for ACLR requirements will not be available before that. Therefore, now is proper time to study all regulatory background, so MPR and A-MPR studies can then consider all necessary requirements.

Proposal 1: RAN4 should study applicable regulatory requirements in ECC/CEPT region and take those requirements into account in MPR and A-MPR evaluations.

Once the study is complete, corrections to PC3 requirements in 36.102 and 38.101-5 should be made too.

Proposal 2: Based on the study outcome, necessary corrections should be made also for PC3 in 36.102 and 38.101-5

Despite the work item listing example bands only as (n)256 and (n)255, it would be useful to consider all 3GPP bands in the regulatory study. The summary of ETSI harmonized standards is provided in Table 1, but this may not be exhaustive.

Table 1: Examples of ETSI harmonized standards

|  | GSO |  | NGSO |  |
| --- | --- | --- | --- | --- |
|  | IoT NTN | NR NTN | IoT NTN | NR NTN |
| 256 (S-band): | EN 302 574-2 (Cat M1) EN 302 574-3 (NB-IoT) | EN 302 574-2 | EN 301 442 |  |
| 255 (L-band) | EN 301 681 |  |  |  |
| 254 L/S-band | EN 301 441 |  |  |  |
| 253 Extended L-band | EN 301 681 |  |  |  |

This may result in 3GPP considering requirements which will not in the end be necessary to be fulfilled in Europe, but it is much easier to remove unnecessary requirements from 3GPP than to backfill new requirements into 3GPP while hardware implementations may already be developing or developed.

Proposal 3: In case ETSI TC SES identifies later that some requirements are not required for market access in Europe, those requirements should be removed from 3GPP.

While NR TN requirements can provide good guidelines on impacts of HPUE requirements, especially NB-IoT is missing corresponding good reference as only up to PC3 operation has been considered. NB-IoT is fundamentally very different from NR due to narrow transmission bandwidth and half-duplex operation, and therefore learnings from NR may not be directly applicable to NB-IoT. Additionally, many NB-NTN requirements refer to 36.101, where requirements have been considered only up to PC3. Here we have provided examples of specification items which would need updates.

- Clause 6.2B.4 in 36.102 for configured max output power refers to 36.101, and 36.101 includes Pcmax tolerances only up to PC3

- IQ-image and Carrier leakage for NB-IoT are currently specified as -25 dBc. With higher output power, IMD between e.g. wanted signal and IQ-image could be driving A-MPR requirements. It should be considered whether -25 dBc is still appropriate to use even up to PC1 in MPR and A-MPR evaluations

- NB-IoT spectrum emission mask is capped to 26 dBm and could need updates for operation at higher than PC3 power levels.

Proposal 4: For NB-NTN, discussion on at least Pcmax tolerance, IQ-image and DC-leakage and spectrum emission mask are needed

Out of these, we took a closer look at NB-IoT spectrum emission mask. Main decision on NB-IoT spectrum mask were made during RAN4#78bis. From the documents back then [4], [5], it can be seen that multiple options both for the mask shape as well as offset to spurious emissions domain were considered. Candidates were GSM-like and E-UTRA like masks. Finally, a conclusion was made to adopt what we see in 36.101 today, which was directly copied over to 36.102. The only difference is that in 36.101 there is an additional requirement to meet also E-UTRA SEM mask after a specific frequency offset, but this is intended for guard-band operation which is not in scope for NTN.

The key observation from the background is that NB-IoT SEM mask was decided within 3GPP, and there were no regulatory emission limits driving the selection.

The main concerns on directly applying the current SEM mask for HPUE operation is that the emissions limits have been set as absolute limits and the top end is set at 26 dBm. Now if UE Tx output power increases from 23 to 31 dBm, the leakage level at only 10 kHz offset from closest subcarrier may be problematic. This is not only the case for fully allocated RB, but e.g. for transmitting single subcarrier at channel edge. Therefore, it is necessary to evaluate whether SEM mask, shown in Table 2, would need updates.

Table 2: Category NB1 and NB2 UE spectrum emission mask (from 36.102)

| ΔfOOB (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0 | 26 | 30 kHz |
| 100 | -5 | 30 kHz |
| 150 | -8 | 30 kHz |
| 300 | -29 | 30 kHz |
| 500-1700 | -35 | 30 kHz |

Finally, when it comes to UE RF architecture, NR TN HPUE conclusions need further consideration. For NR, PC2 is allowed to use dual Tx (tx diversity) implementation, but PC1.5 is specified to always apply dual Tx. For NB-NTN dual Tx operation may not be necessary for high output powers, though it still is one possible option for implementation. For NTN UL-MIMO has not been considered, as UL-MIMO typically needs good SNR to give benefits. For NB-IoT and cat M1 UL-MIMO has never been specified.

Also the signal characteristics are very different between NR-NTN and NB-NTN, as e.g. NB-IoT transmission bandwidth is only up to 180 kHz. This gives opportunities to take advantage of GSM-like high power PAs, which can be able to drive high power signal from single Tx.

Proposal 5: At least for NB-NTN HPUE, single Tx operation should not be ruled out from higher power classes.

## 3	Conclusion

In this NTN HPUE operation was discussed. Following observations and proposals were made

Observation 1: Based on WID discussions on SAR, MPR and A-MPR and RSD are expected.

Proposal 1: RAN4 should study applicable regulatory requirements in ECC/CEPT region and take those requirements into account in MPR and A-MPR evaluations.

Proposal 2: Based on the study outcome, necessary corrections should be made also for PC3 in 36.102 and 38.101-5

Proposal 3: In case ETSI TC SES identifies later that some requirements are not required for market access in Europe, those requirements should be removed from 3GPP.

Proposal 4: For NB-NTN, discussion on at least Pcmax tolerance, IQ-image and DC-leakage and spectrum emission mask are needed

Proposal 5: At least for NB-NTN HPUE, single Tx operation should not be ruled out from higher power classes.

## 4	References

[1]  RP-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung

[2]  R4-2317932, LS reply on 3GPP NTN requirements and potential impact on specification work in ETSI TC SES, ETSI TC SES

[3]  R4-2411017, Blocking requirements for extended L-band, ETSI TC SES

[4]  R4-162807, NB-IoT SEM study, Nokia

[5]  R4-160616, On UE emissions requirement for NB-IoT, Huawei