3GPP TSG-RAN WG4 Meeting #112	R4-2413145

# Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:		8.8.3.1

Source: 		Qualcomm Inc.

Title: 		System parameters and synchronization raster for NTN less than 5 MHz

Document for:		Approval

## 1	Introduction

3 MHz channel bandwidth was specified for TN in rel-18. In the work item approved for NTN, 3 MHz channel bandwidth is to be added with RAN1-design based on rel-18, with no expected RAN1 impact. For synchronization raster the work item description guides to leverage to work on less than 5 MHz for TN. In this contribution synchronization raster for 3 MHz channel bandwidth for NTN is discussed.

## 2	Discussion

2.1 Spectrum utilization

Spectrum utilization was studied for TN, and it was found that 16 PRB in 3 MHz channel bandwidth would result in difficulties to meet the SEM at channel edge, resulting to power backoff. Based on this, 15 PRB was specified to be used in 3 MHz channel. Given the importance of link budget in NTN operation, it is strongly suggested to maintain 15 PRB spectrum utilization also in NTN operation.

Proposal 1: Aligned with TN, 15 RB spectrum utilization for 3 MHz channel bandwidth is used.

2.2 Synchronization raster

The problematics with adding 3 MHz channel bandwidth for NTN are equivalent to 3 MHz for TN: existing sync raster is too sparse to allow using all RF channel positions and therefore new design is needed. The RAN1 design from rel-18 is based on puncturing the SSB to 12 RB bandwidth. The solution from TN specification is provided in Table 1 and 2, and illustrated in Figure 1.

Table 1: GSCN parameters for the global frequency raster for above 3 MHz channel bandwidth

| Frequency range | SS Block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 MHz | N * 1200kHz + M * 50 kHz, N=1:2499, M ϵ {1,3,5} 1 | 3N + (M-3)/2 | 22 – 7498 |
| 3000 – 24250 MHz | 3000 MHz + N * 1.44 MHz N = 0:14756 | 7499 + N | 7499 – 22255 |
| NOTE 1: The default value for operating bands with which only support SCS spaced channel raster(s) is M=3. NOTE 2: GSCN=2 (corresponding to ARFCN-ValueNR = 250) is a reserved value paired with reserved operating band n200. |  |  |  |

Table 2: GSCN parameters for the global frequency for 3 MHz channel bandwidth

| Range of frequencies (MHz) | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 1000 | N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:1665, M ϵ {1,3,5} (Note 1) | 26638+3N + (M-3)/2 | 26640 – 31634 |
| NOTE 1: Only applicable for 15 PRB transmission bandwidth configuration within 3 MHz channel bandwidth with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1. |  |  |  |

Figure 1: Visualization of sync raster for above 5 MHz and 3 MHz channel bandwidths.

In addition to the raster above, special design were made to accommodate GSM-R to NR migration scenario in band n100. Such special accommodations are not needed for NTN. Another evolution from rel-18 is specification of enhanced channel raster, bringing the RF channel step size to 10 kHz. When enhanced channel raster was specified, sync raster was not considered. Same principle should be maintained and design should be based only 100 kHz channel raster.

Proposal 2: NTN sync raster design does not address enhanced channel raster nor other than 15 PRB transmission bandwidth configuration.

Given that changes in RAN1 side are out of scope, no meaningful changes to raster design can be done. However, it also seems there is no motivation to come up with different design for NTN. The TN design can be extended to apply at higher frequencies, and it will enable using 3 MHz channel bandwidth at every possible RF channel position in 100 kHz raster. Furthermore, it keeps the benefit for UE that raster points also identify the SSB bandwidth: when UE detects a raster point intended for 3 MHz, it can identify that 12 RB SSB is being transmitted. Therefore, it would be sufficient to re-use the raster design made for TN, and only adjust the frequency range and range of GSCN to match with frequency range of NTN bands.

Proposal 3: Re-use the TN sync raster design for NTN 3 MHz channel bandwidth, only by adjusting the range of frequencies and GSCN values to NTN band frequency range, e.g. 1500 to 3000 MHz.

## 3	Conclusion

In this contribution system parameters and synchronization raster for less than 5 MHz for NTN were discussed. Following proposals were made

Proposal 1: Aligned with TN, 15 RB spectrum utilization for 3 MHz channel bandwidth is used.

Proposal 2: NTN sync raster design does not address enhanced channel raster nor other than 15 PRB transmission bandwidth configuration.

Proposal 3: Re-use the TN sync raster design for NTN 3 MHz channel bandwidth, only by adjusting the range of frequencies and GSCN values to NTN band frequency range, e.g. 1500 to 3000 MHz.

## 4	References

[1]  RP-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung