3GPP TSG-RAN WG4 Meeting #112	R4-2413147

# Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:		7.17.2

Source: 		Qualcomm Inc.

Title: 		Co-existence considerations for the new S-band

Document for:		Approval

## 1	Introduction

A new WI on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) [1]. The objectives include co-existence analysis, for example UE-UE co-existence, for adjacent band per RAN4 scope. In this contribution these co-existence aspects are discussed.

## 2	Discussion

There are a number of overlapping and adjacent 3GPP operating bands which could be relevant for the co-existence work. These are shown in Table 1.

Table 1: Operating bands close to new S-band frequencies

| n2 | 1850 MHz – 1910 MHz | 1930 MHz – 1990 MHz | FDD |
| --- | --- | --- | --- |
| n25 | 1850 MHz – 1915 MHz | 1930 MHz – 1995 MHz | FDD |
| n34 | 2010 MHz – 2025 MHz | 2010 MHz – 2025 MHz | TDD |
| n39 | 1880 MHz – 1920 MHz | 1880 MHz – 1920 MHz | TDD |
| n65 | 1920 MHz – 2010 MHz | 2110 MHz – 2200 MHz | FDD4 |
| n66 | 1710 MHz – 1780 MHz | 2110 MHz – 2200 MHz | FDD |
| n70 | 1695 MHz – 1710 MHz | 1995 MHz – 2020 MHz | FDD |

Still, not all of these are necessary to be considered for co-existence with the new S-band. The work item justification clarifies that the new band is intended for North America, therefore the focus can be on bands n2, n25, n66 and n70. Furthermore, it would not be necessary to evaluate overlapping bands n66 and n70. For these bands DL frequencies are overlapping with DL frequencies of the new S-band. In general, 3GPP RAN4 cannot effectively address overlapping DL frequencies, any emission requirements would preclude usage of the band.

It should be also noted that work item description specifically discusses working on adjacent band co-existence, not working on overlapping bands.

When it comes to adjacent band co-existence with bands n66 and n70, NTN to TN interference would be lesser than from another TN network due to the separation distance to SAN transmitter. Therefore, no special requirements are expected to address bands n66 and n70.

This leaves bands n2 and n25. New S-band UL at 2000-2020 MHz has only 5 MHz guard band towards n25 DL and 10 MHz guard band to n2 DL. It is necessary to consider UE-to-UE co-existence with DL of n2 and n25.

Proposal 1: Co-existence work regarding co-existence with other 3GPP bands can be focused on NTN UE Tx interference to DL of n2 and n25.

Often -50 dBm/MHz is considered as the baseline for the UE-to-UE co-existence requirements, and it is widely used in both TN and NTN UE RF specifications. It should not however automatically be the correct requirement to be applied and there are a lot of examples of also other choices. For example, very recently -30 dBm/MHz was set to protect n5 and n26 DL from n106 UL. Then there also cases between n1 and n39 as well as n7 and n38 where a stepwise mask has been defined. This is shown in Table 2, where n1 UE Tx requirements right below the low edge of band n1 at 1920 MHz are shown. These requirements comes together with restriction on RB allocation to maximum 54 RBs.

Table 2: n1 (UE TX 1920-1980 MHz) protection requirements towards n39 (TDD, 1880-1920 MHz).

| Frequency range | 1880 | - | 1895 | -40 |
| --- | --- | --- | --- | --- |
| Frequency range | 1895 | - | 1915 | -15.5 |
| Frequency range | 1915 | - | 1920 | +1.6 |

At the same time, there are examples also towards very stringent protection requirements, e.g. NS_24 at -50 dBm/MHz is defined for n256. A-MPR values for NS_24 reach to up to 20 dB, which clearly makes closing the NTN UL link extremely challenging.

Furthermore, NTN co-existence studies, including the new NTN HPUE work [2], are made applying a separation distance between NTN UEs and TN network. This separation distance could enable setting more relaxed co-existence requirements.

Still, while defining the requirements, it is also important to ensure that the chosen requirements do not cause excessive interference towards n2 and n25 DL. For these reasons, further analysis and discussion are needed to set appropriate UE-to-UE coexistence limits which provide protection to n2 and n25 DL, but at the same time are not excessively stringent.

Observation 1: 3GPP has multiple examples of less stringent than -50 dBm/MHz UE-to-UE co-existence requirements.

Observation 2: 3GPP assumes geographical separation between NTN and TN networks in co-existence studies.

Observation 3: UE-to-UE co-existence requirements between new S-band and DL of n2 and n25 should provide protection to n2 and n25 DL.

Proposal 2: Further work is needed to set appropriate UE-to-UE co-existence requirements towards n2 and n25 DL, as it is not obvious what limit should be applied.

## 3	Conclusion

In this contribution co-existence between new NTN S-band and adjacent 3GPP bands was discussed. Following proposals were made.

Proposal 1: Co-existence work regarding co-existence with other 3GPP bands can be focused on NTN UE Tx interference to DL of n2 and n25.

Observation 1: 3GPP has multiple examples of less stringent than -50 dBm/MHz UE-to-UE co-existence requirements.

Observation 2: 3GPP assumes geographical separation between NTN and TN networks in co-existence studies.

Observation 3: UE-to-UE co-existence requirements between new S-band and DL of n2 and n25 should provide protection to n2 and n25 DL.

Proposal 2: Further work is needed to set appropriate UE-to-UE co-existence requirements towards n2 and n25 DL, as it is not obvious what limit should be applied.

## 4	References

- RP-241661, New WID on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL), DISH Network, EchoStar, TerreStar, OmniSpace, Telus

- RP-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung