3GPP TSG-RAN WG4 Meeting #112	R4-2413148

# Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:		8.4.2

Source: 		Qualcomm Inc.

Title: 		Scell bandwidth and sync raster

Document for:		Approval

## 1	Introduction

In RAN4#110bis RAN4 agreed a WF [1] stating that 5 MHz CBW can be added to the WI, pending RAN plenary decision. In this contribution improvements needed to efficiently support both 3 and 5 MHz CBWs in n100 are discussed.

## 2	Discussion

RAN4 specifies synchronization raster for system information acquisition when explicit signalling of the synchronization block position is not present. For less than 5 MHz operation the sync raster was designed so that each sync raster point maps to specific SSB bandwidth, and therefore UE can know what is the SSB bandwidth and transmission bandwidth based on the frequency it locks to.

In case of carrier aggregation, SCell is not required to be in the sync raster. However, the current specification only allows a cell with SSB after puncturing and new CORESET0 for less than 5MHz to be used when associated with the new sync raster points, such as defined in TS38.211 and TS38.213. Therefore, it is not currently fully clear whether SCell with less than 5MHz is also restricted to the new sync raster points or not.

In 38.211,

“For CORESET 0 on a carrier where the SS/PBCH block is detected at sync raster points defined in Tables 5.4.3.1-2 or 5.4.3.1-3 of [14, TS 38.101-1] and configured by the ControlResourceSetZero IE:…”

In 38.213,

“In Table 13-0, configurations with index 0 to 9 are applicable when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in [8-1, TS 38.101-1], configurations with index 10 to 11 are applicable when an associated SS/PBCH block is located according to NOTE 12 of Table 5.4.3.3-1 in [8-1, TS 38.101-1], and non-interleaved CCE-to-REG mapping applies for configurations with index 6 to 9. In Table 13-1, the associated SS/PBCH block is not located according to NOTE 12 of Table 5.4.3.3-1 in [8-1, TS 38.101-1].”

If the SCell with less than 5MHz is associated with new sync raster points, the SSB bandwidth and the transmission bandwidth can be implicitly indicated by the associated sync raster points, similar as PCell.

However, if the SCell with less than 5MHz is NOT associated with new sync raster points, the SSB bandwidth and the transmission bandwidth for SCell need to be known by the UE. If the carrierBandwidth for SCell (indicated by SCellConfig > ServingCellConfigCommon > DownlinkConfigCommon > FrequencyInfoDL > SCS-SpecificCarrier > carrierBandwidth) is 15PRBs for carrier of 3MHz (according to Table 5.3.2-1 [TS38.101-1]), the SSB bandwidth can be determined as 12PRBs, but the transmission bandwidth is still unclear between 15PRBs or 12PRBs. Similarly, if the carrierBandwidth for SCell is not for 3MHz but for 5MHz, the SSB bandwidth can be determined as 20PRBs, but the transmission bandwidth is still unclear between 25PRBs or 20PRBs.

In 38.306 the support for 12 and 20 PRB is specified as follows:

| support5MHz-ChannelBW-20PRB-CORESET0-r18 Indicates whether the UE supports short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS, and the reception of 20 PRB CORESET0. This feature is supported for 15 kHz SCS only. This feature is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41638 of Table 5.4.3.1-3 in TS 38.101-1 [2]. This feature is only applicable to single-carrier operation. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this feature supports configuration of 20 PRB BWP operation. | UE | No | FDD only | FR1 only |
| --- | --- | --- | --- | --- |
| support12PRB-CORESET0-GSCN-41637-r18 Indicates whether the UE supports reception of 12 PRB CORESET0 with an associated SS/PBCH block located at GSCN 41637. A UE supporting this feature shall also indicate support of support3MHz-ChannelBW-Symmetric-r18. This feature is supported for 15 kHz SCS only. This feature is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41637 of Table 5.4.3.1-3 in TS 38.101-1 [2]. NOTE: The UE supporting this FG supports configuration of 12 PRB BWP operation. This feature is only applicable to single-carrier operation. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. | UE | No | FDD only | FR1 only |

Observation 1: Current RAN1 specifications specify a cell with SSB/CORESET0 for less than 5MHz are associated with the new sync raster points.

Observation 2: If SCell with less than 5MHz is not required to be in the new sync raster points, current specification is unclear on the transmission bandwidths when the cell is not a Pcell.

Proposal 1: Require less than 5 MHz SCell to be associated with the new sync raster points for less than 5MHz, similar as PCell.

## 3	Conclusion

In this contribution improvements needed to efficiently support both 3 and 5 MHz CBWs in n100 were discussed. Following observations and proposals were made.

Observation 1: Current RAN1 specifications specify a cell with SSB/CORESET0 for less than 5MHz are associated with the new sync raster points.

Observation 2: If SCell with less than 5MHz is not required to be in the new sync raster points, current specification is unclear on the transmission bandwidths when the cell is not a Pcell.

Proposal 1: Require less than 5 MHz SCell to be associated with the new sync raster points for less than 5MHz, similar as PCell.

## 4	References

[1]  R4-2406623, WF on UE RF requirements for NR channel BW less than 5MHz for FR1 Phase 2, Intel Corporation