3GPP TSG-RAN WG4 Meeting #112	R4-2413149

# Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:		7.20.3

Source: 		Qualcomm Inc.

Title: 		UE RF requirements for PC2 and 40 MHz

Document for:		Approval

## 1	Introduction

Requirements for PC2 for n28 were discussed lengthily in rel-18, but eventually postponed to rel-19. New WI including also 40 MHz CBW was approved in [1]. This document discussed UE RF requirements, many of which can be leveraged from rel-18 work, and new work needed for 40 MHz CBW.

## 2	Discussion

In the following the discussion is split between requirements up to 30 MHz, and requirements for 40 MHz CBW.

##### 2.1 Requirements for up to 30 MHz CBW

According to the WID there could be changes for up to and including 30 MHz CBW for PC3 only if any new regulatory requirements come up. Therefore, PC3 requirements are not discussed further.

For PC2, a lot of evaluations for both RSD and NS_17 and NS_18 A-MPR were done in rel-18. This work can now be taken use of. There was discussion in rel-18 on whether traditional split duplexer or full band duplexer should be assumed. Now when 40 MHz channel bandwidth is included in the scope, it is clear that traditional 2x30 MHz split duplexer cannot be the only architecture option. It would be possible to use a wider filter for the sub-band containing 40 MHz CBW, or use a full band filter.

From RAN4 specification point of view, it is better to avoid tight linkage to final implemented filter solution. It is possible to apply the same A-MPR and RSD values independent of implemented solution. For example, if some implementation needs less or no A-MPR, it can just apply smaller value even if specification allows larger A-MPR.

Proposal 1: Specify a single set of A-MPR and RSD values independent of implemented filter solution.

Therefore, we propose to apply the RSD in Table 1 and Table 2 for PC2 up to and including 30 MHz.

Proposal 2: Adopt the RSD in Table 1 and Table 2 for n28 up to 30 MHz channel bandwidths for PC2

Table 1: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n28 | [0.6] | 0.6 | 0.7 | 0.8 | 1.3 | 2.4 | 2.9 |  |  |  |  |

Table 2 Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n28 | [1.1] | 1.1 | 1.1 | 1.3 | 3.0 | 6.6 | 7.9 |  |  |  |  |

Similarly, we propose to adopt the A-MPR for NS_17 and NS_18 as in Table 3 to Table 6 for PC2 operation.

Proposal 3: Adopt the A-MPR in Table 3 to Table 6 for n28 up to 30 MHz channel bandwidths for PC2

Table 3: A-MPR regions for NS_17 for PC2

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 723 ≤ Fc ≤ 728 | ≤ 0.18 | ≤ 1.44 | A1 |
|  |  | ≥ 0 | > 5.4 | A2 |

Table 4: A-MPR for NS_17 for PC2

| Modulation/Waveform |  | A1 | A2 |
| --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤ 3 | ≤ 3.5 |
|  | QPSK | ≤ 3 | ≤ 4 |
|  | 16 QAM | ≤ 4 | ≤ 4.5 |
|  | 64 QAM | ≤ 4 | ≤ 5 |
|  | 256 QAM |  | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 4.5 | ≤ 6 |
|  | 16 QAM | ≤ 5 | ≤ 6 |
|  | 64 QAM | ≤ 5 | ≤ 6 |
|  | 256 QAM |  |  |

Table 5: Band n28 and n83 25 MHz and 30 MHz A-MPR regions for NS_18 and PC2

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 | 703~733 | >(LCRB*12*SCS)/2+3.6 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+3.6 | ≥5.4 | A4 |
|  |  | ≤6.3 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+3.6 ≤(LCRB*12*SCS)/2+5.76 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥ 5.4 | A6 |
| 30 | 703~733 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+5.22 ≤(LCRB*12*SCS)/2+7.38 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥ 5.4 | A6 |

Table 6: A-MPR for NS_18 for PC2

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | N/A | ≤ 7 | ≤ 3.5 | ≤ 10 | ≤ 4.5 | ≤ 2 |
|  | QPSK | ≤ 3.5 |  | ≤ 7 | ≤ 3.5 | ≤ 10 | ≤ 4.5 | ≤ 2 |
|  | 16 QAM | ≤ 4 |  | ≤ 8 | ≤ 3.5 | ≤ 10 | ≤ 5 | ≤ 2 |
|  | 64 QAM | ≤ 4.5 |  | ≤ 9 | ≤ 3.5 | ≤ 10 | ≤ 5.5 | ≤ 2.5 |
|  | 256 QAM | ≤ 6.5 |  | ≤ 10 | ≤ 3.5 | ≤ 10 | ≤ 5.5 |  |
| CP-OFDM | QPSK | ≤ 5.5 |  | ≤ 9.5 | ≤ 5 | ≤ 11 | ≤ 6 | ≤ 4 |
|  | 16 QAM | ≤ 5.5 |  | ≤ 10 | ≤ 5 | ≤ 11 | ≤ 6 | ≤ 4 |
|  | 64 QAM | ≤ 6 |  | ≤ 10 | ≤ 5 | ≤ 11 | ≤ 6 | ≤ 4 |
|  | 256 QAM | ≤ 9 |  | ≤ 11.5 | ≤ 5 | ≤ 11 | ≤ 7.5 |  |

It should be noted that for NS_18, A-MPR regions A1 and A2 which are not included in Table 5 are specified for PC3 operation in 38.101-1 Table 6.2.3.1-1 and the same regions would be applicable also to PC2.

##### 2.2 Requirements for 40 MHz CBW

It is useful to discuss which regulatory requirements are applicable for 40 MHz channel bandwidth. There was similar discussion when 30 MHz channel bandwidth was being worked on, and some conclusions were agreed in [2] and are reproduced below:

- NS_17 and NS_18 aren’t applicable to China.

- It’s possible for n28 30MHz deployment in EU.

- NS-17 is not needed for 30MHz.

NS_17 is intended for DTV protection especially in Japan, and protection requirement overlaps with the frequencies for 40 MHz channel bandwidth. Therefore, NS_17 is not applicable also for 40 MHz channel bandwidth.

On the other hand, NS_18 was defined for more general protection against DTV. For 30 MHz, NS_18 requirements were specified based on possible deployment in EU. However, in n28 there is less than 40 MHz spectrum available in EU, as seen in [3] and [4]. Therefore, 40 MHz for NS_18 is not needed in EU. As of now, we are aware some countries in APAC region have 40 MHz available which motivates including NS_18 into the scope.

Observation 1: NS_17 and NS_18 are not applicable in China, and in Europe there is less then 40 MHz spectrum available in n28, however in APAC 40 MHz can be available.

Proposal 4: NS_17 is not required for 40 MHz CBW. Based on spectrum allocations in APAC region, NS_18 should be included for 40 MHz.

Additionally, there are number of general co-ex requirements specified for n28. For example, -42 dBm/8 MHz is originating from Europe. While the applicability of the stringent limit is restricted to 10 MHz, the same requirement states that otherwise -25 dBm/8 MHz is to be met. It should be evaluated that all specified coex requirements are also applicable for 40 MHz.

Proposal 5: Evaluate whether the general co-ex requirements specified for n28 are applicable for 40 MHz channel bandwidth.

Finally, we have evaluated PC3 refsens for 40 MHz CBW and propose following RB configuration and REFSENS values to be used for 40 MHz CBW. We will provide PC2 RSD evaluations in October.

Figure 1 PC3 40MHz REFSENS evaluation

Proposal 6: Adopt refsens for 40 MHz channel bandwidth as in Table 7 to Table 8.

Table 7: UL configuration for reference sensitivity

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
| n28 | 15 | 15 | 25 | 251 | 251 | 251 | 251 | 251 |  | 251 |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  | 101 | 101 | 101 | 101 | 101 |  | 101 |  |  |  |  |  |  |  |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 8: Reference Sensitivity power level

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n28 | 15 | -100.2 | -98.5 | -95.5 | -93.5 | -90.8 | -84.2 | -78.5 |  | -65.9 |  |  |
|  | 30 |  |  | -95.6 | -93.6 | -91.0 | -84.2 | -78.6 |  | -65.9 |  |  |

## 3	Conclusion

In this contributions RF requirements for PC2 operation and 40 MHz CBW for n28 were discussed. Following observations and proposals were made:

Proposal 1: Specify a single set of A-MPR and RSD values independent of implemented filter solution.

Proposal 2: Adopt the RSD in Table 1 and Table 2 for n28 up to 30 MHz channel bandwidths for PC2

Proposal 3: Adopt the A-MPR in Table 3 to Table 6 for n28 up to 30 MHz channel bandwidths for PC2

Observation 1: NS_17 and NS_18 are not applicable in China, and in Europe there is less then 40 MHz spectrum available in n28, however in APAC 40 MHz can be available.

Proposal 4: NS_17 is not required for 40 MHz CBW. Based on spectrum allocations in APAC region, NS_18 should be included for 40 MHz.

Proposal 5: Evaluate whether the general co-ex requirements specified for n28 are applicable for 40 MHz channel bandwidth.

Proposal 6: Adopt refsens and RSD for 40 MHz channel bandwidth as in Table 7 to Table 8.

## 4	References

[1]  RP-241660, New WID: Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28, CMCC CBN

[2]  R4-1912979, WF for 30MHz channel bandwidth in band n28, Huawei, HiSilicon, Skyworks Solutions Inc.

[3]	ECC Decision (15)01, Harmonised technical conditions for mobile/fixed communications networks (MFCN) in the band 694-790 MHz including a paired frequency arrangement (Frequency Division Duplex 2x30 MHz) and an optional unpaired frequency arrangement (Supplemental Downlink)

[4]  	COMMISSION IMPLEMENTING DECISION (EU) 2016/687, on the harmonisation of the 694-790 MHz frequency band for terrestrial systems capable of providing wireless broadband electronic communications services and for flexible national use in the Union