# 3GPP TSG-RAN WG4 Meeting # 112	R4-2413158

# Maastricht, Netherlands, Aug 19 – 24, 2024

Source: 	Apple

Title: 	ATG Tx requirement applicability at antenna/TAB connectors

Agenda Item:	5.8.1

Document for:	Discussion

# Introduction

In RAN4#111 meeting, how to apply the Tx requirement at antenna connectors for ATG UE was discussed based on [1-3]. And the CR on ATG UE Tx RF requirements has been agreed in [3]. This contribution will share our view on the requirement applicability at antenna connectors for ATG UE

# Discussion

It has been agreed in [3] that the Tx power related requirement will be specified as the sum of all antenna connectors including UE maximum output power and configured transmitted power. While the applicability for other requirements remains open. Table 1 gives our view for all Tx requirements.

Table 2-1 Tx requirement applicability for ATG UE

| Tx requirements for ATG UE | Requirement applicability |
| --- | --- |
| 6.2J Transmitter power for ATG |  |
| 6.2J.1 UE maximum output power for ATG | Defined as the sum of all antenna connectors or TAB connectors and terminologies have been introduced in [3] |
| 6.2J.2 Configured transmitted power for ATG | Defined as the sum of all antenna connectors or TAB connectors and terminologies have been introduced in [3] |
| 6.3J Output power dynamics for ATG |  |
| 6.3J.1 Minimum output power for ATG | Defined as the sum of all antenna connectors or TAB connectors |
| 6.3J.2 Transmit OFF power for ATG | Defined as the sum of all antenna connectors or TAB connectors |
| 6.3J.3 Transmit ON/OFF time mask for ATG | Defined as the sum of all antenna connectors or TAB connectors for ON/OFF power measurement |
| 6.3J.4 Power control for ATG | Defined as the sum of all antenna connectors or TAB connectors |
| 6.4J Transmit signal quality for ATG |  |
| 6.4J.1 Frequency error for ATG | Defined at each antenna connector or TAB connector |
| 6.4J.2 Transmit modulation quality for ATG | Defined at each antenna connector or TAB connector |
| 6.5J Output RF spectrum emissions for ATG |  |
| 6.5J.1 Occupied bandwidth for ATG | Defined as the sum of all antenna connectors or TAB connectors when measuring power. |
| 6.5J.2 Out of band emission for ATG | Defined as the sum of all antenna connectors or TAB connectors |
| 6.5J.3 Spurious emissions for ATG | Defined as the sum of all antenna connectors or TAB connectors |

Proposal 1: It is proposed that RAN4 adopt the requirement applicability rules as proposed in Table 2-1 for ATG UE.

# Summary

Proposal 1: It is proposed that RAN4 adopt the requirement applicability rules as proposed in Table 2-1 for ATG UE

# References

[1] R4-2406592, (NR_ATG-Core) CR for 38101-1 to update ATG related requirement, Apple

[2] R4-2409320, Clarifications on general definition for ATG UE, Huawei, HiSilicon.

[3] R4-2410739, (NR_ATG-Core) CR for 38101-1 on ATG UE Tx RF requirement, Apple