3GPP TSG-RAN WG4 Meeting # 112                              							     R4-2413159

Maastricht, Netherlands 19th – 23th, Aug 2024

Source: 	Apple

Title: 	Remaining issue on MIMO for ATG UE

Agenda Item:	8.10.2.3

Document for:	Discussion

# Introduction

MIMO for ATG have been discussed in the last RAN4 meeting and the agreements were captured in the WF [1]. The following open issues are for further study.

Issue 4-1-1: ULFPTx mode for UL-MIMO

Agreement:

- FFS on whether ULFPTx mode for UL-MIMO is supported for ATG UE

Issue 4-2-1: how to modify the NR UL MIMO requirement with ATG capability antennaArrayType-r18

Agreement:

- if the legacy requirement for UL MIMO is defined as sum of measurements of all UE transmit antenna connectors, then for ATG UE with capability antennaArrayType-r18, the requirement should be defined as sum of all TAB connector including both polarization.

- If the legacy requirement for UL MIMO is defined as per antenna port or per layer, FFS

Issue 4-2-6: whether to apply coherent UL MIMO requirement for ATG UE

Agreement:

- FFS on whether to apply coherent UL MIMO requirement for ATG UE

Issue 4-3: receiver requirements

Agreement:

- FFS on whether to have Rx requirements for UL-MIMO for ATG UE

This contribution will give further analysis on these open issues and presents our proposals.

Issue 4-1-1: ULFPTx mode for UL-MIMO

Agreement:

- FFS on whether ULFPTx mode for UL-MIMO is supported for ATG UE

Discussion

ULFPTx mode is introduced to address Rel-15 MIMO codebook design and power control limitation that a UE cannot reach its maximum output power. For UE supporting ULFPTx, there could be 3 different implementations corresponding to 3 kinds of ULFPTx mode.

Mode 1: for the UE to support full Tx power in UL transmission, full rated PAs on each Tx chain is supported, e.g. 23dBm + 23dBm

Mode 2: for the UE to support full Tx power in UL transmission, no Tx chain is assumed to deliver full power. E.g. 20dBm + 20dBm

Mode 3: for the UE to support full Tx power in UL transmission, some of Tx chains with full rated Pas is supported. E.g. 23dBm + 20dBm

For ATG UE, the same situation could happen as it uses the same code book design and power control as TN UE. Though the power is declared by signaling, it could have different implementations as described in the 3 ULFPTx modes. So it is natural to support this as well for ATG UE.

Proposal 1: ULFPTx mode is supported for ATG UE.

Issue 4-2-1: how to modify the NR UL MIMO requirement with ATG capability antennaArrayType-r18

Agreement:

- if the legacy requirement for UL MIMO is defined as sum of measurements of all UE transmit antenna connectors, then for ATG UE with capability antennaArrayType-r18, the requirement should be defined as sum of all TAB connector including both polarizations.

- If the legacy requirement for UL MIMO is defined as per antenna port or per layer, FFS

I could not recall why the 2nd bullet is left open. If the legacy requirement for UL-MIMO is defined as per antenna port per layer. It is natural to apply the same principle as for ATG UE. The only difference for ATG UE is that there are omni-directional antenna and antenna array.

- For ATG UE with omini-directional antenna, the requirement is applied at antenna connector or per layer.

- For ATG UE with antenna array, the requirement is applied at TAB connector or per layer.

Proposal 2: For ATG UE with omini-directional antenna, the requirement is applied at antenna connector or per layer. For ATG UE with antenna array, the requirement is applied at TAB connector or per layer.

Issue 4-2-6: whether to apply coherent UL MIMO requirement for ATG UE

Agreement:

- FFS on whether to apply coherent UL MIMO requirement for ATG UE

If there is no additional effort, it would be ok to apply the same UL MIMO requirement for ATG UE.

# Summary

This contribution presented our consideration on the open issues for ATG UE supporting MIMO. The following proposals are concluded.

Proposal 1: ULFPTx mode is supported for ATG UE.

Proposal 2: For ATG UE with omini-directional antenna, the requirement is applied at antenna connector or per layer. For ATG UE with antenna array, the requirement is applied at TAB connector or per layer.

# References

[1] RP-2406594, WF on Rel-19 ATG UE requirements, CMCC, RAN4#110bis