3GPP TSG-RAN4 Meeting #112	R4-2413167

# Maastricht, Netherlands, 19th – 23th August, 2024

Source: 	Qualcomm Incorporated

Title: 	FR2-1 L3 measurement delay by optimizing RX beam sweeping factor

Agenda item:	8.15.2.1

Document for:	Discussion

# Introduction

In this paper, we are presenting view for R19 RRM enhancement phase-5 WI.

# Discussion

## Prior Agreements

Following proposals were agreed/discussed during the last meeting:

#### Issue 2-1-1: Applicability requirement of L3 measurement delay reduction by optimizing Rx BSF Agreement: Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx simultaneous reception are applicable provided that: - the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and - UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC. Note: The ‘other UE CA/DC modes (e.g., 1 or 2 FR2-1 bands CA, or FR1+FR2 CA/DC, or EN-DC)’ and/or the ‘other number of target to-be-measured carrier(s) on FR2-1 band’ can be FFS after concluding the baseline above. These extra FFS parts will NOT delay the WI completion. Issue 2-1-2: Conditions to apply L3 measurement delay reduction by optimizing Rx BSF WF for next meeting: FFS：Conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF - FFS: multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception features - FFS: UE’s mobility status, e.g., whether HST is precluded or not - FFS: RRM measurement with two panels activated, two searchers are occupied by this single carrier - FFS: SSB processing delay/time for processing multiple beams received in a SMTC - FFS: Power consumption issue - FFS: UE has prior knowledge on the cell to be measured - FFS: Rel-19 L3 measurement with multi-Rx DL reception is irrelevant to multi-TRP operation deployment - FFS: Other conditions: cell-centre UE or cell-edge UE - FFS: Simultaneous operation between L3 and L1 measurements Issue 2-1-3: UE measurement procedures to use L3 measurement delay reduction by optimizing Rx BSF WF for next meeting: FFS：Scenarios to use L3 measurement delay reduction by optimizing Rx BSF: - FFS: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - FFS: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - FFS: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - FFS: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - FFS: Handover - FFS: PSCell addition - FFS: RRC Re-establishment/RRC Connection Release with Redirection - FFS: SCell activation - FFS: SCG activation - FFS: CGI identification - FFS: CSI-RS based intra-/inter-frequency measurements, the CSI-RS is configured associatedSSB - The discussion on CSI-RS configured with associatedSSB could be revisited if SSB based L3 measurement delay reduction is concluded.

## Applicability of Fast Beam Sweeping

RAN4 should first focus on defining a framework for the fast beam sweeping factor based L3 measurement and mobility latency enhancement for a single cell. Whether or how to extend the solution and framework to CA/DC scenarios (e.g., FR2 PSCell addition, FR2 SCell activation, FR2 SCG activation) can be discussed after seeing the framework for the single cell scenario.

Also, UE can reduce both detection and measurement periods by applying beam sweeping factors. Hence, the advantage of latency reduction by applying FBS should be applicable to both SSB detection (PSS/SSS detection, PBCH-DMRS reading, MIB decoding) and SSB based L3 measurement period requirements.

Proposal 1: RAN4 to define an overall framework for the fast beam sweeping factor based L3-measurement and mobility latency enhancement for a single serving cell. Whether/how to extend the solution and framework to CA/DC scenarios (e.g., FR2 PSCell addition, FR2 SCell activation, FR2 SCG activation) is FFS.

Proposal 2: The fast beam sweeping factor, if in use, applies to both SSB detection (PSS/SSS detection, PBCH-DMRS reading, MIB decoding) and SSB based L3 measurement period requirements.

## Impact of Channel Condition on Triggering Fast Beam Sweeping

It is important to determine if mobility or cell edge conditions can influence fast beam sweeping. If UE is in cell center and if it has low mobility, it is less likely to require measurement of neighbor cell. UE does not need to employ multi-panel and fast beam sweeping in this scenario. On the other hand, UE is more likely to be handed over to neighbor cell if UE is at cell edge or if UE is moving quickly. Hence, mobility or cell edge conditions should determine the triggering of fast beam sweeping.

Observation 1: If UE is in cell center and if it has low mobility, it does not need to measure neighboring cells frequently. UE does not need to employ multi-panel and fast beam sweeping in this scenario.

Observation 2: UE is more likely to be handed over to neighbor cell if UE is at cell edge or if UE is moving quickly.

Figure 1: Transition between fast beam sweeping and legacy beam sweeping

Figure 1 shows transition between fast and baseline beam sweeping based on channel conditions. Case B and case M refer to baseline beam sweeping and fast beam sweeping respectively. 40ms is the SMTC period. 8 and 4 denote the scaling factor for evaluation periods during baseline beam sweeping and fast beam sweeping respectively.

3GPP has already defined the criteria to determine cell edge and high-speed conditions for relaxed idle/inactive mode measurement and for RLM/BFD evaluation. Appendix A shows the criteria for low mobility and good serving cell quality. These criteria can be reused to determine the cell edge and high-speed conditions to apply fast beam sweeping, too.

Figure 2: Reporting of current channel condition for beam sweeping status and its impact on mobility procedures

In procedures like handover, network needs to know UE’s expected time to complete the procedure. This allows network to reserve contention free random access preambles during handover. If UE’s beam sweep factor depends on channel conditions, it may change from time to time and network needs to be aware of UE’s current channel conditions. Hence, UE needs to inform network when not cell-edge and not high-speed conditions are fulfilled at UE.

Observation 3: In some mobility procedures (e.g., handover), network needs to estimate UE’s expected time to complete the procedure. It is better for network to know when not cell-edge and not high-speed conditions are fulfilled at UE if these conditions influence triggering of fast beam sweeping.

3GPP has already introduced not cell-edge and not high-speed conditions for idle/inactive mode measurement and RLM/BFD relaxation. Appendix A shows the detailed procedures that UE can use to determine if it has fulfilled cell edge or high-speed conditions for RLM/BFD relaxation. Those same procedures can be used to determine “not cell edge (and not high-speed)” conditions and beam sweeping, too.

Observation 4: 3GPP has already introduced not cell-edge and not high-speed conditions for idle/inactive mode measurement and RLM/BFD relaxation.

Existing RLM/BFD relaxation mechanisms have introduced signaling and procedures to report RLM/BFD relaxation status. These signaling and procedures are shown in appendix B. These reports for conveying RLM/BFD relaxation status can be reused or served as a baseline for conveying beam sweeping status.

UE should not immediately report to the network after “not cell-edge and not high-speed” conditions are fulfilled. UE’s condition may change, and immediate reporting may lead to a ping-pong effect. UE should wait for a certain time to report to the network after “not cell-edge and not high-speed” conditions are fulfilled. TTT-like time window or N310 like timer, which starts running or counting upon the first satisfaction of the condition “not cell-edge (and not high-speed)” is observed by the UE, can be configured to avoid frequent status transitions and reports.

Observation 5: 3GPP has introduced reporting configurations to convey RLM/BFD relaxation status.

Observation 6: After “not cell-edge and not high-speed” condition is fulfilled for the first time, UE should wait for a certain time to report its status to the network.

UE should not relax its beam sweep factor to eight after it has received explicit signaling (e.g., FR2 CHO, FR HO, GBBR, etc.) from the network. Changing beam sweep factor after receiving these explicit signaling will confuse network regarding the expected timeline of the mobility procedure.

Observation 7: Relaxing beam sweep factor to eight after receiving explicit signaling like HO, FR2 CHO, etc. until the completion of the procedure will confuse network regarding the procedure’s expected timeline.

Figure 2 shows how UE’s change of beam sweep factor can influence a mobility procedure. UE’s steps in this figure are shown below.

- UE initially reports its capability of fast beam sweep to network

- Later, UE receives measurement object configuration from network.

- After a certain time, UE observes that the conditions of “not cell-edge” and “not high-speed” are fulfilled.

- UE waits for a certain time, governed via a TTT-like window or N310-like timer, and reports its relaxation of beam sweep status to network.

- UE also starts using legacy beam sweeping factor from that instance.

- Later, UE receives an explicit command from network to perform a mobility procedure (e.g., handover).

- UE completes handover using legacy beam sweeping factor.

- After an application delay that corresponds to HO timeline, UE resorts to using fast beam sweeping factor.

This whole procedure can be described through the following proposal:

Proposal 3: RAN4 to adopt the following framework for the fast UE Rx beam sweeping based L3 measurement and mobility requirements:

  - NW provides the following criteria for fast beam sweeping application, and the signaling details are FFS

    - Cell edge condition: Threshold value of absolute L3 SSB-RSRP of SpCell

    - High speed condition: Threshold value of L3 SSB-RSRP variation on SpCell over a time period T

    - When the condition of not cell-edge (and not high-speed, if configured) is met, the UE is allowed to fallback Rx beam sweeping factor to the existing N value

    - Note: the existing criteria defined for the relaxed idle/inactive mode measurement and/or RLM/BFD evaluation can be reused or served as a baseline

  - Report configuration for the status of fast beam sweeping factor application, and the signaling details are FFS

    - A TTT-like time window or N310-like timer, which starts running or counting upon the first satisfaction of the condition “not cell-edge (and not high-speed)” is observed by the UE, can be configured to avoid frequent status transitions and reports

    - Note: the existing report defined for RLM/BFD relaxation status can be reused or served as a baseline

  - Besides, other explicit signaling (e.g. FR2 CHO, FR HO, GBBR, etc.) may disallow the fallback of UE Rx beam sweeping factor to the existing N value until the signaled configuration is no longer in effect or the relevant task has been completed.

  - FFS on the application delay of UE Rx beam sweeping factor switch

# Conclusions

Observation 1: If UE is in cell center and if it has low mobility, it does not need to measure neighboring cells frequently. UE does not need to employ multi-panel and fast beam sweeping in this scenario.

Observation 2: UE is more likely to be handed over to neighbor cell if UE is at cell edge or if UE is moving quickly.

Observation 3: In some mobility procedures (e.g., handover), network needs to estimate UE’s expected time to complete the procedure. It is better for network to know when not cell-edge and not high-speed conditions are fulfilled at UE if these conditions influence triggering of fast beam sweeping.

Observation 4: 3GPP has already introduced not cell-edge and not high-speed conditions for idle/inactive ode measurement and RLM/BFD relaxation.

Observation 5: 3GPP has introduced reporting configurations to convey RLM/BFD relaxation status.

Observation 6: After “not cell-edge and not high-speed” condition is fulfilled for the first time, UE should wait for a certain time to report its status to the network.

Observation 7: Relaxing beam sweep factor to eight after receiving explicit signaling like HO, FR2 CHO, etc. will confuse network regarding the expected timeline of the mobility procedure.

Proposal 1: RAN4 to define an overall framework for the fast beam sweeping factor based L3-measurement and mobility latency enhancement for a single serving cell. Whether/how to extend the solution and framework to CA/DC scenarios (e.g., FR2 PSCell addition, FR2 SCell activation, FR2 SCG activation) is FFS.

Proposal 2: The fast beam sweeping factor, if in use, applies to both SSB detection (PSS/SSS detection, PBCH-DMRS reading, MIB decoding) and SSB based L3 measurement period requirements.

Proposal 3: RAN4 to adopt the following framework for the fast UE Rx beam sweeping based L3 measurement and mobility requirements:

  - NW provides the following criteria for fast beam sweeping application, and the signaling details are FFS

    - Cell edge condition: Threshold value of absolute L3 SSB-RSRP of SpCell

    - High speed condition: Threshold value of L3 SSB-RSRP variation on SpCell over a time period T

    - When the condition of not cell-edge (and not high-speed, if configured) is met, the UE is allowed to fallback Rx beam sweeping factor to the existing N value

    - Note: the existing criteria defined for the relaxed idle/inactive mode measurement and/or RLM/BFD evaluation can be reused or served as a baseline

  - Report configuration for the status of fast beam sweeping factor application, and the signaling details are FFS

    - A TTT-like time window or N310-like timer, which starts running or counting upon the first satisfaction of the condition “not cell-edge (and not high-speed)” is observed by the UE, can be configured to avoid frequent status transitions and reports

    - Note: the existing report defined for RLM/BFD relaxation status can be reused or served as a baseline

  - Besides, other explicit signaling (e.g. FR2 CHO, FR HO, GBBR, etc.) may disallow the fallback of UE Rx beam sweeping factor to the existing N value until the signaled configuration is no longer in effect or the relevant task has been completed.

  - FFS on the application delay of UE Rx beam sweeping factor switch

# Appendix A: Criteria for RLM/BFD Relaxation

The RLM/BFD relaxed measurement criteria for low mobility and good serving cell quality are defined as follows: [2]

# Appendix B: Reporting of BFD/RLM Relaxation State via UAI

UE’s reporting regarding BFD and RLM relaxation status can be summarized through the following:

- bfd-RelaxationReportingConfig > OtherConfig

  - UE uses the config to report the relaxation state of BFD measurements.

- rlm-RelaxationReportingConfig > OtherConfig

  - UE uses the config to report the relaxation state of RLM measurements.

The requirements for providing RLM/BFD relaxation status are defined as follows in RAN2 and RAN4 specs [2] [3].

NR UE employs the following timers to report RLM/BFD relaxation status [2].

# References

[1] R4-2410260, Way Forward for [111][228] NR_RRM_Ph5, Apple, RAN4 #111

[2] 38.331 V 17.7.0

[3] 38.133 V 17.11.0