3GPP TSG-RAN WG4 Meeting # 112	R4-2413168

# Maastricht, Netherlands, 19th – 23th August, 2024

Agenda item:	8.17.2

Source: 	Qualcomm Incorporated

Title: 	On AI/ML RAN4 Use Cases: Beam Management

Document for:	Discussion

# Introduction

We present our view on the scope of AI/ML work item in this contribution.

# Discussion

## Agreements from Past Meetings

Following agreements happened during the last meeting [1].

Issue 2-3: Reported measurements and ground truth Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded Issue 2-5: Test setup needs Agreement: maximum number of set B Tx beams that test system should be able to emulate: [8-16] maximum number of set A Tx beams that test system should be able to emulate: [64-128] FFS on AoAs UE rotation during the test: FFS UE rotation/repositioning between different tests: Yes Issue 2-6: Datasets for training/testing Agreement: - Vendors may take into account the test environment/conditions defined by RAN4 when training the UE - sufficient test environment/conditions should be defined to enable vendors to create the data needed for training - FFS on proposals to augment training data to avoid overfitting of UE models to the test environment - RAN4 to strive to make the test conditions similar to field deployment conditions

## Beam Prediction Testability

There are two cases captured in 38.843 [2] for beam prediction of DL Tx beam use case:

- BM-Case1: Spatial-domain Downlink beam prediction for Set A of beams based on measurement results of Set B of beams

- BM-Case2: Temporal Downlink beam prediction for Set A of beams based on the historic measurement results of Set B of beams

The prediction for set A includes:

- Alt.1: Tx and/or Rx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams

-	e.g., N predicted beams can be the Top-N predicted beams

- Alt.2: Tx and/or Rx Beam ID(s) of the N predicted DL Tx and/or Rx beams and other information

-	e.g., N predicted beams can be the Top-N predicted beams

- Alt.3: Tx and/or Rx Beam angle(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams

-	e.g., N predicted beams can be the Top-N predicted beams

We start with BM-Case 1. To verify the accuracy of the L1-RSRP prediction, sufficient randomness and variation in time and spatial domain of L1-RSRP of Set B and Set A beams has to be configured in the test and emulated by the test equipment. In addition, spatial domain prediction relies on sufficient number of observed beams (in Set B) from different (peak direction) AoAs on UE’s sphere coverage to learn the spatial domain characteristics and statistics to infer correlation between Set B and Set A in order to predict Set A from Set B. Note that beamwidth is one of the important observed characteristics to infer the correlation between beams, and therefore the test configurations have to capture beam patterns representing the Tx antenna gain w.r.t. AoDs.

Observation 1: An effective test case for beam prediction use case BM-Case1 requires the following:

- Sufficient randomness and variation in time and spatial domain of L1-RSRP has to be emulated in the test

- Support emulation of DL Tx beam sweeping with enough number of Tx beams in Set B and Set A

Observation 2: RAN4 L1-RSRP and other measurement test configurations support only:

- Deterministic and static power configuration on each AoA

- Emulation of received signals from up to 2 AoAs

MIMO OTA testing environment defined in Rel-17 support only:

- Emulation of CDL channel with fixed DL beam to the channel

- Emulation of received signals from up to 6 AoAs/probes, and the coverage (in terms of area on the sphere) by the 6 probes is very limited and sparse based on TR 38.827 Table 6.2.3-1, most of them are in Theta [0,30] and Phi [0,120], only one “mirror” point at negative Phi region.

| Probe Number | Theta [deg] | Phi [deg] |
| --- | --- | --- |
| 1 | 0.0 | 0.0 |
| 2 | 11.2 | 116.7 |
| 3 | 20.6 | -104.3 |
| 4 | 20.6 | 104.3 |
| 5 | 20.6 | 75.7 |
| 6 | 30.0 | 90.0 |

From observation 1 and 2, we can understand the gap between RAN4 latest test setup support and the requirements for implementing an effective BM-Case 1 test. The randomness and variation in time and spatial domain of L1-RSRP is typically from the propagation channel, which models pathloss/channel as the function of (1) AoD of the Tx beam (2) AoA of the Rx beam (3) fading on each path, and UE movement, either line movement or rotation.

Observation 3: The following conditions contribute to randomness and variation in time and spatial domain of L1-RSRP

- Propagation conditions as a function of (1) AoD of the Tx beam (2) AoA of the Rx beam (3) fading condition, e.g., a CDL channel

- Tx beamforming gain on the AoDs in the propagation conditions

- UE movement

In MIMO OTA test environment, the receive power of the DUT UE in the test chamber on 6 AoAs can be programmed based on the CDL model to be emulated. However, the input to the CDL channel model on each AoD is fixed, and therefore we can only emulate signals transmitted from a fixed DL Tx beam. Therefore, we need to enable the variation of input to the CDL channel model to emulate the DL Tx beam sweeping in the MIMO OTA test environment. Based on the Observation 1 to 3, the enhancement enables the BM-Case 1 test in the MIMO OTA environment.

Proposal 1: DL Tx beam sweeping for BM-Case 1 test can be emulated by the time-varying input power to the CDL channel model emulated on top of MIMO OTA test environment.

Proposal 1 can be implemented by considering the test system model as the below figure. We explain how RAN4 defines the quantities in the system model and how TE vendors implement the system model in the following:

AoA2

AoA1

Path 1-1

AoA3

Test Chamber

Path 1-2

Path 2

Path 3

AoD1-1

AoD1-2

AoD2

AoD3

Channel emulator + Tx beam gain emulation = probe power control

GTxAOD1-1

GTxAOD1-2

GTxAOD2

GTxAOD3

Figure 1: Proposed setup for beam prediction testing

We first explain why single probe is not sufficient to emulate beam sweeping. Suppose we only have the blue probe, which has AoD = 30 degrees (blue line on the beam gain figure below). With antenna element configuration M=4 and N = 4 and λ=12 and using the antenna model in 38.901 clause 7.3, we plot the Tx beamforming gain in the following (beam peak direction 30 is beam 1, 60 is beam 2, 70 is beam 3):

If we consider single path (without multipath fading), UE always see that beam 1 is 12dB stronger than beam 2, and beam 2 is 4dB stronger than beam 3. To better illustrate this case, we have the picture below for beam 1 and 2:

Base station

Single path channel

DUT

Chamber

TE emulation to determine probe power

Beam 2: gain 28dB

probe

Base station

Single path channel

DUT

Chamber

TE emulation to determine probe power

Beam 1: gain 40dB

probe

In this case, UE can easily predict the best beam by directly using the dataset, e.g., derive the average from dataset (beam 1 is 12dB stronger than beam 2, beam 2 is 4dB stronger than beam 3), then always report beam 1 is the best and RSRP is 12dB stronger than beam 2 or 16dB stronger than beam 3 (if beam 2 and/or 3 are in set B and beam 1 is in set A). No neural network is needed. Moreover, when TE sweeps more beams, there are more and more beams look similar from UE perspective, since in the entire beam pattern of each beam (as plotted above), we only select one AoD angle to observe, and therefore many of the beams look alike even though their beam pattern may look different.

However, if we add a few more probes and consider multi-path fading CDL models, we need to consider the orange and green lines and add them up on fading channel with varying gain also on different AoAs. The above simple algorithm can’t apply, and machine learning model is needed. We describe the mathematical details of the multi-path fading CDL model with beam sweeping below.

We start with explaining how we can emulate the DL Tx beam transmission through a fading channel, e.g., CDL channel, by configuring the power transmitted by the probes on different AoAs.

- Tx beamforming gain as a function of beam index and AOD: GTxiTx,AODj

RAN4 needs to define the set of beam indexes iTx with size LTx, and each beam index is defined by its beam peak direction. With beam peak direction defined and array size (M,N) and polarization P, based on antenna modeling in 38.901 clause 7.3, we can derive GTxiTx,AODj.

- Channel gain (from CDL channel model) on each path: Gpath,jAODj,AOAj is a function of AOD and AOA of the path

- Receive power emulation on AOAk (the power transmitted by the probe on AOAk) when DL Tx beam is iTx and transmission power (before Tx gain) is Pt

PrAOAk,iTx=AOAj=(or≅)AOAkPt×GTxiTx,AODj×Gpath,jAODj,AOAj

Proposal 2: Received power emulation can follow the formula below:

PrAOAk,iTx=AOAj=(or≅)AOAkPt×GTxiTx,AODj×Gpath,jAODj,AOAj

Where GTxiTx,AODj is Tx beamforming gain as a function of beam index and AOD, and channel gain (from CDL channel model) on each path, Gpath,jAODj,AOAj, is a function of AOD and AOA of the path.

RAN4 needs to define the set of AOAk with size LAoA, note that LAoA≠LTx, and LAoA depends on how much coverage (within UE spherical coverage) and granularity on AoA we want to model. In the simplest setup, we can use one probe to emulate the power dynamics on one AoA, which is based on PrAOAk,iTx. However, such a simplest setup doesn’t work for BM-Case 1 test because all other AoAs in the CDL model are missing, the received power emulation is inaccurate and many Tx beams may look very similar since UE can only observe the power on one AoA. Without accurate received power emulation and distinguishable Tx beams, the prediction model testing is ineffective. Therefore, we need sufficient large LAoA to accurately emulate the received power and the UE can observe different received power on different AoAs when different Tx beams are transmitting.

Based on the above formulation, we explain how we can emulate DL Tx beam sweeping by the proposed test system model in the following. Let the Tx beam used in time t denoted by iTxt based on the beam sweeping RS transmission pattern, and the channel gain at time t denoted by Gpath,jAODj,AOAj,t, then DL Tx beam sweeping is emulated by configuring the probe on each AoA following the power as a function of time:

PrAOAk,t=AOAj=(or≅)AOAkPt(t)×GTxiTx(t),AODj×Gpath,jAODj,AOAj,t

Note that Gpath,jAODj,AOAj,t is sampled based on the distribution from CDL model. We can verify TE by checking whether PrAOAk,t trajectory aligns to the distribution based on

AOAj=(or≅)AOAkPt×GTxiTx(t),AODj×Gpath,jAODj,AOAj,t

Proposal 3: Tx beam sweep can be emulated based on the following formula by the probes on AoAk:

PrAOAk,t=AOAj=(or≅)AOAkPt(t)×GTxiTx(t),AODj×Gpath,jAODj,AOAj,t

Where AoAk is the AoA of probe k, channel gain at time t denoted by Gpath,jAODj,AOAj,t sampled from CDL model, and Tx beam used in time t denoted by iTxt based on the beam sweeping RS transmission pattern. Note that the probe power configuration can be verified by comparing the probe power as a function of time and the distribution of PrAOAk,t.

We discuss the impact of Rx beamforming gain and the issue of ground truth availability in the following

- Rx beamforming gain as a function of Rx beam index iRx and angle of arrival AOAk: GRxiRx,AOAk

Combine with the above power on each AoA, we can derive the following:

- UE receives power when DL Tx beam iTx is transmitting

Pr,UEiTx=AOAkGRxiRx,AOAk×AOAj=(or≅)AOAkPt×GTxiTx,AODj×Gpath,jAODj,AOAj

TE is supposed to verify the UE received power/Tx beam rank report against Pr,UEiTx. However, since GRxiRx,AOAk is unknown to TE, ground truth of Pr,UEiTx is also unknown to TE. If we rewrite the received power as a function of time with Tx beam sweeping function iTxt, we have the following:

Pr,UEt=AOAkGRxiRx,AOAk×AOAj=(or≅)AOAkPt×GTxiTx(t),AODj×Gpath,jAODj,AOAj,t

Observation 4: UE received power when DL Tx beam iTx is transmitting can be formulated as:

Pr,UEt=AOAkGRxiRx,AOAk×AOAj=(or≅)AOAkPt×GTxiTx(t),AODj×Gpath,jAODj,AOAj,t

However, this quantity is unknown to TE since TE doesn’t have access to the GRxiRx,AOAk.

One potential resolution is explained in the following observation:

Observation 5: One candidate resolution to ground truth availability is explained in the following. We can configure the test and TE channel emulator (fader) so that the channel from t0 to t1 is the same as channel from t1 to t2

Gpath,jAODj,AOAj,t|t=t0t=t1= Gpath,jAODj,AOAj,t|t=t1t=t2

During t0 to t1, only set B is transmitted; and in t = t1 to t2, set A is transmitted

RAN4 requirement should define for the AI/ML model trained by a proper set of data that aligns with the test setup, i.e., the configurations, such as beams in Set B and Set A, gNB antenna physical characteristics, for training data generation should align with the configurations during the RAN4 test, and the existence of proper signaling based on RAN1/2 network signaling design is a side condition to apply the requirement. Moreover, size and composition of Set B and Set A have impact on the accuracy of beam prediction, and therefore should be taken into account when defining the requirement.

Proposal 4: The following issues should be considered when defining the beam prediction accuracy requirements

- Consistency between training and testing data (from the perspectives of beams in Set B and Set A, physical characteristics of gNB antenna etc.) should be guaranteed by signaling conveyed to UE.

- The impact of size and composition of Set B and Set A on accuracy requirement.

To better emulate the practical operating scenario, we can consider UE rotation, which can be done by rotating the holder of UE. Note that as depicted in Fig. 1, the AoA is based on UE coordination. Therefore, AoA becomes a time varying function, and the probes have to cover t in test timeAOAk(t), but the equation in proposal 3 still hold with AoA replaced by time varying functions. We summarize the UE rotation emulation in the following.

Proposal 5: RAN4 can consider UE rotation by leveraging the physical motion control of the DUT holder. With UE rotation, the probe coverage becomes t in test timeAOAk(t), where AoAs are time varying functions based on UE rotation (note that AoA is represented in UE coordination system). The probe functions for emulating Tx sweeping remain the same as proposal 3 except that AoAs become functions of time:

PrAOAk(t),t=AOAj=(or≅)AOAk(t)Pt(t)×GTxiTx(t),AODj×Gpath,jAODj,AOAj,t

## Performance Metrics

RAN4 defined the following use cases of beam management in its WID [4]:

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

RAN4 studied different performance metrics for AI/ML based beam management during study item discussion. These metrics are captured in [2]:

Both spatial-domain DL beam prediction and temporal DL beam prediction are considered. For metrics for beam management requirements/tests, the following test metrics/KPIs are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options The overhead/latency reduction should be considered for the requirements as the side condition.

Option 2 does not differentiate between the following two scenarios:

- Scenario A: UE’s top-K predicted beams are far from the top-1 strongest beam.

- Scenario B: UE’s top-K predicted beams are adjacent to the top-1 strongest beam but the top-K predicted beams don’t include the top-1 strongest beam.

UE’s actual performance in scenario B is much better than its performance in scenario A. Even if UE predicts a set of beams that don’t include but are adjacent to the strongest beam, UE’s performance during actual deployment may not degrade that much. On the other hand, predicting a set of beams that are far from the strongest beam will impact performance in actual deployment.

Observation 6: Option 2 (beam prediction accuracy) does not distinguish between the following two scenarios

- Scenario A: UE’s top-K predicted beams are far from the top-1 strongest beam.

- Scenario B: UE’s top-K predicted beams are adjacent to but don’t include the top-1 strongest beam

Option 3 does not suffer from this issue. Hence, RAN4 should prioritize option 3 for the performance evaluation of BM-Case 1 and BM-Case 2.

Value of X can be defined based on RAN4 simulation study.

Proposal 6: RAN4 prioritizes option 3 for the performance evaluation of BM-Case 1 and BM-Case 2.

- Value of X is FFS.

- Note: X can be defined based on RAN4 study.

The essential test steps for option 3 are shown below:

- Step 1: TE transmits set B beams. UE reports top K predicted beams of set A based on measurement of set B beams.

- Step 2: TE transmits set A beams. UE reports measured RSRP of all beams of set A. TE compares measured RSRP of strongest beam and maximum RSRP of top K predicted beam.

It is better to randomize the beams of set A while transmitting them in step 2. Through this, UE won’t be able to map its predicted top K beams from step 1 to its measured beams in step 2. UE will simply report its measured RSRP of all beams in step 2

It would be easier to illustrate this with an example. Assume that set A contains {A1, A2, …, AN} beam and set B contains {B1, B2, …, BM} beams where M < N. During step 1, TE transmits set B beams. UE reports top K predicted beams from {A1, A2, .., AN}.

During step 2, TE randomizes set A beam shapes while transmitting. That means, at time instance t1 of step 2, TE does not transmit using the beam shape that it used during prior A1 transmissions. TE transmits using the shape of a beam whose ID is randomly selected from set A. In this way, UE will not know that it is measuring beam A1 during time t1. UE will simply report its measured RSRP of all beams in step 2. Network can map the set A beams of step 1 with the transmitted set A beams of step 2 and check if UE meets requirement. The randomization of set A’s beams during step 2 will ensure UE’s proper implementation to pass the test.

Observation 7: RAN4 considers the following steps to test option 3:

- Step 1:

  - TE transmits set B beams.

  - UE reports top K predicted beams of set A based on measurement of set B beams.

- Step 2:

  - TE randomizes set A beams and then transmits them (e.g., during time t1, UE does not transmit with beam shape A1 and UE transmits using the shape of a beam whose ID is randomly selected from set A).

  - UE reports measured RSRP of all beams of set A.

  - TE compares measured RSRP of strongest beam and maximum RSRP of top K predicted beam.

# Conclusion

Observation 1: An effective test case for beam prediction use case BM-Case1 requires the following:

- Sufficient randomness and variation in time and spatial domain of L1-RSRP has to be emulated in the test

- Support emulation of DL Tx beam sweeping with enough number of Tx beams in Set B and Set A

Observation 2: RAN4 L1-RSRP and other measurement test configurations support only:

- Deterministic and static power configuration on each AoA

- Emulation of received signals from up to 2 AoAs

MIMO OTA testing environment defined in Rel-17 support only:

- Emulation of CDL channel with fixed DL beam to the channel

- Emulation of received signals from up to 6 AoAs/probes, and the coverage (in terms of area on the sphere) by the 6 probes is very limited and sparse based on TR 38.827 Table 6.2.3-1, most of them are in Theta [0,30] and Phi [0,120], only one “mirror” point at negative Phi region.

Observation 3: The following conditions contribute to randomness and variation in time and spatial domain of L1-RSRP

- Propagation conditions as a function of (1) AoD of the Tx beam (2) AoA of the Rx beam (3) fading condition, e.g., a CDL channel

- Tx beamforming gain on the AoDs in the propagation conditions

- UE movement

Proposal 1: DL Tx beam sweeping for BM-Case 1 test can be emulated by the time-varying input power to the CDL channel model emulated on top of MIMO OTA test environment.

Proposal 2: Received power emulation can follow the formula below:

PrAOAk,iTx=AOAj=(or≅)AOAkPt×GTxiTx,AODj×Gpath,jAODj,AOAj

Where GTxiTx,AODj is Tx beamforming gain as a function of beam index and AOD, and channel gain (from CDL channel model) on each path, Gpath,jAODj,AOAj, is a function of AOD and AOA of the path.

Proposal 3: Tx beam sweep can be emulated based on the following formula by the probes on AoAk:

PrAOAk,t=AOAj=(or≅)AOAkPt(t)×GTxiTx(t),AODj×Gpath,jAODj,AOAj,t

Where AoAk is the AoA of probe k, channel gain at time t denoted by Gpath,jAODj,AOAj,t sampled from CDL model, and Tx beam used in time t denoted by iTxt based on the beam sweeping RS transmission pattern. Note that the probe power configuration can be verified by comparing the probe power as a function of time and the distribution of PrAOAk,t.

Observation 4: UE received power when DL Tx beam iTx is transmitting can be formulated as:

Pr,UEt=AOAkGRxiRx,AOAk×AOAj=(or≅)AOAkPt×GTxiTx(t),AODj×Gpath,jAODj,AOAj,t

However, this quantity is unknown to TE since TE doesn’t have access to the GRxiRx,AOAk.

Observation 5: One candidate resolution to ground truth availability is explained in the following. We can configure the test and TE channel emulator (fader) so that the channel from t0 to t1 is the same as channel from t1 to t2

Gpath,jAODj,AOAj,t|t=t0t=t1= Gpath,jAODj,AOAj,t|t=t1t=t2

During t0 to t1, only set B is transmitted; and in t = t1 to t2, set A is transmitted.

Proposal 5: The following issues should be considered when defining the beam prediction accuracy requirements

- Consistency between training and testing data (from the perspectives of beams in Set B and Set A, physical characteristics of gNB antenna etc.) should be guaranteed by signaling conveyed to UE.

- The impact of size and composition of Set B and Set A on accuracy requirement.

Observation 6: Option 2 (beam prediction accuracy) does not distinguish between the following two scenarios

- Scenario A: UE’s top-K predicted beams are far from the top-1 strongest beam.

- Scenario B: UE’s top-K predicted beams are adjacent to but don’t include the top-1 strongest beam

Proposal 6: RAN4 prioritizes option 3 for the performance evaluation of BM-Case 1 and BM-Case 2.

- Value of X is FFS.

- Note: X can be defined based on RAN4 study.

Observation 7: Following steps can be considered to test option 3 for the performance evaluation of BM-Case 1 and BM-Case 2.

- Step 1:

  - TE transmits set B beams.

  - UE reports top K predicted beams of set A based on measurement of set B beams.

- Step 2:

  - TE randomizes set A beams and then transmits them (i.e., during time t1, UE does not transmit with beam shape A1 and UE transmits using the shape of a beam whose ID is randomly selected from set A).

  - UE reports measured RSRP of all beams of set A.

  - TE compares measured RSRP of strongest beam and maximum RSRP of top K predicted beam.

# References

        - R4-2401570, WF on AI/ML, 3GPP TSG-RAN WG4 Meeting #111

        - 3GPP TR 38.843 V18.0.0, Technical Report (TR): Study on artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.

        - 3GPP TR 38.901 V 18.0.0, Technical Report (TR): Study on Channel Model for Frequencies from 0.5 to 100 GHz.

        - RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#102, Edinburgh, Scotland, December 11 – December 15