3GPP TSG-RAN4 Meeting #112	R4-2413169

# Maastricht, Netherlands, 19th – 23th August, 2024

Source: 	Qualcomm Incorporated

Title: 	On AI/ML RAN4 Use Cases: CSI Interoperability

Agenda item:	8.17.4

Document for:	Discussion

# Introduction

In this paper, we are presenting simulation results of the CNN based encoder-decoder pair that got agreed during the email discussion after the last meeting.

# Discussion

## Prior Agreements

Following agreements occurred during the email discussion after the last RAN4 meeting [1].

Based on the two rounds of discussions, the following parameters are proposed: Fig 1. Detailed model diagrams of encoder. Fig 2. Detailed model diagrams of decoder. Table 1. CNN model parameters for encoder. Model architecture parameters for encoder (convolutional); Model type; CNN (Resnet like design). FLOPs; 47M (vs. 1.6M for eTypeII). Model size; 448Kbyte. Num of Resnet blocks; 6. Layer after resnet blocks; 2 layers (one FC layer, one Conv layer). Layer before resnet blocks; 1 layer (one conv layer). Layer within a resnet block; 2 Conv layer and direct combination. CNN kernel parameters; - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter). Kernel/Channel size within a resnet block; (3,3,32). Size of message (feature map) passed across layers; (13,32). Table 2. CNN model parameters for decoder. Model architecture parameters for decoder (convolutional); Model type; CNN (Resnet like design). FLOPs; 740M. Model size; 7.2Mbyte. Num of Resnet blocks; 6. Layer before resnet blocks; 2 layers (one FC layer, one Conv layer). Layer after resnet blocks; 1 layer (one conv layer). Layer within a resnet block; 2 Conv layer and direct combination. CNN kernel parameters; - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter). Kernel/Channel size within a resnet block; (3,3,128). Size of message (feature map) passed across layers; (13,32). Other model related parameters: Activation function; ReLU. Latent message size; 32 float values. Payload size, bits; 64 per layer.. Quantization; 2 bits. System parameters: Frequency range; 2GHz (4GHz optional). MIMO Layers; 1 layer (2 layers optional, companies to. Sub-bands; 13. Channel estimation; ideal. Metrics; SGCS of AI CSI feedback and eTypeII CSI feedback, both relative to ideal CSI.. Companies are also invited to provide the training parameters used.

## Training Parameters for Encoder and Decoder

Observation 1: Following training parameters were used to generate results for the CNN based encoder and decoder pair with the agreed model.

| Parameter | Value/Name |
| --- | --- |
| Training algorithm | Adam optimizer |
| Learning rate of training algorithm | 10^-4 |
| Number of epochs | 150 |
| Batch size | 128 |
| Training set | 450k |
| Training loss function | SGCS + additional terms for quantization aware training |

## Result

Observation 2: SGCS results of AI/ML encoder-decoder and Rel-16 eType2 CSI are as follows:

| Model | SGCS |  |
| --- | --- | --- |
| CNN encoder – CNN decoder (generated based on the agreements of the last meeting) | 0.6934 | -5.13 dB |
| Rel-16 eType2 | 0.6701 | -4.82 dB |

# Conclusions

Observation 1: Following training parameters were used to generate results for the CNN based encoder and decoder pair with the agreed model.

| Parameter | Value/Name |
| --- | --- |
| Training algorithm | Adam optimizer |
| Learning rate of training algorithm | 10^-4 |
| Number of epochs | 150 |
| Batch size | 128 |
| Training set | 450k |
| Training loss function | SGCS + additional terms for quantization aware training |

Observation 2: SGCS results of AI/ML encoder-decoder and Rel-16 eType2 CSI are as follows:

| Model | SGCS (layer 0) |  |
| --- | --- | --- |
| CNN encoder – CNN decoder (generated based on the agreements of the last meeting) | 0.6934 | -5.13 dB |
| Rel-16 eType2 | 0.6701 | -4.82 dB |

# References

[1] R4-[111][133] NR_AIML_air-email discussions after RAN4#111 meeting, https://list.etsi.org/scripts/wa.exe?A2=3GPP_TSG_RAN_WG4;aa5369d9.2406C&S=

[2] 38.331 V 17.7.0

[3] 38.133 V 17.11.0