3GPP TSG-RAN WG4 Meeting #112                                                     R4- 2413195

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:		Nokia

Title:	Test methodology for FR1 non-RedCap headworn XR devices

Agenda item:		8.12.2.1

Document for:	Discussion

# Introduction

Based on the WF from RAN4 111 meeting [1], in this paper, we’d like to share our views of XR devices related issues as listed below:

Issue 2-2-2: Coarse measurement grids Agreements - Legacy measurement grid can be baseline for XR devices TRP TRS measurement and measurement campaign used to derive performance requirements, RAN4 can further check and confirm coarse measurement grid based on measurement results and simulations. - MU analysis for coarse measurement grid is also needed.

# Discussion

## 2.1	TRP/TRS OTA test based on interpolated coarse measurement grids

Based on the 266 or 62 measurement grids in current standard TS 38.161, a simple change to define 62 measurement grids can be made to reduce the measurement time per OTA test and maintain the same accuracy level of using 266 measurement grids in OTA tests.

If we consider the test grid with 266 test points as the fine measurement grid, then we can consider 62 points measurement grid (for example) as interpolated coarse measurement grid. There is no change of numbers of 62 measurement grids in the current TS 38.161. The only change is how to pick up the position of 62 points from the measurement sphere in Figure 1.

Figure 1: Measurement grids of a sphere.

The key point here between interpolated coarse measurement grids and normal coarse measurement grids is that the positions of measurement grids are different from each other among the interpolated coarse measurement grids. Using constant step sized fine measurement grid with 266 test points as an example, it can be considered that we can pick one point from every fourth consecutive measurement test point, within θ=15° and φ=15° spherical angular area, in the 266-point measurement grid (i.e., 264/62=4, except the north pole and south pole, since they are used 4 times), and use it in one of the interpolated coarse measurement grids. We pick up the other one and use it in another one of the interpolated coarse measurement grids, and so on. Eventually, the overlapping of those four 62-point interpolated coarse measurement grids forms a 266-point fine measurement grid.

Therefore, we can find 4 different interpolated coarse measurement grids based on 266 measurement grids, each with 62 test points, which are different from each other, except north and south poles which are repeated in every coarse grid, and each can be considered as one interpolation of the 266-point fine measurement grid, and they are  orthogonal to each other.

To achieve the same accuracy level of TRP measurement results based on the 62-points interpolated coarse measurement grid compared with using 266-points fine measurement grids, we first use the 62-points interpolated coarse measurement grids in the OTA testing of TRP according to the procedure in TS 38.161 clause A.3.3 “Test procedure”. Repeat the OTA test four times by using each of the four interpolated coarse measurement grids, we can get the TRP1, TRP2, TRP3, and TRP4.

Then, the TRPave can be calculated as in the equation below:

TRPave=10log⁡(10TRP1/10+10TRP2/10+10TRP3/10+10TRP4/104)

The calculation of the TRP1/TRP2/TRP3/TRP4 based on EIRP measurements follows the TS 38.521-2 Clause M.4.2 and Clause M.4.3.

Similarly, we can use the same method to measure TRS by using four interpolated coarse measurement grids in the OTA test, then the TRSave can be calculated as in the equation below:

TRSave=10log4/(110TRS110+110TRS210+110TRS310+110TRS410

Observation 1: There are 266 and 62 measurement grids defined in current standard TS 38.161, a simple change of the location of 62 measurement grids based on 266 measurement grids can be made.

Observation 2: Define 4 of 62 interpolated measurement grids based on 266 measurement grids at the same time, the locations of each of four “62 interpolated measurement grids” can be different from each other.

Observation 3: Each OTA test time based on interpolated 62 measurement grids is reduced, and the accuracy of measurement results can be the same as using 266 measurement grids in OTA tests by averaging those results.

Proposal 1: Define 4 of 62-points interpolated measurement grids based on 266 measurement grids at the same time, the locations of each four of 62-points interpolated measurement grids is different from each other on the sphere, they can be considered orthogonal to each other.

Proposal 2: Averaging the measurement results from 4 of interpolated 62 measurement grids by

TRPave=10log⁡(10TRP1/10+10TRP2/10+10TRP3/10+10TRP4/104)

TRSave=10log4/(110TRS110+110TRS210+110TRS310+110TRS410

to achieve the same accuracy level of using 266 measurement grids during OTA tests.

## 2.2	Find Peak EIRP from OTA test based on hybrid measurement grids.

The difference of using the 266-point measurement grids and the 62-point measurement grids in (minimum) peak EIRP OTA test is that the test results of using a coarse measurement grid are less accurate. In order to achieve the same level of accuracy, we can use the below procedure for measuring peak EIRP and saving time and saving battery.

We can consider splitting the 266-point measurement grids into 8 or 16 (for example) equal spherical sectors, one such spherical sector is illustrated in Figure 3. Using 8 equal spherical sectors as one example, each sector is one-quarter of the half sphere.

Figure 2: One-eighth spherical sector of the fine measurement grid in a sphere.

Case 1:

Generally, the procedure of measuring EIRP using the proposed hybrid measurement grid can be described as:

- Use any 62-point (interpolated) coarse measurement grid to do the measurement for 3D sphere in all directions (or even a 32-point coarse measurement grid). Draw the CDF of it and find out which direction the EIRP is higher than a certain point/threshold of cdf, for example, 80%.

- Use the grids only within one spherical sector (8 or 16 spherical sectors based on the 266-point fine measurement grid) corresponding to the direction of high EIRP based on the last step, and do the EIRP test following only those grids in that spherical sector. Find out the highest EIRP as the peak EIPR.

After measurement, we can find out the direction of the (minimum) peak EIRP and the accuracy level of measurement results can be maintained compared with using a 266-point measurement grid.

Case 2:

The direction of peak EIRP found in step 1 may be in the border of one spherical sector, then the modification of step 2 can be described as:

- Combine two or four neighbouring spherical sectors as one large spherical sector. Measure the EIRP again based on the grids in the combined spherical sector.

After measurement, we can find out the direction of the (minimum) peak EIRP and the accuracy level of measurement results can be maintained compared with using a 266-point measurement grid as well.

It is better to split the 266-point measurement grid into 16 spherical sectors. Then, the total measurement points within the combined two or four neighbouring spherical sectors are still significantly reduced. Therefore, the total measurement time can be reduced significantly.

Observation 4: It is possible to split the fine measurement grids into a few sectors and using the fine measurement grids that only within a few sectors to increase the accuracy of OTA measurements for peak EIPR.

Observation 5: Using a coarse measurement grid in OTA test to measure peak EIPR first, then based on the direction of peak EIPR, apply corresponding sectors of a fine measurement grids to increase the accuracy of measured peak EIRP.

Observation 6: Combine the coarse measurement grids and a few sectors of fine measurement grids during OTA test can reduce the total measurement time and maintain high accuracy of peak EIPR measurement.

Proposal 3: Define the 8 or 16 sectors of a fine measurement grids, for example, define 8 or 16 sectors of 266 measurement grids.

Proposal 4:  Combine 62 coarse measurements girds and a few sectors of fine measurements during peak EIPR measurement. For example, using 62 measurement grids in OTA test to measure peak EIPR first, then based on the direction of peak EIPR, apply corresponding sectors of 266 measurement grids to measure the peak EIPR again to increase the accuracy of measured peak EIRP.

# Conclusions

In this contribution, we make the following observations and conclusions:

Observation 1: There are 266 and 62 measurement grids defined in current standard TS 38.161, a simple change of the location of 62 measurement grids based on 266 measurement grids can be made.

Observation 2: Define 4 of 62 interpolated measurement grids based on 266 measurement grids at the same time, the locations of each of four “62 interpolated measurement grids” can be different from each other.

Observation 3: Each OTA test time based on interpolated 62 measurement grids is reduced, and the accuracy of measurement results can be the same as using 266 measurement grids in OTA tests by averaging those results.

Proposal 1: Define 4 of 62-points interpolated measurement grids based on 266 measurement grids at the same time, the locations of each four of 62-points interpolated measurement grids is different from each other on the sphere, they can be considered orthogonal to each other.

Proposal 2: Averaging the measurement results from 4 of interpolated 62 measurement grids by

TRPave=10log⁡(10TRP1/10+10TRP2/10+10TRP3/10+10TRP4/104)

TRSave=10log4/(110TRS110+110TRS210+110TRS310+110TRS410

to achieve the same accuracy level of using 266 measurement grids during OTA tests.

Observation 4: It is possible to split the fine measurement grids into a few sectors and using the fine measurement grids that only within a few sectors to increase the accuracy of OTA measurements for peak EIPR.

Observation 5: Using a coarse measurement grid in OTA test to measure peak EIPR first, then based on the direction of peak EIPR, apply corresponding sectors of a fine measurement grids to increase the accuracy of measured peak EIRP.

Observation 6: Combine the coarse measurement grids and a few sectors of fine measurement grids during OTA test can reduce the total measurement time and maintain high accuracy of peak EIPR measurement.

Proposal 3: Define the 8 or 16 sectors of a fine measurement grids, for example, define 8 or 16 sectors of 266 measurement grids.

Proposal 4:  Combine 62 coarse measurements girds and a few sectors of fine measurements during peak EIPR measurement. For example, using 62 measurement grids in OTA test to measure peak EIPR first, then based on the direction of peak EIPR, apply corresponding sectors of 266 measurement grids to measure the peak EIPR again to increase the accuracy of measured peak EIRP.

# References

[1]	R4-2409931, WF for [111][338] TRP_TRS_MIMO_OTA