3GPP TSG-RAN WG4 Meeting #112                                                    R4- 2413196

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:		Nokia

Title:	Test methodology and radiated performance metric for FR1 NTN devices

Agenda item:		8.12.2.2

Document for:	Discussion

# Introduction

Based on the WF from RAN4 111 meeting [1], in this paper, we’d like to share our views of NR NTN OTA test related issues as listed below:

Issue 3-1-6: Usage scenarios for NR-NTN and IoT-NTN handheld UE Agreements - RAN4 further discuss prioritized usage scenarios. Free space only is not considered. - Option 1: Prioritize the browsing mode (with hand phantom) - Option 2: Prioritize the browsing mode (with hand phantom) and Free Space - Option 3: Prioritize browsing mode (with hand phantom) and talk mode (Head+Hand) - Option 4: Prioritize Head+Hand talk mode, hand only browsing mode and talk mode (new positioning guideline), and Free Space - Option 5: Prioritize Free Space mode only ### Sub-topic 3-2 UE performance metric Issue 3-2-1: Categorized performance metric for NR-NTN handheld Agreements - RAN4 should not adopt direction 1, other directions are FFS - Direction 1: Only single point performance metric, i.e., Peak EIRP/EIS - Direction 2: Single point + measured full sphere (integrated or CDF), e.g., - D2a, Peak EIRP/EIS+TRP/TRS, - D2b, Peak EIRP/EIS+ full Spherical coverage CDF - Direction 3: Single point + measured partial sphere (integrated or CDF), e.g., - D3a, Peak EIRP/EIS+ partial TRP/TRS (e.g., within selected 30, 60, 90, 180 degree range of angles), - D3b, Peak EIRP/EIS+ partial Spherical coverage CDF (e.g., within selected 30, 60, 90, 180 degree range of angles) - Direction 4: Only Full sphere (integrated TRP/TRS or Spherical coverage CDF) with/without weighting, - D4a, Spherical Coverage with EIRP-CDF and EIS-CCDF at [>50%]-ile - D4b, TRP/TRS or Weighted metric, e.g., weighted TRP/TRS (with larger weight for declared hemisphere) - Direction 5: Only Partial sphere (integrated or CDF) (e.g., within selected 30, 60, 90, 180 degree range of angles),

# Discussion

There are various NTN UE types and IoT-NTN types. The size of each type and the location of the antenna on the UEs are different. Some of the UE type has external antenna which is next to the UE, and some external antenna is too big which is cable connected to the UE and far enough from UE that the hand and head phantom does not have much impact on the antenna radiation patterns.

Figure 1 External antenna next to NTN UE

However, regardless of the UE types, user scenarios for NR-NTN and IoT-NTN handheld UEs require the talk mode to be enabled for basic communications. The browser mode is supported in most of PC3 UEs in the market nowadays. It is also very important for daily communications, but it is not mandatary. For the IoT NTN, most of the devices will be operating without human intervention, free space mode is enough.

In Figure 2, it is clear the talk mode and browsing mode only need to consider hand phantom during the test due to external antenna is too big which is cable connected to the UE and far enough from. UE

Observation 1: Talk mode to enable basic NTN communications. The browser mode is very important for daily communications, but it is not mandatary.

Observation 2: In the talk mode and browsing mode, only need to consider hand phantom during the test for the UEs with external antenna, which is too big, cable connected, and far enough from user.

Proposal 1: Option 4: Prioritize Head+Hand talk mode, hand only browsing mode and talk mode (new positioning guideline), and Free Space.

The antenna radiation direction of NTN UEs are mainly pointing to the sky during the communication with satellites, therefore, only measure the UE RF performance for partial sphere (up half sphere) would be enough. For the FR1 frequency range, normal TRP/TIS is the metric to measure the performance of UE due to using omni-directional antenna radiation pattern or half of omni-directional antenna radiation. Whereas in FR2 frequency range, the spherical coverage is normally used to measure the performance of UE due to using directional radiation antenna pattern. Therefore, TRP/TIS or spherical coverage should be both defined for different purposes.

Figure 2 external antenna far away from user

Observation 3: only measure the UE RF performance for partial sphere (up half sphere) would be enough for NTN and IoT NTN UEs

Observation 4: TRP/TIS is the metric to measure the performance of UE in FR1 frequency range. Whereas the spherical coverage is normally used to measure the performance of UE in FR2 frequency range.

Proposal 2: Direction 3: Single point + measured partial sphere (integrated or CDF), e.g., and define both D3a and D3b: D3a, Peak EIRP/EIS+ partial TRP/TRS (e.g., within selected 30, 60, 90, 180 degree range of angles); D3b, Peak EIRP/EIS+ partial Spherical coverage CDF (e.g., within selected 30, 60, 90, 180 degree range of angles).

However, for the external antenna, the radiation impact to the human body, i.e., Specific Absorption Rate (SAR) and Maximum Permissible Exposure (MPE), will be issues. Those need to be studied as well.

Proposal 3: Study the Specific Absorption Rate (SAR) and Maximum Permissible Exposure (MPE) issues for NTN UEs (not IoT NTN UEs), and define the corresponding requirements.

# Conclusions

In this contribution, we make the following observations and conclusions:

Observation 1: Talk mode to enable basic NTN communications. The browser mode is very important for daily communications, but it is not mandatary.

Observation 2: In the talk mode and browsing mode, only need to consider hand phantom during the test for the UEs with external antenna, which is too big, cable connected, and far enough from user.

Proposal 1: Option 4: Prioritize Head+Hand talk mode, hand only browsing mode and talk mode (new positioning guideline), and Free Space.

Observation 3: only measure the UE RF performance for partial sphere (up half sphere) would be enough for NTN and IoT NTN UEs

Observation 4: TRP/TIS is the metric to measure the performance of UE in FR1 frequency range. Whereas the spherical coverage is normally used to measure the performance of UE in FR2 frequency range.

Proposal 2: Direction 3: Single point + measured partial sphere (integrated or CDF), e.g., and define both D3a and D3b: D3a, Peak EIRP/EIS+ partial TRP/TRS (e.g., within selected 30, 60, 90, 180 degree range of angles); D3b, Peak EIRP/EIS+ partial Spherical coverage CDF (e.g., within selected 30, 60, 90, 180 degree range of angles).

Proposal 3: Study the Specific Absorption Rate (SAR) and Maximum Permissible Exposure (MPE) issues for NTN UEs (not IoT NTN UEs), and define the corresponding requirements.

# References

[1]	R4-2409931, WF for [111][338] TRP_TRS_MIMO_OTA