3GPP TSG-RAN WG4 Meeting #112                                                     R4- 2413197

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:		Nokia

Title:	FR1 dynamic MIMO OTA test methodology

Agenda item:		8.12.2.3

Document for:	Discussion

# Introduction

In this contribution we discuss the dynamic channel model generation and validation methodology based on the WF from RAN4 111 meeting [1]. In this paper, we’d like to share our views on below aspects:

Issue 4-1-1: Alignment of dynamic channel model parameters in different SDOs Agreements - RAN4 adopt the UMa and the UMi dynamic channel models defined in CTIA. FFS whether minor modification is needed. - RAN4 develop few new channel models is not precluded. Issue 4-2-3: Test system Dynamic-link-adaption verification Agreements - RAN4 should study a procedure to verify the consistency of different test equipment/test system on Scheduling Algorithm in dynamic channel model system. This can be considered as an additional aspect of system validation. Issue 4-2-4: Environmental condition for RAN4 FR1 dynamic MIMO OTA Agreements - RAN4 adopt the full path loss model from TR 38.901 for the UE Noise-limited environment condition. - With understanding of noise-limited is baseline for dynamic MIMO OTA in RAN4, the study of Interference-limited environment condition for RAN4 FR1 Dynamic MIMO OTA is not precluded.

# Discussion

  - Similarity of a dynamic channel in a span of time duration

For a dynamic channel scenario while UE is moving, it can be illustrated as a car is traveling from one spot to another spot in the map. The channel impulse responses along time t can be written as the below equation:

ht,τ=i=1Nait*δτ-τi*δθ-θi,  i= 0,1….N

where τ is the delay, and θ is the angle of arrival.

In each time slice t, the channel impulse response consists of a few delayed impulse responses and the equation can be simplified as

hτ=i=1Nai*δτ-τi*δθ-θi           i= 0,1….N

The dynamic channel based on a route, i.e. a time span, can be sectioned based on the similarity of the channel environment/surrounding between Tx and Rx. For each of the section, a typical channel impulse response can be used to represent such section of the dynamic channel. Therefore, a dynamic channel can be represented by a few typical channel impulse responses.

Observation 1: A dynamic channel can be represented by a few typical channel impulse responses.

  - Similarity level of the channel impulse response

As UE are moving, UE may experience a similar channel again if the environment along one road is similar to that along another road for a possible route of a UE.

For example, in the New York city, there are many skyscrapers. UE is possibly move from one street to another street (one physical location to another physical location) experience the very similar wireless channels. Therefore, the same static channel model, e.g., CDL model, can be used twice to represent two physical locations of a dynamic channel (a route) in separate time span based on a threshold of similarity of the channel parameters. It is obviously that the same CDL model does not need to apply twice in the test system.

The similarity level can be calculated based on the correlation of every two channel impulse responses. If the threshold would be defined at 0.8, then all the channel impulse responses with correlation higher than 0.8 can be represented by single one channel impulse responses. In case of using a few CDL models to represent one dynamic channel, the corelation of every two CDL models can be calculated. Based on the threshold at 0.8, it is possible to reduce the number of the CDL models to represent such dynamic channel.

Figure 1: Similar channel parameters in different physical location of a possible route.

Observation 2: The same static channel model can be used twice to represent a route of dynamic channel in separate time spans based on a similarity level of channel parameters.

Proposal 1: A similarity level of impulse response should be defined for using the same static channel, i.e., CDL model, to represent two physical locations of a route due to similar channel parameters in different physical locations.

During OTA test, the order of when to apply which CDL model for the test does not matter given the fact that the duplicated CDL model to represent the route of a dynamic channel can be removed from the test system. Therefore, the order of the CDL models to represent a route of dynamic channel become random.

Proposal 2: Using the static CDL models to represent a dynamic channel scenario, the order of CDL models can be randomly applied and no need to apply a CDL model twice during the OTA test.

# Conclusions

In this contribution, we make the following observations and conclusions:

Observation 1: A dynamic channel can be represented by a few typical channel impulse responses.

Observation 2: The same static channel model can be used twice to represent a route of dynamic channel in separate time spans based on a similarity level of channel parameters.

Proposal 1: A similarity level of impulse response should be defined for using the same static channel, i.e., CDL model, to represent two physical locations of a route due to similar channel parameters in different physical locations.

Proposal 2: Using the static CDL models to represent a dynamic channel scenario, the order of CDL models can be randomly applied and no need to apply a CDL model twice during the OTA test.

# References

[1]	R4-2409931, WF for [111][338] TRP_TRS_MIMO_OTA