3GPP TSG-RAN WG4 Meeting #112                                                     R4- 2413198

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:		Nokia

Title:	RF testing methodology for FR2 non-handheld UE that can transmit simultaneously with multi-panel

Agenda item:		8.13.2

Document for:	Discussion

# Introduction

In this contribution we discuss RF testing methodology for FR2 non-handheld UE that can transmit simultaneously with multi-panel based on the WF from last RAN4 meeting in Fukuoka, Japan [1]. In this paper, we’d like to share our views about on Test method for configured transmitted power as listed below:

Issue 1-4-1: AoA separation and UE orientation for EIRP PUMAX,f,c,k testing - Agreements: - FFS on AoA separation and UE orientation for EIRP PUMAX,f,c,k testing considering the following options - Option 1: Consider similar as Multi-Rx UE declaration approach for one AoA pair selection from the set of {30deg, 60deg, 90deg, 120deg, 150deg} and one proper UE orientation listed in TS38.101-2 for EIRP per TCI measurement - How to process the measurement data is FFS - Data processing should avoid repeated selection of peak EIRP per TCI. - Option 2: For the selection of AoA pair, one of AoA is at the beam peak direction of single CC operation without STxMP enable (from Beam Peak Search for MOP in 6.2 of TS 38.101-2), another AoA is decided by UE declared orientation (listed in TS38.101-2) and AoA pair from set of {30deg, 60deg, 90deg, 120deg, 150deg}. Issue 1-4-2: Measurement grid for EIRP PUMAX,f,c,k testing - Agreements: - RAN4 consider legacy PC1 and PC5 measurement grid for beam peak search defined in Annex M.2 of TS38.521-2 as the starting point. The modifications might be needed with further study. Issue 1-5-1: Skipping rule for EIRPmax testing - Agreements: - RAN4 agree on the skipping rule proposal for EIRPmax, i.e., “If the peak EIRP of single CC operation without STxMP (from Peak EIRP for MOP in 6.2 of TS 38.101-2) is smaller than EIRPmax - 3dB –additional margin , the corresponding EIRPmax verification can be skipped”. - Further check on the handling of the test case skip as part of conformance test. FFS on additional margin, such as systematic error of beam peak search, etc. - The slightly change of antenna radiation pattern of one panel when the other panel is activated if any, which might varry the test results of EIRPmax, can be compensated by addition margin. Issue 1-5-2: AoA separation and UE orientation EIRPmax testing - Agreements: - RAN4 to further study the AoA separation and UE orientation EIRPmax testing considering the following options: - Option 1: RAN4 to consider AoA offset = [30°] as the worst case. FFS on the UE orientation selection. - Option 2: For PUMAX,f,c (aggregated EIRP), re-use the UE declared AoA separation and orientation of Multi-RX. FFS on the details of data processing. - Option 3: Define a few AoA separations/offsets in the OTA test for EIRPmax verification. Reuse the orientation definitions in current standards. Issue 1-5-3: Measurement grid EIRPmax testing - Agreements: - RAN4 consider the legacy PC1 and PC5 measurement grid for beam peak search defined in Annex M.2 of TS38.521-2 as the starting point. The modifications might be needed with further study. Issue 1-6-1: Skipping rule for TRPmax testing - Agreements: - RAN4 agree on the skipping rule proposal for TRPmax, i.e., “If the peak TRP of single CC operation without STxMP (from TRP for MOP in 6.2 of TS 38.101-2) is smaller than TRPmax - 3dB – additional margin[], the corresponding TRPmax verification can be skipped”. - Further check on the handling of the test case skip as part of conformance test. FFS on additional margin, such as systematic error of beam peak search, etc. - The slightly change of antenna radiation pattern of one panel when the other panel is activated if any, which might vary the test results of TRPmax, can be compensated by addition margin. Issue 1-6-2: AoA separation and UE orientation TRPmax testing - Agreements: - RAN4 consider the following options for AoA separation and UE orientation TRPmax testing - Option 1: For PTMAX,f,c (total radiated power), re-use the UE declared AoA separation and orientation of Multi-RX - Option 2: The MOP, MPR, AMPR, TRPmax should be tested under same AoA pair for sTxMP. - Option 3: The worst-case conclusion for aggregated EIRP PUMAX,f,c testing should apply for aggregated TRP PTMAX,f,c, testing.

# Discussion

  - Views on EIRPmax and TRPmax testing

In RAN4 109 meeting, RAN4 has defined the configured transmitted power for simultaneous transmission to multiple directions (STxMP) and the changes has been implemented in 38.101-2. This requirement basically provides a maximum power output limitation of the STxMP in term of EIRP, TRP, and this is for both overlapping and non-overlapping cases of the two main beams without indicating the transmission angles of the two TX beams.

Considering the EIPRmax and TRPmax means the integrated EIRP and integrated TRP based on transmissions of the joint TCI states for two panels, and this is a reciprocal of receiving data streams from the same two panels based on two TCI states, i.e., Multi-Rx. Therefore, a similar test method for Multi-Rx requirement based on AoA pair set of {30deg, 60deg, 90deg, 120deg, 150deg} in Clause 7.3K.3 of 38.101-2 can be deployed.

However, for the max EIPR and max TRP of the joint transmission of two panels (STxMP), depend on the UE vendor design, it is not obvious to define a threshold and use it to calculate the percentage of how many angles of test passing the threshold for each AoA pair set, because this may be a very important parameter to separate the good and bad UEs in the market in terms of power saving, multi power class support, and so on.

Therefore, record the EIRPmax and TRPmax of joint transmission based on joint TCI states for each AoA pair set for the whole spherical test is a straightforward method. The final results can be illustrated as a similar table compared with Multi-Rx requirement.

| AoA separation (degrees) | EIPRmax (dBm) | TRPmax (dBm) |
| --- | --- | --- |
| 30 | highest measured value | highest calculated value |
| 60 | highest measured value | highest calculated value |
| 90 | highest measured value | highest calculated value |
| 120 | highest measured value | highest calculated value |
| 150 | highest measured value | highest calculated value |

Resue of the same set up of OTA test equipment is also very cost-efficient. There is no need to design another test method with different OTA test equipment set up to measure EIRPmax and TRPmax.

Observation 1: Record the EIRPmax and TRPmax of joint transmission based on joint TCI states for each AoA pair set for the whole spherical test is a straightforward method.

Observation 2: Resue of the same set up of OTA test equipment in multi-Rx is also very cost-efficient.

Proposal 1: Record the EIRPmax and TRPmax of joint transmission based on joint TCI states for each AoA pair set for the whole spherical test can be illustrated as a similar table compared with Multi-Rx requirement:

| AoA separation (degrees) | EIPRmax (dBm) | TRPmax (dBm) |
| --- | --- | --- |
| 30 | highest measured value | highest calculated value |
| 60 | highest measured value | highest calculated value |
| 90 | highest measured value | highest calculated value |
| 120 | highest measured value | highest calculated value |
| 150 | highest measured value | highest calculated value |

  - Views on separation and UE orientation for TRPmax

TRP is the integration of all the EIRP in all the 3D directions. There is no different of overlapping and non-overlapping in terms of TRP OTA test. However, during the OTA test of EIRP, the data are available, the calculation of TRP based on tested data is achievable.

Observation 3: TRPmax can be calculated based on EIRP OTA measurement data.

Proposal 2: No need to redefine TRP OTA tests, the TRPmax verification can be done together with EIRPmax verification.

  - Views on Skipping rule for TRPmax testing

For a full test of each UE with STxMP capability, the UE need to pass single Tx OTA tests for each panel, as well as pass the STxMP OTA test for using the two Tx. The antenna radiation pattern will be slightly different between only activating one panels and activating two panels at the same time.

It is also true that the impact of one antenna to the other antenna in FR2 frequency range is less significant compared with that in FR2 frequency range, because the PCB itself is the main radiator in the FR1 frequency range, but PCB radiation in FR2 frequency range is less significant. However, the impact of one activated panel to the other one will still change the measurement results, and it depends on each single design of the UE, i.e., the physical shape of antenna and the surroundings, the ground connections on the PCB, the matching components, etc., There are a huge amount of UEs from different vendors in the market, such impact of radiation patterns between two activated antennas vary significantly.

In the WF from last meeting [1], the addition margin is agreed to be added to avoid the slightly change of antenna radiation pattern of one panel when the other panel is activated. We suggest the addition margin is 3dB due the variety of UE designs in the market.

Considering the other fact that the real EIRPmax may fall in the gap of measurement grids due the OTA test, this is another level of measurement uncertainty, and this is the limitation of OTA test and it is not possible to avoid it. We suggest consulting OTA equipment vendors for a proper addition margin.

Observation 4: Addition margin is agreed to be added to avoid the slightly change of antenna radiation pattern of one panel when the other panel is activated.

Observation 5: There is huge amount of UEs from different vendors in the market, such impact of radiation patterns between two activated antennas can vary significantly.

Proposal 3: Suggest the addition margin is 3dB due the variety of UE designs in the market.

# Conclusions

In this contribution, we make the following observations and conclusions:

Observation 1: Record the EIRPmax and TRPmax of joint transmission based on joint TCI states for each AoA pair set for the whole spherical test is a straightforward method.

Observation 2: Resue of the same set up of OTA test equipment in multi-Rx is also very cost-efficient.

Proposal 1: Record the EIRPmax and TRPmax of joint transmission based on joint TCI states for each AoA pair set for the whole spherical test can be illustrated as a similar table compared with Multi-Rx requirement:

| AoA separation (degrees) | EIPRmax (dBm) | TRPmax (dBm) |
| --- | --- | --- |
| 30 | highest measured value | highest calculated value |
| 60 | highest measured value | highest calculated value |
| 90 | highest measured value | highest calculated value |
| 120 | highest measured value | highest calculated value |
| 150 | highest measured value | highest calculated value |

Observation 3: TRPmax can be calculated based on EIRP OTA measurement data.

Proposal 2: No need to redefine TRP OTA tests, the TRPmax verification can be done together with EIRPmax verification.

Observation 4: Addition margin is agreed to be added to avoid the slightly change of antenna radiation pattern of one panel when the other panel is activated.

Observation 5: There is huge amount of UEs from different vendors in the market, such impact of radiation patterns between two activated antennas can vary significantly.

Proposal 3: Suggest the addition margin is 3dB due the variety of UE designs in the market.

# References

[1]	R4-2409921, WF for [111][339] NR_FR2_OTA_afteroffline