3GPP TSG RAN WG4 Meeting #112		R4-2413199

Maastricht, NL, August 19th – 23rd, 2024

Source: 	Nokia

Title: 	UE RF requirements (MIMO Phase5)

Agenda item:       8.18.2

Document for:     Discussion

# 1	Introduction

New WID: NR MIMO Phase 5 in Rel-19 [1] has been agreed in RAN#102. The specific objectives in the WID RP-234007 are listed below. In this paper, we’d like to share our views on objectives 4 and 5 for RAN4 impacts:

The detailed objectives are as follows: RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

# 2	Discussion

2.1	Discussion for 3Tx

The general of idea to support 3Tx is based on 4Tx in the current 3GPP standards. In RAN1, the offline discussion before the meeting shows that the 3Tx case can be considered as a special case of 4Tx with one Tx muted. From this respect, the current standard can be considered supporting 3Tx already in terms of SRS transmission, non-codebook based transmission and codebook based transmission.

In terms of SRS antenna switch for 3Tx, 3Tx UE might be considered as an evolution from 2Tx UE. The antenna switching time should be between 2T4R and 2T8R which are supported for 2Tx UE in the current specification. Therefore, the current specification should be also reused for 3Tx antenna switching. Therefore, it seems there is no impact to RAN4 RF specifications right now related to 3Tx. We should wait for RAN1 to further develop 3Tx specifications.

Observation 1: The 3Tx case can be considered as a special case of 4Tx with one Tx muted based on the current standard

Observation 2: The 3Tx antenna switch time should be between 2T4R and 2T8R which are supported for 2Tx UE in the current specification.

Proposal 1: There is no impact for RAN4 UE RF specification to support 3Tx at the moment. Wait for RAN1 to further develop 3Tx specifications.

2.2 Discussion for UL only TRP

Asymmetric DL sTRP/UL mTRP deployment scenarios can be illustrated as in the picture below, where the DL transmission is from only a single TRP but one or more “UL-only TRPs” are deployed, potentially close to the cell edge in order to improve coverage.

The current discussion for such scenario is mainly related to UE capability and scheduling for UL transmissions towards one or more than one TRP. There is no difference from RF point of view that UE transmits to one TRP or more than one TRPs based on RF specifications. It seems that there is no impact to RAN4 RF specifications right now related to Asymmetric DL sTRP/UL mTRP deployment scenarios. We should wait for RAN1 to further develop it.

Figure 1 Asymmetric DL sTRP/UL mTRP scenario

Observation 3: The current discussion for such scenario is mainly related to UE capability and scheduling for UL transmissions toward more than one TRP. There is no RF impact foreseen.

Proposal 2: There seems no UE RF impact for asymmetric DL sTRP/UL mTRP deployment scenarios, wait for RAN1 to further develop it.

# 3	Conclusions

In our contribution, related uplink 3Tx topics are discussed. Here is the list of our proposals:

Observation 1: The 3Tx case can be considered as a special case of 4Tx with one Tx muted based on the current standard

Observation 2: The 3Tx antenna switch time should be between 2T4R and 2T8R which are supported for 2Tx UE in the current specification.

Proposal 1: There is no impact for RAN4 UE RF specification to support 3Tx at the moment. Wait for RAN1 to further develop 3Tx specifications.

Observation 3: The current discussion for such scenario is mainly related to UE capability and scheduling for UL transmissions toward more than one TRP. There is no RF impact foresee.

Proposal 2: There seems no UE RF impact for asymmetric DL sTRP/UL mTRP deployment scenarios, wait for RAN1 to further develop it.

# References

- RP-234007, “New WID: NR MIMO Phase 5,” RAN#102, Edinburgh, Scotland, Dec 11-15, 2023

# RAN1 Previous Agreements

- RAN1 #116 agreements

Agreement

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for single-layer transmission.

13100,13010,13001

Agreement

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for two-layer transmission.

13100100, 13100001, 13 001001

Agreement

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for three-layer transmission.

13100010001

Agreement

For SRS configuration supporting codebook-based UL transmission by a 3TX UE, down-select one of

- Alt1 – Support configuration of X 4-port SRS resources in a resource set where one the ports is muted

- Alt2 – Support configuration of X SRS resources with equal/unequal number of ports (e.g. 2 + 1 or 1 + 1 + 1) in a resource set,

The value for X is FFS, and it will be determined according to the selected alternative.

Agreement

For a 3TX UE, down-select one of the following options for the number of PTRS ports,

- Option-1: A single PTRS port is supported.

- Option- 2: Up to 2 PTRS port may be configured.

Agreement

For a 3-antenna-port codebook-based UL transmission, study PTRS-DMRS association.

Agreement

For a 3-antenna-port codebook-based UL transmission, study power split for each port of SRS and PUSCH.

Agreement

For codebook-based uplink transmission by a 3TX UE, support full-power Mode 0, subject to UE capability.

Conclusion

There is no consensus in RAN1 to support antenna switching for 3TX UE in Rel-19

Agreement

For performance evaluation of 3TX UE, adopt the following Table as the reference EVM for LLS evaluation

- Companies may provide additional evaluation results per their case of interest

- LLS is optionally used for 3Tx UL evaluation, if needed

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 3.5 GHz |
| Waveform | CP-OFDM |
| SCS | 30 KHz |
| System bandwidth | 20 MHz, 100 MHz |
| Scheduled PRBs | 5, 25, 50, 260 PRBs |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (2,2,2,1,1,2,2) with (dH , dV ) = (0.5, 0.5)λ |
| UE speed | 3 Km/h |
| Number of Layers | Adaptive, Fixed (reported by company) |
| AMC | Adaptive, Fixed (reported by company) |
| DMRS configuration | Type 1; 1 front loaded + 1 additional symbol |
| Channel estimation | Real |
| Channel Model | CDL-A (30ns), CDL-B (100ns), CDL-C (300ns) |

Agreement

For performance evaluation of 3TX UE, adopt the following Table as the reference EVM for SLS evaluation.

- Companies may provide additional evaluation results per their case of interest.

Note: The considered EVM is to be used as a baseline set of assumption for future potential studies related to 3TX.

| Parameter | Value |
| --- | --- |
| Frequency range | 3.5 GHz |
| Multiple access | OFDMA |
| Numerology | 14 CP-OFDM symbol slot SCS , 30 KHz |
| Scenario | eMBB: Dense Urban (200m), 3.5GHz Outdoor FWA: UMa (500m), 3.5GHz |
| UE Outdoor/Indoor (%) | eMBB: 80%, 20% Outdoor FWA: 100%, 0% |
| System bandwidth | 20 MHz, 100 MHz |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 Optional: Classical: two 8x1 xpols, 4λ apart; 4 TXRUs tilt=[104°] |
| gNB antenna radiation pattern parameters | - Outdoor/Indoor Per 38.901, Table 7.3-1 |
| gNB receiver noise figure | 5dB |
| gNB receiver | MMSE-IRC |
| gNB scheduler | Single user with proportional fair |
| Modulation | - Up to 64 QAM - Up to 256QAM |
| MIMO scheme | SU-MIMO with rank adaptation |
| UE speed | 3 Km/h |
| UE antenna config | eMBB: - Xpol+1pol; isotropic ULA - Xpol+1pol; 110°, 4 dBi Outdoor FWA: - Xpol+1pol; isotropic ULA - 3 directional 1pol: 110°, 4 dBi |
| Traffic model | - FTP model 1: Packet size 500KB, RU= 50% and suggested low/high RU of values of 20% and 70% - Full buffer (optional) |
| Suggested benchmarking | Rel-15 2Tx non-coherent |
| Precoder granularity | Wideband |
| Power control | Open loop, - alpha = 0.8 - P0 = -50, -80 dBm to be selected according to the deployment scenario |
| UE power rating | eMBB: 23 dBm, UL FPTx mode 0 or Rel-15 power scaling Outdooe FWA: 31 dBm, UL FPTx mode 0 |
| Metric | UL mean-user throughput, 5%-ile and 95%-ile UPT |

Agreement

For performance evaluation of 3TX UE, consider following reference configurations,

- A linear array (1D) of single-polarized antenna configuration with a spacing of 0.5λ,

  - For example:		 |←0.5λ→|←0.5λ→|

- A configuration of a cross-polarized and a single-polarized antennas,

  - For example:		 ×←0.5λ → ⁄

Agreement

For SRS configuration supporting codebook-based UL transmission by a 3TX UE, one SRS resource set is configured for single TRP operation.

Agreement

For codebook-based transmission by a 3TX UE,

- Only PUSCH antenna ports 1000, 1001, 1002 are used

- Option- 2: Subject to UE capability, up to 2 PTRS ports may be configured in PTRS-UplinkConfig,

  - FFS whether a single bit or 2 bits are used for PTRS-DMRS association indication.

Above is only for single panel transmission.

- RAN1 #116bis agreements

Agreement

To support codebook-based UL transmission by a 3TX UE, the agreed rank1 precoders in RAN1#116 can also be used when transform precoding is enabled (DFT-s-OFDM ).

Agreement

To indicate precoding information for codebook-based UL transmission by a 3TX UE,

- Reuse legacy TPMI indication framework where TPMI and TRI are jointly indicated

- TPMI field is 2 or 3bits for 3-antenna-port transmission

  - For maxRank equals to 1, TPMI field is 2 bits for DFT-s-OFDM and CP-OFDM

  - For maxRank equals to 2 or 3, TPMI field is 3 bits for CP-OFDM

Agreement

For SRS configuration supporting codebook-based UL transmission by a 3TX UE, support Alt1,

- Alt1: Support configuration of X 4-port SRS resources in a resource set where one the ports is muted

  - FFS muting mechanism

where X can be up to 2, subject to UE capability.

Agreement

- For codebook-based UL transmission by a 3TX UE, when 2 PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, PTRS-DMRS association indication is as follows:

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |

- Note: PUSCH antenna port 1000 and 1002 in indicated TPMI(s) share PT_RS port 0, and PUSCH antenna port 1001 is associated with PT_RS port 1

- Number of bits used for the indication

  - 1 bit

Agreement

For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, support the following for muting one of the ports of the configured 4-port SRS resource,

- Option 3: Always a same port is muted, e.g., the 4th port

Agreement

For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, UE splits a linear SRS power equally across the 3 unmuted antenna ports of the 4-port SRS resource.

Agreement

For 3-port codebook-based PUSCH transmission for a 3TX UE, scale factor s should be the ratio of the number of antenna ports with a non-zero PUSCH transmission power to 3 (except for full-power Mode 0).

- FFS: Whether specification needs to be updated to reflect the above

Agreement

For codebook-based UL transmission by a 3TX UE, when 1 PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig, PTRS-DMRS association indication is as follows:

- Alt2: 2-bit indication

PTRS-DMRS association when 1 PT-RS port is configured

| Value | DMRS port |
| --- | --- |
| 0 | 1st scheduled DMRS port |
| 1 | 2nd scheduled DMRS port |
| 2 | 3rd scheduled DMRS port |
| 3 | 4th scheduled DMRS port Reserved |

Agreement

For a 3TX UE, support Rel-17 M-TRP PUSCH repetition where,

- Two SRS resource sets, each with up to 2 of 4-port SRS resources are configured,

Note: The configured 4 port SRS resources are used to enable 3-port SRS transmission

- RAN1 #117 agreements

Agreement

Update the agreement made in RAN1 #116bis as the following.

Agreement For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, support the following for muting one of the ports of the configured 4-port SRS resource, Option 3: Always a same port is muted, e.g., i.e., the 4th port

Agreement

For codebook-based M-TRP PUSCH repetition by a 3TX UE, scheduled by DCI format 0_1/0_2,

- Reuse Rel-17 M-TRP PUSCH repetition design, where the second precoding information field only indicates TPMI index, and applies same rank as indicated by the first precoding information field.

Agreement

For codebook-based M-TRP PUSCH repetition by a 3TX UE, scheduled by DCI format 0_1/0_2,

- Introduce new tables as Table I, II, III for the second precoding information field, for maxRank=1 or 2 or 3, respectively.

  - Table I: Second precoding information for 3 antenna ports if maxRank=1

| Bit field | codebookSubset=NonCoherent |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |
| 3 | Reserved |

  - Table II: Second precoding information for 3 antenna ports if maxRank=2

| Bit field | codebookSubset=NonCoherent |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |
| 3 | 1 layer: Reserved |
| 0 | 2 layer: TPMI=0 |
| 1 | 2 layer: TPMI=1 |
| 2 | 2 layer: TPMI=2 |
| 3 | 2 layer: Reserved |

  - Table III: Second precoding information for 3 antenna ports if maxRank=3

| Bit field | codebookSubset=NonCoherent |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |
| 3 | 1 layer: Reserved |
| 0 | 2 layer: TPMI=0 |
| 1 | 2 layer: TPMI=1 |
| 2 | 2 layer: TPMI=2 |
| 3 | 2 layer: Reserved |
| 0 | 3 layer: TPMI=0 |
| 1~3 | 3 layer: reserved |

Agreement

For codebook-based M-TRP PUSCH repetition by a 3TX UE, for indication of PTRS-DMRS association,

- When 1 PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig, reuse Rel-17 multi-TRP TDM repetition,

- When 2 PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, and maxRank = 2 or 3,

  - A second PTRS-DMRS association field (1 bit) is used to indicate the association between PTRS ports and DMRS ports for the 2nd SRS resource set (i.e.,2nd TRP).

Agreement

For codebook-based UL transmission by a 3TX UE, subject to its capability,

- A 3TX UE may report a maximum number of 3 layers

- A 3TX UE may report a maximum number of SRS ports of up to 3

Note: SRS resource definition is not changed nor the number of SRS ports in the SRS resource.