3GPP TSG-RAN WG4 Meeting#112							         			R4-2413200

Maastricht, Netherland, 19 – 23 August 2024

Title: 	On NS_27 A-MPR for larger than 40MHz UL CBW for n48

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.12.2	UE RF requirements	[NR_bands_CA_ENDC_R19_BWs-Core]

Document for:	Approval

# 1	Background

In RAN#104, a new band BW basket [1] was agreed upon. The only request made at the time is for the support of larger UL CBW for Band n48. In this contribution, we discuss the required work and the potential impact of NS_27.

# Discussion

## 2.1 New CBW support request

Below, we extracted the table of the new CBW request in [1].

| Band | Channel bandwidth and SCS | Contact name, company | Other supporting companies | Justification |
| --- | --- | --- | --- | --- |
| n48 | 50 MHz (15, 30 and 60kHz SCS) 60, 70, 80, 90, 100 MHz (30 and 60 kHz SCS) | Ruoyu Sun, Cable Labs | Charter, Comcast, Samsung | This channel BWs are already supported in DL, they should be supported in UL as well, see RP-241541. |

Observations: The request aims to provide support from 50MHz up to 100MHz CBW in UL compared to the current maximum UL CBW of 40MHz. However, it seems that the support of 50MHz CBW with 30kHz SCS is not requested.

Proposal: Proponent should clarify whether 50MHz CBW at 30kHz SCS is needed or not.

## 2.2 A-MPR for NS_27 with larger CBW

The following tables are the extract of the NS_27 related Band n48 requirement with up to 40MHz UL CBW for the cases region requiring the highest backoff (region A). Tables 2 and 3 show that for CBW above 20MHz a significant region A7 A-MPR >10dB applies for an increasing number of allocation and channels. This is due to the stringent -40dBm/MHz requirement at 20MHz offset below and above the band that is 150MHz wide as shown in Table 1.

Table 1: Additional requirements for "NS_27"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20, 30, 40 |  |
| 9 kHz – 3530 MHz | -40 | 1 MHz |
| 3530 MHz – 3540 MHz | -25 |  |
| 3710 MHz – 3720 MHz | -25 |  |
| 3720 MHz – 12.75 GHz | -40 |  |

Table 2: NS_27 A-MPR for modulation and waveform type

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | QPSK | 4.5 | 6 | 4 | 4 | 4 | 4 | 10.5 | 4 |
| CP-OFDM | QPSK | 5.5 | 7 | 6 | 4 | 6 | 4 | 11.5 | 4 |
| NOTE 1: The backoff applied is max (MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2 |  |  |  |  |  |  |  |  |  |

Table 3: A-MPR for NS_27

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS | RBend*12*SCS | LCRB*12*SCS | A-MPR |
| 15 MHz | 3557.5 ≤ FC < 3562.5 | <1.8 MHz |  |  | A3 |
|  | 3687.5 < FC ≤ 3692.5 | >11.52 MHz |  |  |  |
| 15 MHz | 3562.5 ≤ FC < 3567.5 | ≤1.08 MHz |  | <1.44 MHz | A4 |
|  | 3682.5 < FC ≤ 3687.5 |  | ≥13.22 MHz |  |  |
| 20 MHz | 3560 ≤ FC < 3570 | <3.6 MHz |  |  | A5 |
|  | 3680 < FC ≤ 3690 | >12.96 MHz |  |  |  |
| 20 MHz | 3570 ≤ FC < 3580 | ≤2.16 MHz |  | <1.44 MHz | A6 |
|  | 3670 < FC ≤ 3680 |  | ≥16.92 |  |  |
| 30 MHz | 3565 ≤ FC < 3585 | < 7.38MHz |  |  | A7 |
|  |  | ≥ 7.38MHz ≤24.48MHz |  | ≥ 15.3MHz | A2 |
|  |  |  |  | < 15.3 MHz | A1 |
|  |  | ≥24.48MHz |  | < 2.7 MHz | A7 |
|  |  |  | > 19.44 MHz |  | A7 |
|  | 3665 < FC ≤ 3685 |  | ≤19.44MHz ≥3.24 MHz | ≥ 15.3 MHz | A2 |
|  |  |  |  | < 15.3 MHz | A1 |
|  |  |  | <3.24 MHz | < 2.7MHz | A7 |
|  | 3585 ≤ FC ≤ 3665 | ≤[3.96] MHz |  | < 1.44MHz | A8 |
|  |  |  | ≥24.48MHz |  | A8 |
| 40 MHz | 3570 ≤ FC < 3600 | <11.34 MHz |  |  | A7 |
|  |  | ≥11.34 MH, ≤31.0 MHz |  | ≥18 MHz | A2 |
|  |  |  |  | <18 MHz | A1 |
|  |  | >31.0 MHz |  | <3.6 MHz | A7 |
|  | 3650 < FC ≤ 3680 |  | >24.48 MHz |  | A7 |
|  |  |  | ≤24.48 MHz, ≥6.48 MHz | ≥18 MHz | A2 |
|  |  |  |  | <18 MHz | A1 |
|  |  |  | <6.48 MHz | <3.6 MHz | A7 |
| 40 MHz | 3600 ≤ FC ≤ 3650 | ≤6.12 MHz |  | <1.44 MHz | A8 |
|  |  |  | ≥ 32.76 |  |  |

Further increasing the UL CBW will further reduce the number of channels that do not have their ACLR1 region overlapping with the -40dBm/MHz requirement, which benefit from the reduced A-MPR of 4dB of region A8:

- For 40MHz the region is 3600MHz ≤ FC ≤ 3650MHz and based on the 30MHz and 40MHz progression for this region,

- For 50MHz this region would be 3605MHz ≤ FC ≤ 3645MHz

- For 90MHz this region would reduce to 3625MHz ≤ FC ≤ 3625MHz thus one channel

- There would be no A8 relaxed case for 100MHz. Still with increased CBW the PSD reduces so some improvement may be possible.

- Overall it is to be seen how many wider channels with wide allocation bandwidth would see a reasonable A-MPR to be more efficient than smaller channels.

Observation: with increased CBW beyond 40MHz, the number of channels and allocation that are not affected by the -40dBm/MHz stringent emission requirement reduces significantly, while the UE testing becomes significantly more complex.

Proposal:

- RAN4 studies Band n48 NS_27 A-MPR for 50, 60, 70, 8, 90 and 100MHz CBW

- Based on the number of channels and allocations benefiting from moderate A-MPR, it may be decided to still limit the UL CBW below 100MHz to avoid complex UE testing for limited user benefit in the field.

# Conclusion

In this contribution, we discussed the impact of NS_27 on the A-MPR for UL CBW > 40MHz, and make the following proposal.

Proposal: Proponent should clarify whether 50MHz CBW at 30kHz SCS is needed or not.

Proposal:

- RAN4 studies Band n48 NS_27 A-MPR for 50, 60, 70, 8, 90 and 100MHz CBW

- Based on the number of channels and allocations benefiting from moderate A-MPR, it may be decided to still limit the UL CBW below 100MHz to avoid complex UE testing for limited user benefit in the field.

# References

[1] RP-241684 New WID: Basket WID on adding channel bandwidth support to existing NR bands, Ericsson, RAN#104