3GPP TSG-RAN WG4 Meeting #112                                                                 R4-2413202

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Samsung

Title:                        Discussion on multi-Rx remaining issues

Agenda item:			5.13.1

Document for:	Discussion

1. Introduction

In this paper, we would like to share our understandings on the remaining issues for Rel-18 multi-Rx just for measurement restriction.

2 Discussion

The measurement restriction issue has been discussed for a long time, but no identified conclusions achieved. The Spec. was captured as below

9.5.5.2 Measurement restriction for CSI-RS based L1-RSRP For FR2-1, when the CSI-RS for L1-RSRP measurement on the one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC, UE supporting [multi-Rx capability] according to the conditions described in clause 3.6.y shall be able to measure the CSI-RS for L1-RSRP measurement without restriction when the following conditions are met: - Both the CSI-RS for L1-RSRP and the other CSI-RS are not in any CSI-RS resource set configured with repetition ON, and - Resources of the active TCI states for two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report, and - [The two CSI-RSs and both PDSCHs are overlapped on the same OFDM symbol, and] - The CSI-RS for L1-RSRP has same QCL source as the active TCI state of one [PDSCH], and the other CSI-RS has same QCL source as the active TCI state of the other [PDSCH], and Editor’s note 1: FFS the condition CSI-RS(s) and PDSCH(s) are overlapped on the same OFDM symbol(s). Editor’s note 2: FFS how to capture UE is activated with multi-Rx operation.

From our understanding,

- The measurement restriction is just for RSs (the two CSI-RSs are overlapped) based measurement, which is not relevant for data scheduling. GBBR can be used to determine which beam pair can be recieved simultanesouly. Then, the same beam pair can be used for the following RSs simultaneous reception, and at this time, the QCL association can be seen as a bridge. Besides, if we introduce “scheduling” into the “measurement”, the issues may become complicated.

- Actually, from configuration perspective, the case [The two CSI-RSs and both PDSCHs are overlapped on the same OFDM symbol] can exist, however, it is a corner and rare case. Since from RAN1 perspective, the data has high priority, if the CSI-RS is overlapped with PDSCH, the CSI-RS will become less important.

- Furthermore, from another aspect, the logic is a little vague. Since based on the GBBR procedure, when the GBBR is configured/triggered for the UE, the UE to activate multiple panels/Rx beams to perform measurement based on the received RSs to find the DL beam pair (s) from differnet directions with the good measured L1-RSRP values/best values, then report them to NW. After the report, once the pair of the DL RSs act as QCL sources in active TCI states, UE can use the Rx beams corresponding to the RSs for DL PDSCH reception. So, if the UE is fully aware of the information that which Rx beam pair can be used to receive PDSCH simultaneously, why to measure the CSI-RS overlapped with the PDSCH, since theoretically, if the two resources are overlapped, the Rx characteristics of them is the same. In this sense, linking them together will make the situation unclear

From this,

Proposal 1: The measurement restriction is just about the measurement when the two CSI-RSs are overlapped, which is not relevant for data scheduling

Proposal 2: Remove [The two CSI-RSs and both PDSCHs are overlapped on the same OFDM symbol, and]

3 Conclusion

In this contribution, we provided our viewpoints to requirement impact analyses on multi-Rx measurment restriction. The following observations and proposals are obtained:

Proposal 1: The measurement restriction is just about the measurement when the two CSI-RSs are overlapped, which is not relevant for data scheduling

Proposal 2: Remove [The two CSI-RSs and both PDSCHs are overlapped on the same OFDM symbol, and]