3GPP TSG-RAN WG4 Meeting #112 	R4-2413209

Maastricht, NL, August 19th – 23th, 2024

Agenda Item: 8.19.2.1

Source: MediaTek inc.

Title: Discussion on SBFD

Document for: Discussion

# Introduction

The work item (WI) for Evolution of NR duplex operation: Sub-band full duplex (SBFD) has the following objectives [1]:

The objectives are as follows: - For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In this contribution, we focus on highlighting the impacted RRM specification and whether new RRM requirements are needed to handle the new SBFD operation and CLI handling mechanism.

# Discussion

The discussion on the RRM impact for the evolution of NR duplex operation for SBFD is provided in this section. This section provides high level discussion for both agendas: (i) CLI handling; and (ii) SBFD operation.

# Discussion on the RRM impact for CLI handling mechanisms

In legacy 5G NR TDD, the RRM specification cover the CLI measurements delay and accuracy for SRS-RSRP and CLI-RSSI. The UE reports these measurements to the gNB to help the dynamic allocation of uplink/downlink in time. Nevertheless, these measurements are based on L3 measurements, i.e. it requires L3 filtering, which has longer measurement delay compared to L1 measurements. Therefore, in SBFD Rel-19 WID objective, new CLI measurements and reporting shall be introduced for L1 based UE-to-UE CLI with new measurement quantities, such as L1-SRS-RSRP, L1-CLI-RSSI. However, these measurements quantities and framework should be defined by RAN1 WG while measurements accuracy and delay should be defined by RAN4 RRM. Hence, RAN4 might wait for RAN1 to complete the framework design for these new measurements.

Observation 1: The measurement delay for the new measurement quantities, such as L1-SRS-RSRP, L1-CLI-RSSI, should be faster compared to the existing measurement delay for SRS-RSRP.

Observation 2: The measurement framework for the new measurement quantities should be defined by RAN1 while the measurement accuracy and delay are specified by RAN4.

- RAN4 RRM should wait for RAN1 WG to define the new measurement quantities and framework before defining the new RRM requirements.

# Discussion on the RRM impact due SBFD operation

In SBFD operation UE remains a half-duplex node. This implies that in SBFD symbols, the UE can perform either DL reception (on DL subbands) or UL transmission (on UL subband), but not both. To avoid collisions between DL receptions and UL transmissions, the network must provide a way for UE to determine the link direction on SBFD symbols, i.e., whether the UE transmit or receive in a SBFD symbol.

Before assessing which measurements are impacted with the SBFD operation, RAN4 RRM should take into consideration the agreements made in RAN1. Selected RAN1 agreements related to the measurements are provided below:

In RAN1#117, the following agreements were made. SBFD TX/RX/measurement procedures Agreement Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission - FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. Agreement For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured)

Measurements based SSB:

Based on the agreement made in RAN1 that the SSB is prioritized over the UL transmissions, hence the SSB signals are not impacted by the SBFD operation from the UEs within the serving cell. Thus, serving cell measurements based SSB are not impacted by the SBFD operation.

Observation 3: The SSB resources are prioritized over UL transmission and hence there RLM/BFD/RRM measurements based SSB are not impacted by the SBFD operation.

Measurements based CSI-RS:

Based on the agreement made in RAN1 that the CSI-RS can be overlapped with SBFD subbands and the CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware. Hence, CSI-RS based measurements are impacted with the SBFD operation. Besides, as stated in the scope of the WID, for SBFD operation in a slot defined as downlink and/or flexible by tdd-UL-DL-ConfigurationCommon, uplink transmission is within uplink subband only whereas downlink reception is within downlink subband(s) only. Therefore, the CSI-RS based measurements are based on the CSI-RS frequency resources within DL usable PRBs. Figure 1 depicts the configured CSI-RS resources, which are overlapped with SBFD subbands. The figure also depicts the CSI-RS resources mapped within DL usable PRBs.

Observation 4: The configured CSI-RS resources can be overlapped with SBFD subbands and hence the CSI-RS based measurements might be impacted.

Figure 1: depicts the configured CSI-RS resources overlapped with SBFD subbands.

Based on RAN1 agreements and the observation above, whether the following core requirements are impacted due to SBFD operation are summarised below. It should be highlighted that some of the following impacted scenarios might require simulation to define the new requirements.

- Cell detection: this is based on PSS/SSS detection latency and given that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission, hence, no impact to existing requirements.

- PBCH reading (basic SI reading time): given that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission, hence, no impact to existing requirements.

- PBCH-DMRS time index reading: Similarly to PBCH reading, the PBCH-DMRS is not impacted due to the SBFD operation.

- SSB based RLM and BFD: These measurements are not impacted because SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission.

- CSI-RS based RLM and BFD: The channel bandwidth might be impacted due to SBFD operation, hence, RAN RRM should check whether there is a need to introduce new requirements.

- SSB based L3 measurements (RSRP/RSRQ/SINR): SSB based RRM measurements depend on the SS signal to determine the power level, and given that the SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission, hence no impact.

- SSB based L1-RSRP: the measurements depend on the SS signal to determine the power level, and given that the SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission, hence no impact.

- CSI-RS based L1-RSRP: given that the channel bandwidth might be reduced due to SBFD operation and these measurements depend on the entire CSI-RS BW, hence they are impacted because part of the BW is used for UL configuration.

Hence, the scenarios that require study to help defining the new requirements are summarised below:

| The scenario | Whether it requires simulation or not? |
| --- | --- |
| Cell detection | No |
| PBCH reading | No |
| PBCH-DMRS time index reading | No |
| SSB based RLM and BFD | No |
| CSI-RS based RLM and BFD | Yes |
| SSB based RRM (RSRP/RSRQ/SINR) | No |
| SSB based L1-RSRP | No |
| CSI-RS based L1-RSRP | Yes |

- RAN4 should study and define requirements for: (1) CSI-RS based RLM and BFD, (2) CSI-RS based L1-RSRP.

# Summary

In this contribution, we have the following proposals:

Proposal 1: RAN4 RRM should wait for RAN1 WG to define the new measurement quantities and framework before defining the new RRM requirements.

Proposal 2: RAN4 should study and define requirements for: (1) CSI-RS based RLM and BFD, (2) CSI-RS based L1-RSRP.

# References

- RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June 2024.