3GPP TSG-RAN WG4 Meeting #112 	R4-2413210

Maastricht, NL, August 19th – 23th, 2024

Agenda Item: 8.24.2

Source: MediaTek inc.

Title: Discussion on XR (eXtended Reality) for NR Phase 3

Document for: Discussion

# Introduction

The work item (WI) for XR (eXtended Reality) for NR Phase 3 has the following objectives [1]:

The Rel-19 XR ph3 objectives are as follows: - Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2]. - Note: Check in RAN#105 (check also other WG involvement if needed). - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. - Note: LCP implementation complexity need to be taken into account when evaluating solutions. - Note: Check in RAN#105 - Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - If justified, define a mechanism for transmitter to inform the receiver of SN gap (or missing SNs) in PDCP. - Specify Core requirements related to the above objectives as necessary [RAN4]

In addition, RAN1 has sent the following LS [3]:

RAN1 discussed UE assistance information related to measurements occasions and related to channel conditions, traffic, UE mobility. The following agreement and conclusions were agreed: Agreement RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study: - FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme. Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information. Conclusion RAN1 does not further discuss new UE assistance information related to channel conditions, traffic, UE mobility. Conclusion There is no consensus in RAN1 to support UE assistance information related to measurements occasions. The reason for this situation is lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). It is up to other working groups to trigger further work in RAN1 on UE assistance information.. Particularly, it was discussed whether impact on RRM performance from skipping measurement occasions may be reduced if UE sends additional information: - Information about the maximum number of MGs/SMTC with restrictions that can be skipped within a time period. - Information about the patterns of gap(s)/restriction(s) where skipping is feasible or acceptable. # Actions To RAN4 ACTION: RAN1 kindly asks RAN4 to consider the above information into account and decide whether or not to introduce any UE assistance information related to measurement occasions.

In this contribution, the potential directions that leads to enhance the gap cancellation mechanisms are discussed then other RRM impact issues are raised, which requires some studies.

# Discussion

On one hand, the typical characteristics of XR DL and UL packet traffic is periodic and bursty with a tight delay budget [2]. On the other hand, measurement gaps are periodic with DL/UL data interruption duration of up to 6 ms. As shown in Figure 1, when the XR packet arrival timing overlaps with RRM measurement occasions, the XR transmission/reception will be impacted and therefore causing reduced system capacity.

Figure 1 Periodic XR traffic overlapping with RRM measurement occasions diminishes system capacity

According to the WID objectives, RAN4 RRM should specify enhancements allowing transmission/reception in the gap occasions or scheduling restrictions slots/symbols. Hence, in this contribution, we provide potential directions for completing this objective.

# Discussion on potential solutions

UE assistance information:

In existing legacy NR spec, the UE assistance information (UAI) is a special corresponding message to the RRC reconfiguration, which is sent from the UE to the NW, in which, the UE inform the NW with its internal status. In addition, not all the UAI messages have defined corresponding actions from the NW nor from the UE, yet, some UAI messages indicate the initiation of a different UE behaviour. For example, A UE capable of relaxing its RLM measurements of a cell group in RRC_CONNECTED state shall initiate the procedure for providing an indication of its relaxation state for RLM measurements (a UEAssistanceInformation message with rlm-MeasRelaxationState) upon being configured to do so, and upon change of its relaxation state for RLM measurements in RRC_CONNECTED state.

For Rel-19 XR, RAN1 has discussed different possibilities for UAI, which are summarised in their agreements listed in the LS sent to RAN4 [3]. This includes: (i) UE assistance information related to measurement occasions, (ii) UE assistance information related to channel conditions, (iii) UE assistance information related to traffic, and (IV) UE assistance information related to UE mobility. In addition, RAN1 has deprioritised UAI related to channel condition, traffic and UE mobility. Besides, RAN1 mentioned that there is no consensus on whether to support UAI related to measurements occasions. However, it is not clear the reason for deprioritisation on the channel condition, traffic and UE mobility. Hence, RAN4 should also check the feasibility of using all UAI options regardless of RAN1 decision.

Observation 1: RAN1 decision to deprioritise the UAI related to channel condition, traffic and UE mobility are not clearly stated in the LS.

- RAN4 should study all possible UAI options regardless of RAN1 deprioritisation decision.

NW controlled gap cancellation:

As the WID objective state, RAN WG shall specify mechanism to allow transmission/reception during the gaps/restrictions. Provided that the measurement gaps cause interruption for scheduled data UL/DL, hence, to allow the Tx/Rx during MG occasions some MG occasions have to be dropped and replaced by scheduled Tx/Rx. Such gap occasions cancellation might be limited to UE processing similar to that specified for pre-configured measurement gaps (Pre-MG). Therefore, RAN4 should study the feasibility of dropping MG and what is the best mechanism to enable that.

Observation 2: In order to enable Tx/Rx during gaps occasions, a gap cancellation mechanism is needed to inform UE/NW on cancelled gap occasions.

- RAN4 RRM shall study all possible options that allow gap cancellations.

# Summary

In this contribution, we have the following proposals:

Proposal 1: RAN4 should study all possible UAI options regardless of RAN1 deprioritisation decision.

Proposal 2: RAN4 RRM shall study all possible options that allow gap cancellations.

# References

- RP-234080, “New WID: XR (eXtended Reality) for NR Phase 3”, MediaTek (Moderator), Dec 2023.

- TR 38.838, v17.0.0, “Study on XR (Extended Reality) Evaluations for NR (Release 17)”, 2021-12.

- R1-2405736, “LS on UE assistance information”, Nokia, May, 2024.