3GPP TSG-RAN WG4 Meeting #112	R4-2413217

# Maastricht, Netherlands, August 19th – August 23rd, 2024

# Title: 	Consideration of Linear Polarization in Ku Band Coexistence Studies

# Source:	SKY Perfect JSAT

# Type:	Discussion

# Document for:	Discussion

# Agenda Item:	8.9.2 Coexistence study based on ITU regulations

Release	Rel-19

- Introduction

In RP#104, a new WI [1] for Ku band was approved and the phrase 'Reuse Ka-band coexistence study assumptions as much as possible, with modifications where necessary' is stated in the Work Item Description (WID) for the Ku-band standardization. The following is an excerpt from the WID related to the coexistence study.

- Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 excluding US: - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 excluding US - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate TX/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services .

In this document, we will provide our views on whether it is necessary to consider polarization under the condition of determining whether additional practical co-existence scenarios need to be considered while re-using the Ka-band coexistence study assumptions as much as possible.

- Discussion

The results from the NTN-related co-existence study are documented in TR 38.863. Within this document, the antenna and beamforming pattern modelling for polarization shown in table 2-1 is used.

Table 2-1 Beam layout definition for single satellite simulation

|  | Option 1: FRF=1 | Option 2: FRF=3 | Option 3: FRF=2 |
| --- | --- | --- | --- |
| Polarization re-use | Option 1: Disable Option 2: Enable Note: Polarization re-use should apply only if circular polarization for terminal antenna is considered |  |  |

Since GEO satellites in Ku-band use linear polarization in most cases, it is necessary to clarify whether the results of the coexistence study are affected by the choice between linear and circular polarization and to discuss whether a separate coexistence study for the linear polarization model is required.

Observation 1: GEO satellites in Ku-band use linear polarization in most cases and this does not appear to have been considered in the NTN-related coexistence study of TR 38.863.

Proposal 1: RAN4 to discuss whether the results of the coexistence study are affected by the choice between linear and circular polarization and whether it is also necessary to conduct a separate study on shared use considerations for the linear polarization model.

- Conclusions

Observation 1: GEO satellites in Ku-band use linear polarization in most cases and this does not appear to have been considered in the NTN-related coexistence study of TR 38.863

Proposal 1: RAN4 to discuss whether the results of the coexistence study are affected by the choice between linear and circular polarization and whether it is also necessary to conduct a separate study on shared use considerations for the linear polarization model.

- References

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat