3GPP TSG-RAN WG4 Meeting #112	R4-2413223

Maastricht, NL, Aug 2024

Source	:	Qualcomm Incorporated

Title	:	On UE Rx requirements for the LPWUR

Agenda item	:	8.22.2.2

Document for	:	Approval

# 1.	Introduction

The low-power wake-up signal and receiver belong to a class of features that bring only indirect benefit to the network through lower energy consumption in the UE. To maximize the benefit from the feature and increase the probability of deployment, it is beneficial to identify as many gainful scenarios as possible, including connected mode and FR2 bands. It is also important to establish the definition of SNR for OOK signals to facilitate future work.

# 2. 	Discussion

## 2.1	FR2 motivation

### 2.1.1	WID scope

While the notion of specifying LPWUS functionality in FR2 bands is not controversial, it is useful to establish that this course of action is well within the scope of the WID. The WID lists both 38.101-1 and 38.101-2 as ‘impacted TS, and there is no preclusion of either FR in the objectives section.

Observation 1: 38.101-2 is expected to be changed by the Rel-19 LP-WUS WI

In RAN1, there is already agreement that the LPWUS and LPSS signals will be defined with some QCL-D relation, and only the detail on number of RBs remain to be formalized. The prevailing sentiment is to retain the same RB count for FR2, so this open detail is of low concern. We therefore think that FR1 and FR2 requirement sets can be co-developed. We next look at how FR1 and FR2 networks compare for coverage.

### 2.1.2	FR1/FR2 coverage comparison

The appendix contains a comparative coverage analysis across FR1 and FR2, based on values recorded in TR38.830 (coverage enhancement TR). The main observation is that FR1 and FR2 bands are similar from an implementation margin perspective because their respective UL and DL coverages scale similarly enough.

Delving into the detail of the calculations, FR2 bands seem to have a benefit of about 4 dB over FR1 implementations, considering noise limitations. Part of this benefit could get eroded away due to the additional challenge presented by frequency and timing errors at FR2 frequencies. On the other hand, this loss of benefit could be recovered and the benefit increased further if the UE can find a way to use refined beams for LPWUS detection. Refined-beam usage is feasible in connected mode, due to the need to measure SSBs anyway.

Observation 2: The challenge of designing a LP-WUR with sufficient coverage is equivalent for FR1 and FR2.

Also evident from the analysis is that LPWUR sensitivity requirements derived solely from the coverage criterion would be very relaxed, i.e. it is sufficient to derive sensitivity requirements from NF for feasible and reasonable receiver implementations. To elaborate, the coverage criterion translated into physical parameters for 2.6 GHz is reproduced here from the annex:

24.3 dB -SNRsensitivityDL WUS+NFDL+impLossDL ≥0

In our estimation the SNR is expected to be inside [-10 to 0] based on channel conditions and other side conditions. Implementation losses are not precisely defined but left here as a temporary place holder. Its value range is tentatively 0 to 3dB and is intended to capture implementation choices that differ slightly from the study options. Thes choices leave significant NF margin for reasonable implementations, while maintaining MSG3 coverage.

Observation 3: For the Rel-19 feature, UE REFSENS requirements for LPWUS are driven mainly by LPWUR NF rather than the MSG3 coverage criterion.

Given the ‘who benefits, who pays’ argument for this feature however, it behoves the UE community to not specify sensitivity based on the bare minimum receiver with a very relaxed noise figure, just because the coverage criterion may allow. Better sensitivity means low MDR in Rel-19, and a better platform on which to build future enhancements (like offloading progressively more RRM functionality).

Observation 4: From a system design and evolutionary perspective, it is advantageous to retain an aggressive net NF target (i.e sum of NF and implementation loss targets) for REFSENS specification.

### 2.1.3	FR2 scenarios

Our studies [1] have shown that FR2 connected mode operation stands more to gain significant benefit from this feature relative to FR1 promise. Moreover, there is a pathway for further benefit if LPWUS functionality is enhanced to include more RRM functionality. Now, RAN4 UE RF requirements are expected to be invariant of idle or connected mode. Given the promise of better savings in connected mode, the side conditions may be better constructed to reflect both modes.

Proposal 1: RAN4 to reflect both idle and connected mode conditions in the side conditions for the LPWUR requirements, at least for FR2.

We have attempted to show in previous sections that RAN4 requirements for FR2 can be codeveloped with those for FR1. The choice of example FR2 band can be based on current and imminent FR2 deployments. A ‘low-band’ (24-30 GHz) seems like a good representative for this purpose, and for the sake of definition, it is proposed that n258 be used as the example FR2 band.

Proposal 2: RAN4 to consider using n258 as the example FR2 band and co-develop the requirements alongside FR1.

## 2.2	Link sim results

### 2.2.1	The SNR problem

The WF [2] outlines many important conditions that enable this activity; however it is not clear that SNR for OOK signals will be calculated in a consistent manner across RAN4. For example: is SNR computed during the ‘ON’ part of the underlying carrier? Or is SNR calculated as ratio of signal power during the ‘ON’ phase to noise power over the whole ON-OFF cycle? Without this basic alignment, it may not be possible to compare results

Proposal 3: RAN4 to define SNR for OOK signals prior to comparison of simulation data

Pending this important detail, we have a limited set of results to begin with preliminary calibration and adjustment.

In our simulations, the following definition of SNR was adopted: It is the ratio of the total signal energy in one coded bit of LPWUS to the total noise energy in the BW occupied by the LPWUS over the same period. With Manchester coding, the noise power is accrued over both, the ‘ON’ and ‘OFF’ phases, but the signal power is only accrued over the ‘ON’ phase.

Our implementation assumes an OFDM receiver. The results are for an AWGN channel, and for no repetition (‘channel structure’ of 8 or 16 does not matter). We also viewed the simulation exercise through an implementation perspective, and one of the outcomes is to deviate from the WF and assume non-zero timing error. Our FAR characteristic has a floor determined by implementation choices like where to place the decision threshold and number of timing hypotheses to evaluate, among other factors. At this time the focus is on the bare LPWUS, without ACS or other jammers.

### 2.2.2	FR1 SNR

The MDR and FAR are presented below for FR1. For a target MDR of 1%, the required SNR for AWGN channels is in the -8 dB range.

### 2.2.3	FR2 SNR

The MDR and FAR are presented below for FR2. For a target MDR of 1%, the required SNR for AWGN channels is projected to be in the -7 to -8 dB range (virtually identical to FR1). The RMS time errors are similar across FR1 and FR2, but the absolute frequency error is understandably higher for FR2. This frequency error is better viewed in context of the LPWUS BW, which is 4 times higher in FR2 – this is the reason the seemingly higher frequency error does not manifest as a worse SNR vs MDR relationship, compared to FR1.

# 3.	Conclusion

Observation 1: 38.101-2 is expected to be changed by the Rel-19 LP-WUS WI

Observation 2: The challenge of designing a LP-WUR with sufficient coverage is equivalent for FR1 and FR2.

Observation 3: For the Rel-19 feature, UE REFSENS requirements for LPWUS are driven mainly by LPWUR NF rather than the MSG3 coverage criterion.

Observation 4: From a system design and evolutionary perspective, it is advantageous to retain an aggressive net NF target (i.e sum of NF and implementation loss targets) for REFSENS specification.

Proposal 1: RAN4 to reflect both idle and connected mode conditions in the side conditions for the LPWUR requirements, at least for FR2.

Proposal 2: RAN4 to consider using n258 as the example FR2 band and co-develop the requirements alongside FR1.

Proposal 3: RAN4 to define SNR for OOK signals prior to comparison of simulation data

# 4.	References

[1] 	R4-2408032, ‘On LPWUR system considerations’, Qualcomm, RAN4#111, May 2024

[2] 	R4-2410569, ‘WF for [111][136] NR_LPWUS_UERF’, vivo, RAN4#111, May 2024

# 5.	Appendix: Coverage analysis (comparison b/w FR1 and FR2)

There have been some concerns regarding LP-WUS coverage in FR2, if supported. However, the requirement is to meet the msg-3 PUSCH coverage in the same carrier/FR. For FR2, msg-3 PUSCH coverage is smaller compared to FR1 in general.

Figure 15: LP-WUS and Msg-3 PUSCH coverages in different FRs

Below, we provide an analysis of LP-WUS coverage with respect to FR2. Firstly, let us consider analysis for FR1 (2.6GHz). As per TR38.830, following parameters are used. Occupied bandwidth is assumed to be 12 PRBs with SCS 30kHz for LP-WUS.

Table A-1.	Parameters to derive LP-WUS MIL for FR1 (2.6GHz)

| Parameter | Value |
| --- | --- |
| BS number of antenna elements | 192 |
| UE number of antenna elements | 1 |
| Power spectrum density | 33dBm per MHz |
| Occupied bandwidth | 4.32MHz |
| System bandwidth | 95MHz |
| Antenna element gain | 8dBi for BS Tx 5dBi for UE Rx |
| Thermal noise density | -174dBm/Hz |

From the parameters, we can derive EIRP, receiver gain, received signal level, as follows

EIRP=psdPerMhz+10⋅log10occBw+dBiTx+10⋅log10numTxEl=33+10⋅log104.32+8+10⋅log10192=70.2dBm

rxGain=sdBiRx+10⋅log10numRxEl=0+10⋅log101=0dB

sigLevRx=-174+NFDL+10⋅log10occBw+SNRsensitivityDL WUS=-174+NFDL+10⋅log104.32⋅106+SNRsensitivityDL WUS

where NFDL and SNRsensitivityDL WUS are noise figure and sensitivity of LP-WUS. From them, LP-WUS MIL can be derived as

MILLPWUS=EIRP+rxGain-sigLevRx-impLossDL=177.8-NFDL-SNRsensitivityDL WUS-impLossDL

where impLossDL is an implementation loss. The condition that LP-WUS coverage matches msg-3 PUSCH coverage can be expressed as

MILLPWUS≥MILMsg3

RAN1 agreed to use MILMsg3=153.51dB from TR38.830 Table 5.1.1.2-2 at the RAN1#116bis meeting. Putting these together, we can rewrite the condition as follows:

MILLPWUS-MILMsg3=24.3 dB-SNRsensitivityDL WUS+NFDL+impLossDL ≥0

SNRsensitivityDL WUS is the required SNR to achieve the target performance of LP-WUS and is dependent on LP-WUS design. NFDL and impLossDL are dependent on the LP-WUR implementation.

Next, we can do the same for FR2. As per TR38.830, following parameters are used. Occupied bandwidth is assumed to be 12 PRBs with SCS 120kHz for LP-WUS.

Table A-2.	Parameters to derive LP-WUS MIL for FR2

| Parameter | Value |
| --- | --- |
| BS number of antenna elements | 128 (per polarization) |
| UE number of antenna elements | 4 (per polarization) |
| Power spectrum density | 40dBm per system BW |
| Occupied bandwidth | 17.28MHz |
| System bandwidth | 95MHz |
| Antenna element gain | 8dBi for BS Tx 5dBi for UE Rx |
| Thermal noise density | -174dBm/Hz |

From the parameters, we can derive EIRP, Rx gain, Rx signal level, as follows

EIRP=psdSysBw+10⋅log10occBwsysBw+dBiTx+10⋅log10numTxEl=40+10⋅log1017.2895+8+10⋅log10128=61.7dBm

rxGain=sdBiRx+10⋅log10numRxEl=5+10⋅log104=11dB

sigLevRx=-174+NFDL+10⋅log10occBw+SNRsensitivityDL WUS=-174+NFDL+10⋅log1017.28⋅106+SNRsensitivityDL WUS

From them, LP-WUS MIL can be derived as

MILLPWUS=EIRP+rxGain-sigLevRx-impLossDL=174.3-NFDL-SNRsensitivityDL WUS-impLossDL

The condition that LP-WUS coverage matches msg-3 PUSCH coverage can be expressed as

MILLPWUS≥MILMsg3

Same as for FR1, MILMsg3 can refer to the value captured in TR38.830 Table 5.2.1.1-2, i.e., MILMsg3=139.72dB. From the above, we can rewrite the condition as follows:

MILLPWUS-MILMsg3=34.6 dB -SNRsensitivityDL WUS+NFDL+impLossDL ≥0

Note here that the same beam shapes were assumed at the UE for receiving LP-WUS as well transmitting msg 3. A worse situation for WUS reception is if the UE uses less refined beams for it. In this case there would be a 6 dB reduction in FoM, calculated originally as 34.6 dB above

Table A-3

| FR | The condition that LP-WUS coverage matches or exceeds Msg-3 coverage |
| --- | --- |
| FR1 | 24.3 dB -SNRsensitivityDL WUS+NFDL+impLossDL ≥0 |
| FR2 | 28.6 dB -SNRsensitivityDL WUS+NFDL+impLossDL ≥0 (*) (*) assuming rough beam for LPWUS detection |

Observation: The coverage criteria (i.e. implementation challenges) looks very similar across FR1 and FR2

It could be argued that the FR2 LPWUR has about a 4 dB easier time meeting the coverage criterion compared to FR1 bands (although the exact value of the benefit will vary depending on the bands being compared).