3GPP TSG-RAN WG4 Meeting #112	R4-2413224

Maastricht, NL, Aug 2024

Source	:	Qualcomm Incorporated

Title	:	On asymmetric connection scenarios for NR MIMO Phase 5

Agenda item	:	8.18.2

Document for	:	Approval

# 1.	Introduction

The Rel-19 WID on NR MIMO Phase [1] includes the following objective:

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

In this contribution, we discuss how the objective can be translated to a RAN4 requirement concept, while respecting the associated constraints in the WID.

# 2. 	Discussion

## 2.1	Possible asymmetric scenarios for FR2

### 2.1.1	Overview

The WID objectives apply to both FR1 and FR2, but since FR1 networks do not have the complication of UE beam management, the FR1 objective is limited to 5.a in the WID excerpt above (power control and pathloss calculations).

The RF challenge on the other hand does apply to FR2. In the overview table below, we identify various asymmetric FR2 scenarios that warrant further investigation. The WID identifies that the UL TRP may or may not be associated with DL. When different from a DL TRP, they are referred to in this contribution as ‘UL-specific’ TRP. The scenarios are determined based on the ability of UL-specific TRPs to generate transmission (for example SSB), and whether the UE uses STxMP or not. STxMP refers to simultaneous transmission to two different TRPs (multi-TCI-state).

| (FR2 asymmetric scenarios for Rel-19) |  | Tx capability of UL-specific TRP |  |
| --- | --- | --- | --- |
|  |  | Can transmit SSB | No transmitter |
| UE UL TCI-state | STxMP Simultaneous transmission with multiple TCI states | ‘Scenario 1’ | ‘Scenario 3’ |
|  | Single UL TRP can be same or different from DL TRP | ‘Scenario 2’ | ‘Scenario 4’ |

TRP 1

TRP 2

SSB + PDSCH

SSB only

2.1.2	Scenario 1

In scenario 1, the UE transmits UL to multiple TRPs simultaneously (STxMP), and the network leverages spatial diversity to improve UL quality (throughput or reliability). Both TRPs associated with UL would need to furnish some form of beam management RS (i.e. the RS identified for the QCL-D relation), which is natural for TRP1 in the figure.

Based on previous mTRP DL studies in RAN4, the probability of supporting 2 TRP simultaneous connection is low for commercial UEs. Moreover, STxMP is a challenging feature to specify meaningfully, and remains open as of this writing.

gNB

UL-specific TRP (*)

DL

SSB + PDSCH

UL

(*) – limited DL capability, for example limited SSB only

SSB only DL

2.1.3	Scenario 2

In scenario 2, the UE transmits the UL to a single TRP that maybe the same or different from the DL TRP. ‘Same’ TRP for UL and DL is plain vanilla legacy operation and not interesting in Rel-19 context. So, for Rel-19, the focus is on a single UL directed at a TRP that is different from the DL TRP. In this scenario too, all TRPs transmit location information for beam management (for example: SSBs).

There are multiple motivations for this scenario:

- Legacy configured Tx power requirement applies, due to single TCI state transmission

- Can be used for MPE related direction re-evaluation by UE and network

- Compatibility with HetNet deployments

- Further evolution in beam management framework (i.e. lifting of constraints in beam management) can be absorbed in the future.

### 2.1.4	Scenarios 3 and 4

These scenarios are based on accommodating strictly UL-only TRPs that do not broadcast a QCL-D RS. In Rel-19 context, this would require a blind search by the UE using SRS beam sweeping, and strong network coordination. How the UE would establish time and frequency synchronization with the silent UL TRPs is also not fully resolved in RAN1. Furthermore, dynamic scenarios and UL congestion (due to SRS beam sweeping) are challenges to overcome. For Rel-19, these scenarios have limited promise and are best deprioritized in RAN4.

## 2.2	Summary

Scenario 1 seems to address a throughput or reliability problem, while scenario 2 is simpler and focuses on improving coverage. Given the state of FR2 deployments, it is the latter that is a more pressing concern. Other scenarios involving UL-only TRPs with no DL (listen only) are not ready for building out into a feature due to lack of appropriate beam refinement framework for SRS based refinement.

Proposal 1: RAN4 to adopt scenario 2 above as the basis for an FR2 UE RF requirement for the Rel-19 asymmetric connection objective.

An example requirement could be UE spherical coverage with a different TCI state for UL compared to DL. Further RAN4 discussion will help determine if necessary to specify other requirements unique to this configuration.

# 4.	References

- RP-234007, New WID: NR MIMO Phase 5, Samsung.