3GPP TSG-RAN WG4 Meeting #112	R4-2413225

Maastricht, NL, Aug 2024

Source	:	Qualcomm Incorporated

Title	:	On wider applicability of higherPowerLimit-r17

Agenda item	:	8.1.1.1.3

Document for	:	Approval

# 1.	Introduction

In RAN#103 a new work item was created to study HPUE for carrier aggregation for terrestrial networks, with a sub-objective to investigate feasibility of general application of the capability higherPowerLimit-r17. A previous WF [5] established that the release 17 capabilities, higherPowerLimit-R17 and higherPowerLimtMRDC-R17, would be used as a starting point for further work.

In this contribution we share our view on scenarios to be considered for this enhancement.

# 2. 	Discussion

## 2.1	Applicability of higherPowerLimit-r17: rule, or exception?

The requirements for UEs that indicate higherPowerLimit-r17 (an optional capability) are unambiguously known [4]

Observation 1: If a CA band combination is already enabled in the standard, all the UE requirements are unambiguously known, and it is up to UE capability to support higherPowerLimit-r17 in that combination.

Observation 1 suggests that higherPowerLimit-r17 is ready for automatic wider application to other band combinations. More specifically, there is no good justification to selectively meter out what band combinations and power class aggregations get to use this optional capability, and which do not.

The original limitation is recorded in the UE maximum output power section of 38.101-1 and -3 respectively, -1 shown below as example:

An enhancement consistent with the WID and obs. 1 can be implemented by removing the power class and TDD/FDD limitation in the note that enables use of higherPowerLimit-r17:

NOTE 7: The UE that is configured for uplink CA with a single carrier for each individual band may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2A.4.1.3. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise.

Proposal 1: For Rel-19, enable use of higherPowerLimit-r17 as the default case rather than the exception.

## 2.2	Which power class aggregations?

The WF [5] documents several different proposals for applicability of the higher power limit capability. One significant source of divergence is associated with extension to combined inter- and intra-CA combinations. While relevant, it helps to take a staged approach due to the complexity of intra-band configured power.

Proposal 2: RAN4 discusses applicability of higherPowerLimit-r17 to basic inter-band ULCA configurations with single CC per band before addressing mixed inter- + intra- combinations.

Consider the first stage outlined above, a basic inter-band configuration with single CC per band.  For an already optional per-BC capability, there is no need for restriction in Rel-19 for the basic capability involving one CC per band. This observation is useful to get out of the way as an agreement if there is consensus:

Proposal 3: For Rel-19, the basic inter-band configurations (with no intra-band component) is enabled for all power class aggregations that are introduced into the standard. Exceptions can be accommodated on a case-by-case basis.

# 3.	Conclusion

Observation 1: If a CA band combination is already enabled in the standard, all the UE requirements are unambiguously known, and it is up to UE capability to support higherPowerLimit-r17 in that combination.

Proposal 1: For Rel-19, enable use of higherPowerLimit-r17 as the default case rather than the exception.

Proposal 2: RAN4 discusses applicability of higherPowerLimit-r17 to basic inter-band ULCA configurations with single CC per band before addressing mixed inter- + intra- combinations.

Proposal 3: For Rel-19, the basic inter-band configurations (with no intra-band component) is enabled for all power class aggregations that are introduced into the standard. Exceptions can be accommodated on a case-by-case basis.

# 4.	References

[1] 	RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#103, March 18th – March 21st , 2024

[2] 	R4-2404208, ‘RF requirements for HPUE for CA terrestrial networks’, Qualcomm, RAN4#110-Bis, Apr. 2024

[3] 	R4-2406583, ‘WF on HPUE UE for UL CA and EN-DC’, Samsung, RAN4#110-Bis, Apr. 2024

[4] 	R4-2408034, ‘Feasibility of wider application of higherPowerLimit-r17’, Qualcomm, RAN4#111, May 2024

[5] 	R4-2410750, ‘WF on higher power UE’, Samsung, RAN4#111, May 2024