3GPP TSG-RAN WG4 Meeting #112	R4-2413226

Maastricht, NL, Aug 2024

Source	:	Qualcomm Incorporated

Title	:	Activation-based FR2 MPR enhancement

Agenda item	:	8.1.1.2.3

Document for	:	Approval

# 1.	Introduction

In RAN#103 a new work item [1] was created to investigate if lower MPRs can be made applicable to FR2 UEs configured for UL CA without the need for a new effort to simulate back off requirements. In a previous WF [2] it was agreed to have a configuration-based enhancement targeted at UEs that use separate R/T LOs, but activation-based enhancement was left open. In this contribution we share our view on an activation-based MPR enhancement.

# 2. 	Discussion

The relevant agreements from WF [5] are reproduced here for reference:

## 2.4. Applicable MPR for FR2 UL CA with DL intra band CA Way forward: - MPR based on UL BWchannel_CA applies instead that based on cumulative aggregated channel BW (CABW) with UE indication of new capability for MPR improvement - If only 1 UL CC is activated, the MPR requirements of single carrier could be reused - FFS whether new MPR requirement could be defined for CABW < 400 MHz (e.g., 200 MHz).

The agreement above notionally defines a Rel-19 capability where the MPR table applies based on only those configured CCs that are enabled for UL. For reference, the legacy arrangement derived MPRs from the MPR table based on the cumulative aggregated channel BW. The agreed enhancement above forms a good basis for an activated-CC- based enhancement also, because no unique physical impediment exists for a UE to comply with the lower MPRs based on CC activation.

In the standard however, it is necessary to address corner cases also, like the scenarios identified below:

- Non-contiguous CCs are activated inside a contiguous CA configuration.

- The activated CCs that are also enabled for UL do not form a contiguous block.

The first option is to identify all the corner case like the two above and accommodate them with careful wording. Experience has shown there can be significant readability problems as well as potential for missing cases that come to light over time. The other option for compactness and simplicity is to limit the enhancement to cases where the activated CCs enabled for UL form a contiguous block. This latter method is also preferable from a deployment standpoint because it encourages operators to activate CCs in a way that maximizes the enhancement.

Proposal 1: RAN4 to consider as an optional capability:

FR2 UE CA MPR is based on the activated subset of the CCs in the CA configuration that are also enabled for UL, provided the activated CCs with UL comprise a contiguous block. (For reference, legacy CA MPR is based on cumulative aggregated channel BW).

# 2.	References

[1] 	RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#103, March 18th – March 21st , 2024

[2] 	R4-2410749, ‘WF on power domain enhancements’, Huawei, RAN4#111, May 2024