3GPP TSG-RAN WG4 Meeting #112	R4-2413227

Maastricht, NL, Aug 2024

Source	:	Qualcomm Incorporated

Title	:	Discussion on feasibility of FR2 UEs with low EIRP

Agenda item	:	5.35

Document for	:	Approval

# 1.	Introduction

Emerging FR2 markets may well have denser deployment and more use cases than current practice in North America. A previous proposal [1] suggested that these new scenarios will be better served by a new UE power class. In this contribution we discuss next steps.

# 2. 	Discussion

## 2.1	Low EIRP, wide bandwidth and high UL throughput

It is well understood that UL throughput capacity increases with UL PSD. Low EIRP as well as quest for wider channels and intra-band CA work against this goal. The attributes listed above can therefore come together successfully only for a narrow set of conditions, and this set gets progressively smaller as any of the attributes above are developed in the direction intended by the original proposal for the new UE type [1].

Observation 1: High UL throughput is not compatible with low EIRP

## 2.2	Low complexity and CA, ULMIMO support

FR2 UE complexity and accompanying high power consumption can manifest from many sources, be it sophisticated beam management or enhanced baseband capabilities. Power draw especially in DL is a strong function of CA and MIMO capability. Low complexity and power saving is better served by a RedCap UE variant rather than a UE with enhanced baseband capabilities.

Observation 2: A goal to maintain low complexity is not compatible with enhanced baseband capabilities like wide channel BW support, CA support and UL MIMO support.

## 2.3	New UE type (power class)?

The key to investigating if new UE types are warranted is to define the use case and deployment condition for each. Ultimately, a believable link budget must be created, so all companies in the ecosystem understand the assumptions and risks associated with a particular design of UE.  It is possible that the list of use cases may be diverse enough that making just one new power class to cover them all would be counterproductive. An example of this situation is trying to accommodate multiple uses cases where some are DL heavy while others are UL heavy. Yet others may require both UL and DL.

To delve deeper than the subjective arguments above, some identified use case examples were investigated as target for a new UE type.

## 2.4	Some use case examples for the new UE type

### 2.4.1	Video surveillance in ‘Urban Micro’

Here, a 100m ISD is assumed in keeping with the expectation of dense deployments. Also due to clutter and congestion, non-LOS conditions are assumed for the link. Real world effects like SNR ceilings, doppler, etc are ignored for this simplistic calculation. While this case is a UL heavy scenario (estimated to be ~ 10 Mbps), the DL throughput projection is useful to estimate reasonable gNB size (EIRP and panel size).  While the UE is assumed to have a 2x1 module with a realistic gain range of +2 dBi to -13 dBi over the whole sphere, it is assumed to be properly pointed at the best cluster, being a fixed installation. The result is that gain range of the UE in the simulation is clustered at the high end. As such the UE gain range is assumed to be +2 to -2 dBi.  Even the smallest size of gNB studied can support good DL performance that can be spread over multiple such video UEs. (> 50 Mbps for 70% of UEs).

The corresponding spread of UL throughput is shown with the least powerful of the ‘sufficient’ gNBs. With a target UL tput of 10 Mbps, the curves suggest that the proposed 16 dBm EIRP UE cannot sustain the target for > 50% of the time – such an arrangement may be unsuitable for meaningful video surveillance.

Observation 3: Video surveillance in an urban micro setting requires considerably higher EIRP than what is guaranteed by PC7 (FR2 Redcap).

One could also conclude that depending on the ask, a PC3 UE with limited beam sweeping may be able to satisfy these requirements. An alternative and possibly better solution may be a RedCap variant of a PC5 CPE, which offers significant simplification of base band capabilities. Finally, one could argue that fixed installations do not have an energy source limitation (ex: small battery).

### 2.4.2	Indoor XR type UE

This type of UE is assumed to have significant throughput requirements (~10 Mbps DL, ~2 Mbps UL), but the deployment assumption is presence of an indoor gNB. Here too the DL throughput study suggests that the smallest of the studied gNB sizes will suffice (25 dBm EIRP, 10m ISD, nLOS).

Even with this smallest gNB size, the UL throughput for a randomly pointed UE with a 2x1 module and 16 dBm peak EIRP seems to suffice for expectations less than 5 Mbps. Specifically, the UE’s UL will degrade below 2 Mbps only 5% of the time. For this use case, the proposed UE seems to be suitable. Note also that this throughput is accomplished by rank1 and a 100 MHz channel. The question to ask here is: can a new UE type be justified for this use case when a PC7 RedCap UE can fulfil this expectation?

Observation 4: Most reasonable expectations of indoor only UEs can be met by PC7 (FR2 RedCap).

Note that assuming gNBs with higher gain only strengthens the PC7 case, so the studied case is a worst-case scenario for the use of PC7.

## 2.2	Summary

The performance projections shared above depend on deployment assumptions, UE and gNB characteristics and crucially, throughput targets. It is not possible to evaluate a proposal for a new UE type without this type of scrutiny. A subjective motivation based on mixing and matching attributes from existing power classes would suffice as a basis for further study, but that method is not always suitable for a concrete technical proposal, as is the case here. Moreover, as shown above there seems to be no justification for a bespoke UE type, and existing UE types could satisfy some of the asks.

Proposal 1: RAN4 to clearly define use case, throughput targets and deployment conditions to support a new UE power class, so companies may study further.

# 3.	Conclusion

Observation 1: High UL throughput is not compatible with low EIRP

Observation 2: A goal to maintain low complexity is not compatible with enhanced baseband capabilities like wide channel BW support, CA support and UL MIMO support.

Observation 3: Video surveillance in an urban micro setting requires considerably higher EIRP than what is guaranteed by PC7 (FR2 Redcap).

Observation 4: Most reasonable expectations of indoor only UEs can be met by PC7 (FR2 RedCap).

Proposal 1: RAN4 to clearly define use case, throughput targets and deployment conditions to support a new UE power class, so companies may study further.

# 4.	References

[1] 	R4-2408997, ‘Discussion on extension FR2 UE power class 7 to non-RedCap UE’,  Huawei, RAN4#111, May. 2024