3GPP TSG-RAN WG4 Meeting#112							         			R4-2413233

Maastricht, Netherland, 19 – 23 August 2024

Title: 	On DL CA_n66(2A) and CA_n66(3A) with UL CA_n66(2A)

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.3.2	UE RF requirements for NR intra-band CA combinations for x CC DL/y CC UL (NR_CA_R19_Intra with/without UL-MIMO)	[NR_CADC_SUL_R19-Core]

Document for:	Approval

# 1	Background

In RAN#104, a new NR CA basket was introduced. This includes intra-band DL/UL CA combinations as one of the objectives. There is an existing request to support CA_n66(2A) or CA_n66(3A) in DL with CA_n66(2A) in UL. In this contribution, we provide clarification on the request, the work related to UL CA_n66(2A), and its preferred implementation.

# Discussion

## 2.1 CA_n66(xA) request

The requested BCS for CA_n66(xA) is copied in the table below:

| NR CA configuration | Uplink NR CA configurations | channel bandwidths for carrier [MHz] | channel bandwidths for carrier [MHz] | channel bandwidths for carrier [MHz] | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n66(2A) | CA_n66(2A) | 5, 10, 15, 20, 40 | 5, 10, 15, 20, 40 |  | 80 | 2 |
| CA_n66(3A) | CA_n66(2A) | 5, 10, 15, 20, 40 | 5, 10, 15, 20, 40 | 5, 10, 15, 20, 40 | 80 | 0 |

The current n66(xA) cases in the specification are copied in the table below.

| NR CA Configuration | Uplink CA Configurations or single uplink carrier5 | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n66(2A) | n663 | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 60 | 0 |
|  |  | 5, 10, 15, 20, 25, 30, 40 | 5, 10, 15, 20, 25, 30, 40 |  | 80 | 1 |
|  |  | 5, 10, 15, 20, 40 | 5, 10, 15, 20, 40 |  | 80 | 2 |
|  |  | See n66 channel bandwidths in Table 5.3.5-1 for each carrier |  |  | 85 | 4 and 5 |
| CA_n66(3A) | - | 5, 10, 15, 20, 40 | 5, 10, 15, 20, 40 | 5, 10, 15, 20, 40 | 80 | 0 |

Observation: In both the new request, and the existing BCS in the specification, and given that the total aggregated bandwidth is 80MHz, it is not possible to have three-time 40MHz nor two-times 40MHz + one more CBW. The highlighted 40 MHz CBW should not be present in the first two columns.

Proposal: the 40MHz CBW should be removed in the two first “channel Bandwidths for carrier (MHz)” columns in both the request and the specification for the DL CA_n66(3A) case.

Separately, the UL bandwidth of band n66 is only 70MHz. Thus, UL CA_n66(2A) cannot be aggregated up to 80MHz, considering at least one 5MHz gap. Conversely, the maximum aggregated CBW is 65MHz which, within the BCS definition would actually be 60MHz, based on a 20MHz+40MHz case. Therefore, CA_n66(2A) and CA_n66(3A) DL support with CA_n66(2A), requires the support of asymmetric UL/DL in some cases.

Proposal: A note is added for the CA_n66(2A) UL configuration. This note provides an explanation on the need for asymmetric UL/DL support and that UL aggregated bandwidth is limited to 60MHz.

## 2.2 Work related to the support of CA_n66(2A) in UL.

To add UL CA_n66(2A) support, an assessment is necessary regarding:

- Potential MSD related to ULCA with the largest separation (70MHz), while the duplex gap is 330MHz. Thus, one of the DL channels may be affected by IMD11, with recent studies showing that MSD may also occur at least up to IMD13 interference.

- A-MPR for the specified n66 NS: NS_100, NS_03 and NS_03U.

  - It may be essential that operators using Band n66 with CA_n66(2A) UL confirm whether NS_100 must be supported.

  - It may be also neessary to introduce CA_NC_NS_03 and the related A-MPR.

- Given its limitations in terms of CBW and gap size restrictions, and the amount of effort required, we recommend deprioritizing the single PA approach.

Proposal for the CA_n66(2A) UL RAN4 work:

- UL CA_n66(2A) MSD is studied (even with the large duplex gap).

- Operator interested in using n66(2A) UL must clarify if NS_100 support is needed.

- ULCA A-MPR for NS_03 is studied and introduction of a related ULCA additional SEM requirement discussed.

- Only dualPA is supported.

# Conclusion

In this contribution, we analyzed the request for CA_n66(2A) support and its potential requirements and make the following proposals.

Proposal: the 40MHz CBW should be removed in the two first “channel Bandwidths for carrier (MHz)” columns in both the request and the specification for the DL CA_n66(3A) case.

Proposal: A note is added for the CA_n66(2A) UL configuration. This note provides an explanation on the need for asymmetric UL/DL support and that UL aggregated bandwidth is limited to 60MHz.

Proposal for the CA_n66(2A) UL RAN4 work:

- UL CA_n66(2A) MSD is studied (even with the large duplex gap).

- Operator interested in using n66(2A) UL must clarify if NS_100 support is needed.

- ULCA A-MPR for NS_03 is studied and introduction of a related ULCA additional SEM requirement discussed.

- Only dualPA is supported.

# References