3GPP TSG RAN WG4 #112	R4-2413234

Maastricht, Netherlands, Aug 19 – 23, 2024

Agenda item:		8.11.3

Source:	Nokia

Title:	On OTA test enhancements - CLTA

Document for:		Approval

# Introduction

A Way Forward [1] was agreed in RAN4#111 in Fukuoka, Japan. WF enlists the following agreements to be further discussed:

- Recommended for further discussion

  - Further discuss the wideband antennas and check its possibility of translation between reference co-location antenna and wideband antenna;

  - Further discuss co-location scenario and  coupling loss between two AAS BS especially for FR1 high bands;

  - Further discuss whether the side-by-side deployment is typical scenario for co-location deployment, e.g., in SBFD deployment TX and RX panels could not be in the same height

  - Other issues are not precluded.

This contribution further elaborates Nokia’s view on wideband horn antennas and the translation with co-location reference antenna. It also presents our view of the coupling loss between two AAS BS and typical deployment scenarios.

# Wideband horn antennas

We have proposed in our earlier contributions in RAN4#110bis and RAN4#111 that using wideband horn antennas as CLTA’s would be very beneficial concerning cost and test efforts. Single column passive antennas are not readily available for all of the required frequency bands, and they typically meet the CLTA requirements only at relatively narrow frequency ranges. Currently a lot of custom-made antennas are needed in many cases. This makes testing especially in higher and lower frequencies become highly complex and time-consuming. One set of comparison measurements was done to demonstrate the differences of wideband horn antennas vs. CLTA’s. The gain of one CLTA and a wideband horn antenna were measured for 698MHz…2690MHz. After that the spurious emissions were measured to get comparison data for the same frequency band. The spurious emissions measurements were done for a single AAS and it was measured with side-by-side distance of 10cm for CLTA and the horn antenna. The horn antenna vertical alignment was in the center of the DUT as shown in Figure 1.

Figure 1. Measurement setup for the wideband horn antenna measurements.

Example of the measurement results can be seen in Figure 2. Here a comparison of spurious emissions between CLTA and wideband horn antenna is seen.

Figure 2. Example of spurious emissions result between CLTA and horn antenna.

It is seen that there is a correlation between the CLTA measurement result and the horn antenna measurement result. It must be noted that this is just one measurement with 10cm distance measured in the vertical center of the DUT. The wideband horn antenna has wider vertical beamwidth than the CLTA, which will lead to higher power level of the results. More measurements with different setups are needed to give more confidence with the alignment between CLTA and horn antenna but the first result looks promising.

It is seen that there is a correlation between CLTA and wideband horn antenna spurious emissions results.

# Co-location and coupling loss between two AAS BS

It has been discussed in previous RAN4 meetings if the current coupling loss assumption of 30dB between two AAS BS is still valid. In our view this requirement is still valid. If we look at the current deployments occupied in the field, it is seen that site masts can be full of equipment as new sites are very expensive to establish and there is no space for new installations so existing masts will be used also in the future. Even nowadays there have been cases where the coupling loss might actually be less than the current assumption of 30dB but we feel that there is no need to tighten the requirement. The 30dB coupling loss reflects well what is actually witnessed in the field and should not be likely changed.

30dB coupling loss reflects well with actual field deployments.

Keep the existing assumption of 30dB coupling loss.

# Typical deployment scenarios

As stated above in Chapter 3, the existing site masts will be used for new deployments and typical deployment will still be side-by-side as the space that is available is very limited. Operators will upgrade their equipment and replace older equipment with new ones but still place them in similar fashion to the sites.

# Conclusion

This document shares Nokia’s views on OTA test enhancements. Following observations and proposals were introduced:

- It is seen that there is a correlation between CLTA and wideband horn antenna spurious emissions results.

- 30dB coupling loss reflects well with actual field deployments.

- Keep the existing assumption of 30dB coupling loss.

# References

- R4-240995 “Way Forward for [111][311] NR_BS_RF”, RAN4#111, Fukuoka, Japan 20th - 24th May, 2024

- TS38.104 18.4.0 “5G; NR; Base Station (BS) radio transmission and reception”

- TS38.141-2 18.4.0 “5G; NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing”

- TR37.941 17.2.0 “Radio Frequency (RF) conformance testing background for radiated Base Station (BS) requirements”