3GPP TSG RAN WG4 #112	R4-2413235

Maastricht, Netherlands, Aug 19 – 23, 2024

Agenda item:		8.19.2.3

Source:	Nokia

Title:	On existing BS RF Rx requirements for SBFD operation

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#111 [1] regarding evolution of NR duplex operation. The following agreements were made that have impact on the existing BS RF requirements for SBFD operation:

  - Modification of existing requirements - TX

#### Issue 2-1-1: PSD scaling for normal and SBFD slots/symbols

- Agreement:

  - PSD scaling for normal and SBFD slots/symbols:

    - No need to introduce the restriction on PSD scaling for normal and SBFD slots/symbols

    - Vendors can declare different TX power values for normal and SBFD slots/symbols

#### Issue 2-2-1: TX intermodulation requirement

- Agreement:

  - The transmitter intermodulation requirement is applicable in SBFD slots/symbols:

    - Follow the existing requirement

    - No receiver requirement is specified.

#### Issue 2-3-1: The necessity of Co-location ACLR requirement

- Agreement:

  - RAN4 further study the ACLR requirement by applying the existing ACLR requirement for SBFD-capable BS in SBFD symbols/slots

    - FFS the applicable deployment scenario where different ACLR requirement could be useful, whether it justify the different ACLR requirement.

#### Issue 2-3-2: OBUE

- Agreement:

  - Further discuss how to define OBUE requirement for SBFD capable BS in order to ensure feasible co-existence performance for SBFD BSs operating in adjacent frequency.

The Way Forward also describes the necessity for additional co-existence study for in-band blocking:

- Way forward:

  - The group will define the case/scenarios for additional co-existence, with the assumptions to be discussed in details.

  - Companies are encouraged to provide the evaluation results for in-band blocking in the next meeting:

    - The scenarios to be studied are Scenario 1, 3, 5, 6, 9, defined in TR 38.858

    - The case to be studied is Case 3 (TDD DL to SBFD UL subband).

    - FFS the power level which should be collected from the simulation results to derive in-band blocking levels:

      - Option-1: absolute power level of interference from adjacent channel only, i.e., SBFD network internal interference should not be considered in the simulation, from the perspective of definition of in-band blocking requirement.

      - Other options are not precluded.

    - FFS the reference point for the power level which should be before or after RX beamforming.

      - Option-1: after RX subarray beamforming gain and before array gain, i.e, at TAB.

    - FFS different grid-shift values should be considered.

      - Option-1: 10%.

      - Option-2: Other percentage values are not precluded.

    - FFS how to use the power level probability distributions to derive the in-band blocking levels

      - Option-1: Find the X% tile of the UL SBFD wideband received power CDF to define the blocking requirement in each of the down-selected scenarios.

        - FFS the value of X

        - FFS the impact of CLI handling scheme introduced in RAN1.

This contribution further elaborates Nokia’s views on the existing BS RF requirements for SBFD operation at gNB.

# Impact on BS RX requirements

## In-band blocking simulations

In the previous RAN4#111 meeting, companies agreed to provide system-level simulation results to determine the in-band blocking SBFD BS requirements. Following the methodology and assumptions from the study item [2], Case 3 is assumed, i.e., the victim network uses SBFD while aggressor network in the adjacent channel uses TDD DL. The goal is to measure the in-band blocking power at the SBFD base stations. Note that, as also described in [2], the in-band blocking power is defined as a sum of wanted signal and all sources of interference signals as bellow:

Pin=Pw+Iintra-SB, inter-UE+Iinter-SB,inter-BS+IACI+Iself+Ico-site,inter-sector,

where Pw is the wanted signal from the scheduled UE, Iintra-SB, inter-UE is the inter-cell interference from other UEs scheduled in UL within the victim network, Iinter-SB,inter-BS is the BS-to-BS interference within the victim network, IACI is the adjacent channel interference from TDD BS transmitting on the DL channel within the aggressor network, Iself is the victim BS self-interference, and Ico-site,inter-sector is the interference coming from the neighbour sectors in a 3-sector macro BS configuration. It is important to highlight that the effect of ACLR and ACS are not accounted in the interference power calculation.

The interference components for the in-band blocking calculation do not account for the ACLR and ACS effects.

There is a first FFS in the WF about whether to include all the signals components described in the equation above, or whether to only include the adjacent channel interference component, i.e., the interference generated from TDD DL BS towards SBFD BS in UL. This decision depends on the in-band blocking requirement definition. As stated in TS 38.104, the current in-band blocking requirement assumes that the interfering signal is outside the base station RF bandwidth. Thus, following the same approach now for SBFD, it would mean that the SBFD-specific interferences such as BS self-interference or inter-sector interference are left out of the in-band blocking requirement. However, those interferences also impact the receiver performance since they are also received by the BS LNA. The outcome of the discussion would depend on whether the previous definition of the RAN4 in-band blocking requirement is updated to accommodate the SBFD co-channel interferences or not. In any case, RAN4 should include a clear statement on how the in-band blocking requirements for SBFD BS are derived in order to avoid possible misunderstandings.

Current BS RF in-band blocking requirements only consider adjacent channel interference.

Whether or not to include the SBFD co-channel interference as part of the in-band blocking derivation depend on whether current definition of the RAN4 in-band blocking requirement is updated or not.

RAN4 should include a clear statement on how the in-band blocking requirements for SBFD BS are derived in order to avoid possible misunderstandings.

Consider only the adjacent channel interference power to determine the in-band blocking power.

Furthermore, we should also decide which grid-shift should be assumed. In our view, the grid shift of 10% would provide the worst-case in terms of adjacent channel interference and therefore we prefer to limit the scope of the simulations to this grid shift value. In the spirit of reducing the scope the results, we also propose to only simulate 1 SBFD antenna configuration, when applicable. We expect simulation results to be same regardless of whether Antenna Config.1 or Antenna Config.2 is assumed. The reason is that the Rx beamforming gain is not considered in the coupling-loss calculation (as indicated in the WF), and therefore, the effect of having a different number of elements on the SBFD BS panel should not affect. For the TDD BS transmit power, we suggest to only simulate those indicated as baseline (or 1st priority) assumptions in the Annex E of TR 38.858.

To reduce the scope of the simulations, 10% grid-shift is the only considered grid-shift value.

To reduce the scope of the simulations, consider only SBFD antenna configurations and TDD BS transmit power defined as baseline or 1st priority according to TR 38.858 Annex E.

Companies should also agree on which probability level of the in-band blocking power CDF should be considered to derive the RF requirements. TR 25.942 describes the methodology used to derive the current RAN4 BS RF in-band blocking requirements. There, the in-band blocking power was derived using the 99.9th percentile. In a similar manner, we propose to use the 99th percentile of the in-band blocking power CDF. To ease the collection of results, we propose as well to re-use the excel spreadsheet template from the calibration of coexistence simulations during the SI.

Use the 99th percentile of the in-band blocking power CDF (X = 99) to derive the BS RF requirements.

Re-use the excel spreadsheet from simulator calibration during the study item to collect the in-band blocking power results from different companies.

Lastly, there is an FFS on whether to include the effect of the CLI handling on the in-band blocking power. Our view is that the CLI handling should not be included due to various reasons. First, the several CLI handling schemes discussed in RAN1 are not a mandatory feature and therefore should not be assumed as baseline for all SBFD BS. Moreover, the benefits of applying CLI handling schemes is highly dependent on the type of scenario, load conditions, antenna configuration, etc., therefore agreeing on a specific value to account for the impact of the CLI handling schemes is quite challenging. Additionally, the applicability of the RAN1 discussed CLI handling schemes to mitigate the adjacent channel interference is also questionable.

RAN4 should not consider any effect of the CLI handling mechanisms for the calculation of the in-band blocking power levels.

As shown in the attached excel spreadsheet, we simulated Case 3 for the scenarios 1, 3, 5, 6, and 9. A summary of the results can be found in the following table, where the 99th percentile of the in-band blocking power is collected. For these results, only the adjacent channel interference has been considered for the in-band blocking power calculation.

Table 1. In-band blocking power [dBm] per scenario and antenna configuration

| Case 3 scenarios | In-band blocking power at 99th percentile | Current in-band blocking in TS 38.104 |
| --- | --- | --- |
| Scenario 1 (Urban macro FR1) | -2.1 dBm | Wide Area FR1 BS: -43 dBm |
| Scenario 3 (Urban micro FR1) | -7.3 dBm | Medium Range FR1 BS: -38 dBm |
| Scenario 5 (Indoor FR1) | -39.2 dBm | Local Area FR1 BS: -35 dBm |
| Scenario 6 (Urban macro FR2) | -38.1 dBm | Wide Area FR2-1 BS: [-86, -63] dBm |
| Scenario 9 (Indoor FR2) | -57.8 dBm | Local Area FR2-1 BS: [-76, -53] dBm |

Based on the results above the following is observed:

For FR1 wide area SBFD BS, the in-band blocking power at the 99th percentile is 1.8 dBm, which is approximately 40 dB higher than the current in-band blocking requirement.

For FR1 medium range SBFD BS, the in-band blocking power at the 99th percentile is -7.3 dBm, which is approximately 30 dB higher than the current in-band blocking requirement.

For FR1 local area SBFD BS, the in-band blocking power at the 99th percentile is -39.2 dBm, which is on the same order of magnitude than the existing local area FR1 BS.

For FR2-1 wide area SBFD BS, the in-band blocking power at the 99th percentile is -38.1 dBm, which is approximately 25 dB higher than the current in-band blocking requirement.

For FR2-1 local area SBFD BS, the in-band blocking power at the 99th percentile is -57.8 dBm, which is on the same order of magnitude than the current in-band blocking requirement.

## Reference sensitivity level and OTA sensitivity

In the study phase of the Evolution of NR Duplex Operation, the following was agreed:

Regarding Reference sensitivity requirement for SBFD-capable BS, due to the self interference caused internally to receiver side, RAN4 reached the following consensus:

-	For BS type 1-H if supported: The existing requirement for conducted reference sensitivity level shall also be applied to BS in SBFD symbols, i.e, no sensitivity degradation is allowed.

-	Otherwise, OTA sensitivity requirement could be derived based on the following equation:

PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+[ desens target]-G

-	The candidate value [0.5~1.0]dB degradation and final value will be specified in the WI phase.

-	The following aspects need more discussion during a WI phase

-	The declaration of maximum TRP for the requirement of OTA sensitivity within SBFD time slot

-	If OTA sensitivity should be defined considering all of the scenarios including self-interference, inter-site interference and inter-sector interference.

Even though SBFD is non-overlapping, it is still full duplex as the transmitter and receiver are operational at the same time in adjacent subbands. This makes it a completely different case compared to current TDD deployments and testing environment. Self-interference plays a major role in OTA sensitivity, but it is crucially important to also take into account the inter-sector and inter-site interference as they have a significant interfering impact, and they are much more difficult to cancel than self-interference. Including inter-sector interference and inter-site interference to OTA sensitivity testing would ensure the sensitivity degradation target is not exceeded. However, these types of interferences are not feasible to be tested in any reasonable test environment that would reflect real deployments that require isolating materials and other methods for needed isolation and for this reason, this is not seen as sensible method. As the benefits of functional SBFD system are in the UL improvements, we cannot sacrifice the sensitivity any more than the 1dB that was assumed in the study phase. Since the OTA sensitivity requirement does not capture the effects from inter-sector and inter-site interference but self-interference only, the maximum value that can be accepted for [desens target] in the OTA sensitivity formula should be 0.5dB with leaving 0.5dB for desensitization due to inter-sector and inter-site interference.

Use maximum of 0.5dB for desensitization target value for the OTA sensitivity requirement due to self-interference.

# Conclusion

In this contribution, our views on the SBFD BS RF requirements are presented. There were observations and proposals as listed below.

- The interference components for the in-band blocking calculation do not account for the ACLR and ACS effects.

- Current BS RF in-band blocking requirements only consider adjacent channel interference.

- Whether or not to include the SBFD co-channel interference as part of the in-band blocking derivation depend on whether current definition of the RAN4 in-band blocking requirement is updated or not.

- For FR1 wide area SBFD BS, the in-band blocking power at the 99th percentile is 1.8 dBm, which is approximately 40 dB higher than the current in-band blocking requirement.

- For FR1 medium range SBFD BS, the in-band blocking power at the 99th percentile is -7.3 dBm, which is approximately 30 dB higher than the current in-band blocking requirement.

- For FR1 local area SBFD BS, the in-band blocking power at the 99th percentile is -39.2 dBm, which is on the same order of magnitude than the existing local area FR1 BS.

- For FR2-1 wide area SBFD BS, the in-band blocking power at the 99th percentile is -38.1 dBm, which is approximately 25 dB higher than the current in-band blocking requirement.

- For FR2-1 local area SBFD BS, the in-band blocking power at the 99th percentile is -57.8 dBm, which is on the same order of magnitude than the current in-band blocking requirement.

- RAN4 should include a clear statement on how the in-band blocking requirements for SBFD BS are derived in order to avoid possible misunderstandings.

Consider only the adjacent channel interference power to determine the in-band blocking power.

To reduce the scope of the simulations, 10% grid-shift is the only considered grid-shift value.

To reduce the scope of the simulations, consider only SBFD antenna configurations and TDD BS transmit power defined as baseline or 1st priority according to TR 38.858 Annex E.

Use the 99th percentile of the in-band blocking power CDF (X = 99) to derive the BS RF requirements.

Re-use the excel spreadsheet from simulator calibration during the study item to collect the in-band blocking power results from different companies.

RAN4 should not consider any effect of the CLI handling mechanisms for the calculation of the in-band blocking power levels.

Use maximum of 0.5dB for desensitization target value for the OTA sensitivity requirement due to self-interference.

# References

- R4-2409958 “Way Forward for [111][313] NR_duplex_evo”, RAN4#111, Fukuoka, Japan, 20th-24th May, 2024

- TR 38.858 V18.1.0 “Study on Evolution of NR Duplex Operation”

- TR 25.942 V18.0.0 “Radio Frequency (RF) system scenarios”