3GPP TSG RAN WG4 #112	R4-2413238

Maastricht, Netherlands, Aug 19 – 23, 2024

Agenda item:		8.19.1

Source:	Nokia

Title:	On SBFD general aspects and system parameters

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#111 [1] regarding evolution of NR duplex operation. The following agreements were made that are general or have impact on the system parameters:

#### Issue 1-1-6: CLI handling impact on RF requirement

- Agreement:

  - The CLI handling impact on RF requirement:

    - Option 1: RAN4 define RF requirements for the frequency outside of wanted carrier based on no CLI handling as baseline.

    - Option 2: FFS CLI handling impact on RF requirements

#### Issue 1-2-2: How to handle guard band and subband configurations in specification

- Agreement:

  - Add clarification that only DUD and DU patterns are specified for the sub-band configuration.

  - FFS which channel bandwidth(s) or all channel bandwidths shall be defined for RF requirements

  - For a certain channel bandwidth which RAN4 agree to introduce RF requirements:

    - FFS RAN4 only define the UL/DL subbands configuration(s) for RF requirements

      - FFS which UL/DL subbands configuration(s) will be defined in RAN4

      - FFS Guard band size is declaration based and can be different for different BS classes

      - FFS the limitation on the maximum guard band

      - FFS possible range for UL/DL subband sizes

    - From RAN4 perspective, FFS restriction or no restriction to RAN1 definition for UL/DL subband sizes within the transmission configuration for this channel bandwidth, except:

      - 1RB granularity (already introduced in RAN1)

This contribution further elaborates Nokia’s views on the SBFD general aspects and system parameters at gNB.

# Discussion

## CLI handling impact on RF requirement

Two options are listed in the WF:

  - The CLI handling impact on RF requirement:

    - Option 1: RAN4 define RF requirements for the frequency outside of wanted carrier based on no CLI handling as baseline.

    - Option 2: FFS CLI handling impact on RF requirements

In our view, the CLI handling should not be assumed as baseline for the derivation of the RF requirements. We find the following arguments to support Option 1 in the WF. First, the CLI handling schemes discussed in RAN1 are mainly applicable for co-channel CLI and their applicability to adjacent channel CLI is not always possible. Moreover, the discussed CLI handling schemes are only optional features that are not expected to be supported by all the base stations in the field. Lastly, the impact of each CLI handling is quite challenging to measure due to the fact that it is quite dependent on the specific deployment, load conditions, frequency of the information exchange across nodes, etc.

RAN4 to not consider any CLI handling scheme impact on the RF requirements discussions.

## Subband configurations and guardbands

The following was agreed in RAN4#111:

#### Issue 1-2-2: How to handle guard band and subband configurations in specification - Agreement: - Add clarification that only DUD and DU patterns are specified for the sub-band configuration. - FFS which channel bandwidth(s) or all channel bandwidths shall be defined for RF requirements - For a certain channel bandwidth which RAN4 agree to introduce RF requirements: - FFS RAN4 only define the UL/DL subbands configuration(s) for RF requirements - FFS which UL/DL subbands configuration(s) will be defined in RAN4 - FFS Guard band size is declaration based and can be different for different BS classes - FFS the limitation on the maximum guard band - FFS possible range for UL/DL subband sizes - From RAN4 perspective, FFS restriction or no restriction to RAN1 definition for UL/DL subband sizes within the transmission configuration for this channel bandwidth, except: - 1RB granularity (already introduced in RAN1)

As a first step, we think RAN4 should narrow down a list of possible carrier bandwidths where requirements for SBFD operation should be specified. The Release 18 study focused primarily on 100 MHz channel bandwidths for FR1. In our view, SBFD operation may be supported for smaller carrier bandwidths down to 50 MHz as well. Below this value, the size of the SBFD guardbands may become excessive with respect to the usable data resource blocks, and it may also become challenging to place reference signals and channels such as SSB and CORESET 0.

Supporting SBFD for small carrier bandwidths, e.g. < 40 MHz, is unpractical since the SBFD guardbands may become excessive with respect to the usable data resource blocks, and it becomes also challenging to place reference signals and channels such as SSB and CORESET 0.

For FR1, RAN4 to prioritize SBFD requirements for BS carrier bandwidths between 50 MHz and 100 MHz.

Next, we discuss the FFS from the agreement above on the size of the subband and/or guardband. In line with the R18 study, the frequency resources can be split following roughly a 80:20 ratio between DL and UL as a starting point. This configuration allows to obtain UL coverage and latency benefits without significantly impacting the DL performance. In terms of arrangement of the UL and DL subbands, both DUD and DU/UD configuration should be supported.

SBFD configurations with both DUD and DU arrangement of the subbands should be supported. The size of the UL subband should be around 20% of the channel bandwidth.

We think it is furthermore convenient that the size of the UL subband (in number of RBs) is aligned one of the transmission bandwidth configuration allowed in TS 38.104 specifications. That is, for 30 kHz SCS configuration, there are 273 RBs in the 100 MHz carrier bandwidth where 51 RBs (equivalent to the number of RBs in a 20 MHz carrier) can be dedicated to the UL subband.

For FR1 SBFD requirements, the size of the UL subband is aligned with the transmission bandwidth configurations specified in TS 38.104. That is 51 RBs for a 20 MHz channel and 30 kHz SCS.

The next question is whether guardbands should be fixed as well. Five resource blocks between UL and each DL subband was assumed for FR1 30 kHz SCS configuration, which was primarily motivated by the existing minimum guardband requirements specified in TS 38.104. At least from base station perspective, this requirement can be considered as starting point for the BS RF requirements which leaves 212 remaining RBs for DL in case of DUD configuration (106 RBs in each DL subband), and 217 RBs for DU configuration. These DL subband sizes are also conveniently aligned with the transmission bandwidth configurations for 40 MHz and 80 MHz channels. The following configurations are therefore proposed as a starting point:

For FR1 SBFD requirements, from base station perspective, the following assumptions for guardbands and UL and DL subbands can be used as starting point:

  - For FR1 DUD configuration in 100 MHz channel and 30 kHz SCS: < ND, NU, NG > = <106, 51, 5>

  - For FR1 DU or UD configuration in 100 MHz channel and 30 kHz SCS: < ND, NU, NG > = <217, 51, 5>

On FR2-1, possible bandwidths are 50, 100, 200, and 400 MHz. We have slight preference for focusing on 100 MHz and 200MHz channels with 120 kHz SCS. For this case, the SBFD subband configuration can follow the assumptions agreed by RAN1 in the study item phase, that is < ND, NU, NG > = <25, 14, 1> and < ND, NU, NG > = <52, 26, 1>, respectively

For FR2-1 SBFD requirements, from base station perspective, the following assumptions for guardbands and UL and DL subbands can be used as starting point:

  - For FR2-1 DUD configuration in 100 MHz channel and 120 kHz SCS: < ND, NU, NG > = <25, 14, 1>

  - For FR2-1 DUD configuration in 200 MHz channel and 120 kHz SCS: < ND, NU, NG > = <52, 26, 1>

Note that the requirements proposed above are primarily defined from base station testing and declaration perspective. However, the SBFD configurations which the base station can operate with should not be restricted to those specified by RAN4. For instance, considering different possible UE RF implementations and performance (e.g. new SBFD-aware UEs may apply digital filter, while legacy UEs may not), deployment types and cross-link interference conditions in SBFD deployments, the gNB may operate with a different guardband (e.g. larger) that that for which requirements were defined. As proposed by some companies in RAN4#111, we think it is convenient to specify a limited set of configurations in RAN4 specifications, and it is up to the gNB vendor to determine (and declare) which of the configurations it supports. We agree with Ericsson that the number of SBFD configurations can be limited to five for FR1, and three for FR2-1.

The value of guardbands, DL and UL subbands for SBFD operation in the field are not restricted to those in RAN4 specifications. RAN4 may agree one or more ‘typical’ values for which requirements are defined, while the gNB should still have the possibility to operate with a different (e.g. larger) guardband and UL and DL subband sizes.

Requirements are defined for a limited number of SBFD configurations (considering different bandwidth, center frequency, and DUD or DU arrangement). Limit the number of SBFD configurations to five for FR1, and three for FR2-1.

# Conclusion

In this contribution, our views on the SBFD BS RF requirements are presented. The following proposals were listed.

- Supporting SBFD for small carrier bandwidths, e.g. < 40 MHz, is unpractical since the SBFD guardbands may become excessive with respect to the usable data resource blocks, and it becomes also challenging to place reference signals and channels such as SSB and CORESET 0.

- RAN4 to not consider any CLI handling scheme impact on the RF requirements discussions.

For FR1, RAN4 to prioritize SBFD requirements for BS carrier bandwidths between 50 MHz and 100 MHz.

SBFD configurations with both DUD and DU arrangement of the subbands should be supported. The size of the UL subband should be around 20% of the channel bandwidth.

For FR1 SBFD requirements, the size of the UL subband is aligned with the transmission bandwidth configurations specified in TS 38.104. That is 51 RBs for a 20 MHz channel and 30 kHz SCS.

For FR1 SBFD requirements, from base station perspective, the following assumptions for guardbands and UL and DL subbands can be used as starting point:

  - For FR1 DUD configuration in 100 MHz channel and 30 kHz SCS: < ND, NU, NG > = <106, 51, 5>

  - For FR1 DU or UD configuration in 100 MHz channel and 30 kHz SCS: < ND, NU, NG > = <217, 51, 5>

For FR2-1 SBFD requirements, from base station perspective, the following assumptions for guardbands and UL and DL subbands can be used as starting point:

  - For FR2-1 DUD configuration in 100 MHz channel and 120 kHz SCS: < ND, NU, NG > = <25, 14, 1>

  - For FR2-1 DUD configuration in 200 MHz channel and 120 kHz SCS: < ND, NU, NG > = <52, 26, 1>

The value of guardbands, DL and UL subbands for SBFD operation in the field are not restricted to those in RAN4 specifications. RAN4 may agree one or more ‘typical’ values for which requirements are defined, while the gNB should still have the possibility to operate with a different (e.g. larger) guardband and UL and DL subband sizes.

Requirements are defined for a limited number of SBFD configurations (considering different bandwidth, center frequency, and DUD or DU arrangement). Limit the number of SBFD configurations to five for FR1, and three for FR2-1.

# References

- R4-2409958 “Way Forward for [111][313] NR_duplex_evo”, RAN4#111, Fukuoka, Japan, 20th-24th May, 2024