3GPP TSG-RAN4 Meeting #112	R4-2413243

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2496 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_n14-Core, TEI16) Correction of notes for UE output power |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, NR_n14-Core |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-16, band n14 was introduced with the following Note 6 introduced in Table 6.2.1-1 (UE Power Class): NOTE 6: Generally, PC1 UE for Band n14 is not targeted for smartphone form factor. The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. As Note 6 in Table 6.2.1-1 has been reused for other bands in Rel-18, in this CR we provide corrections to the Note 6 wording to make it band-agnostic, while Band14-specific Note is introduced as a separate entry, i.e. Note 7. Furthermore, referring to TR 21.801 clause 6.6.5.6, 3GPP drafitng rules are properly implemented for the notes. Examples of propoper notes handling in tables can be found in TR 21.801 table 2, or table H.2, or table K.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Note 6 wording is updated to make it band-agnostic, while Band14-specific Note is introduced as a separate entry, i.e. Note 7. 3GPP drafting rules are implemented for the notes in Table 6.2.1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity of Note 6 in Table 6.2.1-1 and its n14 content being reused for other bands would remain. 3GPP drafting rules would not be followed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

### 6.2.1	UE maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n2 |  |  |  |  | 26 | +2/-3 (Note 3) | 23 | ±2 (Note 3) |
| n3 |  |  |  |  | 26 | +2/-3 (Note 3) | 23 | ±2 (Note 3) |
| n5 |  |  |  |  |  |  | 23 | ±2 |
| n7 | 31 (Note 6, 7) | +2/-3 |  |  | 26 | +2/-3 (Note 3) | 23 | ±2 (Note 3) |
| n8 |  |  |  |  | 26 | +2/-3 (Note 3) | 23 | ±2 (Note 3) |
| n12 |  |  |  |  |  |  | 23 | ±2 (Note 3) |
| n13 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n14 | 31 (Note 6) | +2/-3 |  |  | 26 | +2/-3 | 23 | ±2 |
| n18 |  |  |  |  |  |  | 23 | ±2 |
| n20 |  |  |  |  |  |  | 23 | ±2 (Note 3) |
| n24 |  |  |  |  |  |  | 23 | +2/-3 (Note 3) |
| n25 | 31 (Note 6) | +2/-3 |  |  | 26 | +2/-3 (Note 3) | 23 | ±2 (Note 3) |
| n26 |  |  |  |  |  |  | 23 | ±2 (Note 3) |
| n28 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n30 |  |  |  |  |  |  | 23 | ±2 |
| n31 | 31 (Note 6) | +2/-3 |  |  |  |  | 23 | ±2 |
| n34 |  |  | 29 (Note 5) | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n38 |  |  |  |  |  |  | 23 | ±2 |
| n39 |  |  | 29 (Note 5) | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n40 | 31 (Note 6) | +2/-3 | 29 (Note 5) | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n41 | 31 (Note 6) | +2/-3 | 29 (Note 5) | +2/-3 (Note 3) | 26 | +2/-3 (Note 3) | 23 | ±2 (Note 3) |
| n47 |  |  |  |  |  |  | 23 | ±2 |
| n48 |  |  |  |  |  |  | 23 | +2/-3 |
| n50 |  |  |  |  |  |  | 23 | ±2 |
| n51 |  |  |  |  |  |  | 23 | ±2 |
| n53 |  |  |  |  |  |  | 23 | ±2 |
| n54 |  |  |  |  |  |  | 23 | ±2 |
| n65 |  |  |  |  |  |  | 23 | ±2 |
| n66 | 31 (Note 6) | +2/-3 |  |  | 26 | +2/-3 | 23 | ±2 |
| n70 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n71 | 31 (Note 6) | +2/-3 |  |  | 26 | +2/-3 | 23 | +2/-2.5 |
| n72 | 31 (Note 6) | +2/-3 |  |  |  |  | 23 | ±2 |
| n74 |  |  |  |  |  |  | 23 | ±2 |
| n77 | 31 (Note 6) | +2/-3 | 29 (Note 5) | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n78 | 31 (Note 6) | +2/-3 | 29 (Note 5) | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n79 |  |  | 29 (Note 5) | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n80 |  |  |  |  | 26 | +2/-3 (Note 3) | 23 | ±2 (Note 3) |
| n81 |  |  |  |  |  |  | 23 | ±2 |
| n82 |  |  |  |  |  |  | 23 | ±2 |
| n83 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n84 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n85 | 31 (Note 6) | +2/-3 |  |  | 26 | +2/-3 (Note 3) | 23 | ±2 (Note 3) |
| n86 |  |  |  |  |  |  | 23 | ±2 |
| n89 |  |  |  |  |  |  | 23 | ±2 |
| n91 |  |  |  |  |  |  | 23 | ±2 (Note 3, 4) |
| n92 |  |  |  |  |  |  | 23 | ±2 (Note 3, 4) |
| n93 |  |  |  |  |  |  | 23 | ±2 (Note 3, 4) |
| n94 |  |  |  |  |  |  | 23 | ±2 (Note 3, 4) |
| n95 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n97 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n98 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n99 |  |  |  |  |  |  | 23 | +2/-3 (Note 3) |
| n100 | 31 (Note 6) | +2/-3 |  |  |  |  | 23 | ±2 |
| n101 | 31 (Note 6) | +2/-3 |  |  |  |  | 23 | ±2 |
| n104 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n105 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n106 |  |  |  |  |  |  | 23 | ±2 |
| n109 |  |  |  |  |  |  | 23 | ±2 (Note 3, 4) |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Power class 3 is default power class unless otherwise stated. NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. NOTE 5: Achieved via dual Tx. NOTE 6: PC1 UE is not targeted for smartphone form factor. NOTE 7: The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. |  |  |  |  |  |  |  |  |

------------------------------ End of modified section -------------------------