# 3GPP TSG-RAN WG4 Meeting #112	R4-2413246

# Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	Follow-up on MU/TT handling for OTA measurements

Agenda Item:	5.31.3

Source: 	Huawei, HiSilicon

Document for:	Discussion

# Introduction

During RAN4#111, WF on the MU/TT values for OTA conformance testing of NCR was approved in [2]. More specifically, it was about potential reuse of MU/TT values from RAN5 FR2 UE specification, for the purpose of radiated MT requirements testing in case of the NCR type 2-O.

The MU/TT approach in WF [1] was decided to be confirmed in the maintenance phase.

In this contribution we provide a follow-up discussion on the MU/TT values for OTA conformance testing of NCR.

# Discussion

# Analysis of UE TT values

Due to DUT size and required OTA chamber (and quiet zone) dimensions, it was identified in the WF [1] that there is some restriction in the reuse of the UE-specific MU/TT values for the MT-related requirements. While NCR is composed of NCR-Fwd and NCR-MT, during OTA testing of NCR device would need to fit within quite zone of the OTA chamber for radiated requirements testing.

According to RAN5 specification TS 38.521-2 annex F.3, the MU/TT values were defined for the EUT with max device size ≤ 30 cm for FR2 subranges FR2a, FR2b, FR2c, and FR2d (i.e. from 23.45 to 49 GHz), as extracted below for reference for the IFF (Indirect Far Field) test:

Observation 1: RAN5 specification captures FR2 UE size limit for the IFF (i.e. CATR) testing.

Based on the above observation, previous proposals captured in the WF [2] are justified, i.e. Reuse of 38.521-2 TT values for radiated conformance testing of MT requirements of NCR type 2-O may not be justified for some products due to too large NCR product dimensions (i.e. EUT size > 30 cm). Reuse of TT values from UE specification TS 38.521-2 is technically justified only under certain conditions, i.e.: to keep the MU budget valid, the test methodology needs to be followed, and the device size limit od 30cm needs to be respected, as captured in TS 38.521-2.

Based on the above discussion, the following is proposed:

Proposal 1: As a justification for the reuse of FR2 UE TT values from TS 38.521-2 into NCR test specification TS 38.115-2 for MT-specific requirements, reflect device size limitation of 30cm in TS 38.115-2.

Related CR to TS 38.115-2 was submitted in [3].

# Analysis of UE spec reuse

At least for some of NCR requirements it is not possible to reuse BS, nor IAB values of MU/TT, as those are UE-specific requirements. Referring to the TS 38.106 NCR core specification, the below listed tests refer to the UE specification TS 38.101-2, for MT requirement specified for the Local Area class of the NCR. Test specification TS 38.115-2 further refers to the UE conformance testing specification TS 38.521-2. We provide brief analysis, highlighting some of the issues related to the reuse of UE conformance testing specification for NCR conformance testing.

- OTA ACLR requirements for NCR type 2-O: refer to clause 6.5.2.3 in TS 38.101-2. Related test requirement for UE PC3 is captured in TS 38.521-2, clause 6.5.2.3.5, where the following TT values were captured so far:

Table 6.5.2.3.5-1a: Test Tolerance (Adjacent channel leakage ratio) for PC3

|  |  | Channel bandwidth / NRACLR / Measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Test ID | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| NRACLR for band n257, n258, n261 | 1-2, 4-5 | 4.10 | 4.49 | 4.66 | 5.06 |
|  | 3, 6 | 4.08 | 4.45 | 4.59 | 5.06 |
|  | 7-9 | 4.15 | 4.59 | 4.85 | 3.34 |
|  | 10-12 | 4.36 | 4.98 | 4.06 | 1.46 |
|  | 13-15 | 4.17 | 4.62 | 4.91 | 2.99 |
| NRACLR for band n260 | 1-2, 4-5 | 4.48 | 4.65 | 4.97 | - |
|  | 3, 6 | 4.45 | 4.58 | 4.84 | - |
|  | 7-9 | 4.58 | 4.84 | 5.31 | - |
|  | 10-12 | 4.97 | - | - | - |
|  | 13-15 | 4.62 | 4.90 | - | - |
| NRACLR for band n259 | 1-2, 4-5 | [5.61] | [5.91] | [6.44] | - |
|  | 3, 6 | [5.55] | [5.79] | [6.23] | - |
|  | 7-9 | [5.79] | [6.23] | - | - |
|  | 10-12 | [6.44] | - | - | - |
|  | 13-15 | [5.84] | [6.33] | - | - |

However, the NCR test specification TS 38.115-2 has reused network node measurement uncertainty as specified in the following table:

Table 6.5.2.5-7: Measurement uncertainty for NRACLR for FR2-1

|  | The applicable frequency range |  |  |
| --- | --- | --- | --- |
|  | 24.25 – 29.5 GHz | 37 – 43.5 GHz | 43.5 GHz < f ≤ 48.2 GHz |
| Relative ACLR | ±2.3 dB | ±2.6 dB | ±2.8 dB |
| Absolute ACLR | ±2.7 dB | ±2.7 dB | ±2.9 dB |

- OTA OBUE requirements for NCR type 2-O: refers to clause 6.5.2.1 in TS 38.101-2. Related test requirement for UE is captured in TS 38.521-2, clause 6.5.2.1.5, where the following TT values were captured:

Table 6.5.2.1.5-1a: Test Tolerance (Spectrum emission mask) for PC3

| Test Metric | FR2a | FR2b | FR2c |
| --- | --- | --- | --- |
| IFF (Max device size ≤ 30 cm) | 3.33 dB | 3.58 dB | 4.46 dB |

However, the NCR test specification TS 38.115-2 has reused network node measurement uncertainty as specified in the following table:

Table 6.5.3.4.1-4: measurement uncertainty for OBUE for FR2-1

|  | The applicable frequency range |  |  |
| --- | --- | --- | --- |
|  | 24.25 – 29.5 GHz | 37 – 43.5 GHz | 43.5 GHz < f ≤ 48.2 GHz |
| OBUE: | ±2.7 dB | ±2.7 dB | ±2.9 dB |

- OTA Tx spurious emissions requirements for NCR type 2-O: refers to clause 6.5.3 in TS 38.101-2. Related test requirement for UE is captured in TS 38.521-2, clause 6.5.3.1, where TT = 0 was used, as this is regulatory requirement.

- OTA transmitter transient period: refers to clause 6.3.3.2 in TS 38.101-2. Related test requirement for UE is captured in TS 38.521-2, clause 6.3.3.2, where TT = 0 was used.

- OTA reference sensitivity: refers to PC3 in clause 7.3.2.3 in TS 38.101-2. Related test requirement for UE PC3 is captured in TS 38.521-2, clause 7.3.2, where the following TT values were captured for PC3 UE:

Table 7.3.2.5-3e: Test Tolerance (Reference sensitivity for power class 3)

| Test Metric | FR2a, FR2b | FR2c |
| --- | --- | --- |
| IFF (Max device size ≤ 30 cm) | 2.41 dB, NTC 2.52 dB, ETC | 2.85 dB, NTC 2.92 dB, ETC |

- OTA maximum input level: refers to clause 7.4 in TS 38.101-2. Related test requirement for UE is captured in TS 38.521-2, clause 7.4.

When referring to the TS 38.521-2 clause 7.4, the Measurement uncertainty is listed as an open item and marked as FFS. Furthermore, TS 38.521-2 clause 7.4.2 states that this requirement is not testable:

7.4.2 Test applicability The minimum conformance requirements in this test case are not testable due to maximum input level unachievable in IFF OTA test setup. Other test setups have not been analysed. Thus the test case will not be tested as part of UE conformance testing. NOTE: This does not preclude the test from being used for R&D or other purposes if deemed useful to all types of NR UEs release 15 and forward.

Observation 2: OTA maximum input level is currently not testable.

- OTA adjacent channel selectivity: refers to clause 7.5 in TS 38.101-2. Related test requirement for UE is captured in TS 38.521-2, clause 7.5.5.

When referring to the TS 38.521-2 clause 7.5 on ACS, one can find that there are still unresolved issues:

Editor’s note: The following aspects are either missing or not yet determined: - Measurement Uncertainty is FFS for power classes other than 1, 3 and 5. - The test case is incomplete for band n259 and for band n262. - The minimum conformance requirements for Case 2 in this test case are not testable due to maximum input level unachievable in IFF OTA test setup. Other test setups have not been analysed. - For power class 1, if testing were extended beyond 100MHz, potential relaxation required is FFS.

Furthermore, the referred test requirement in TS 38.521-2, clause 7.5.5 captures the following notes:

NOTE 4: Core requirement cannot be tested due to testability issue and test requirement for wanted signal and interferer includes relaxation to achieve feasible interferer power level. NOTE 5: Core requirement cannot be tested due to testability issue.

Observation 3: Multiple cases of the OTA ACS requirements cannot be tested due to testability issues.

- OTA blocking characteristics: refers to clause 7.6 in TS 38.101-2. Related test requirement for UE is captured in TS 38.521-2, clause 7.6.2.5.

When referring to the TS 38.521-2 clause 7.6.2 on in-band blocking, one can find that there are still unresolved issues:

Editor’s note: The following aspects are either missing or not yet determined: - Measurement uncertainty is FFS for power classes other than 1, 3 and 5. - The test case is incomplete for band n259 and for band n262. - For power class 1, if testing were extended beyond 100MHz, potential relaxation required is FFS.

Furthermore, the referred test requirement in TS 38.521-2, clause 7.6.2.5 captures the following notes:

NOTE 7: Core requirement cannot be tested due to testability issue and test requirement for wanted signal and interferer includes relaxation to achieve feasible interferer power level. NOTE 8: Core requirement cannot be tested due to testability issue.

Observation 4: Multiple cases of the OTA blocking requirements cannot be tested due to testability issues.

- OTA spurious emissions: refers to clause 7.9 in TS 38.101-2. Related test requirement for UE is captured in TS 38.521-2, clause 7.9.5. While test description and test requirement is captured in the TS 38.521-2, there are still unresolved issues:

Editor's note: Following aspects are either missing or not yet determined: - The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n259, n260 and n261. - TRP Measurement uncertainty is TBD for above 87 GHz. - Measurement Uncertainties and Test Tolerances are FFS for power class other than PC1, PC3 and PC5. - Connection diagram between SS and UE in TS 38.508-1 [10] Annex A is FFS. - Test procedure is FFS for laptop. - For a transition period until RAN5#103 meeting (May 2024), previous fine/coarse TRP measurement grid and offset values for corresponding coarse TRP measurement in TS 38.521-2 V17.2.0 are allowed for TE implementation.

As observed above, there are multiple issues with the reuse of TS 38.521-2 requirements. By referring from TS 38.115-2 to TS 38.521-2, it makes TS 38.115-2 incomplete, and NCR not fully testable. Therefore, in order to resolve this issue and reassure proper specification handling, we proposed the following:

Proposal 2: Agree on the LS to RAN5, informing on the reuse of the FR2 UE TT values in TS 38.115.2, for the purpose of MT-specific requirements testing. RAN4 to ask RAN5 to be updated on any further plans or updates of test requirements reused in TS 38.115-2.

Related draft LS to RAN5 was submitted in [4].

# Conclusion

Based on the above discussion, the following proposals were formulated:

Proposal 1: As a justification for the reuse of FR2 UE TT values from TS 38.521-2 into NCR test specification TS 38.115-2 for MT-specific requirements, reflect device size limitation of 30cm in TS 38.115-2.

Proposal 2: Agree on the LS to RAN5, informing on the reuse of the FR2 UE TT values in TS 38.115.2, for the purpose of MT-specific requirements testing. RAN4 to ask RAN5 to be updated on any further plans or updates of test requirements reused in TS 38.115-2.

# References

[1]	R4-2409793	Proposal on MU/TT handling for OTA measurements, Huawei, HiSilicon

[2]	R4-2409973	WF on MU/TT values for OTA conformance testing of NCR, 	Huawei, HiSilicon

[3]	R4-2413254	CR to TS 38.115-2: NCR type 2-O testing: MT requirements testing restrictions, Huawei, HiSilicon

[4]	R4-2413254	Draft LS on NCR testing, Huawei, HiSilicon