# 3GPP TSG-RAN WG4 Meeting #112	R4-2413247

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	Huawei, HiSilicon

Title: 		Initial discussion on for Ku operating bands

Agenda Item:	8.9.3

Document for:	Discussion

# Introduction

During RAN#104 meeting, new WID on the Ku band introduction was agreed in [1].

In this contribution, we provide initial discussion on the standardisation of the Ku band, looking into system parameters aspects.

# Discussion

## Frequency range consideration

WID wording has captured that the introduction of the Ku band will require extension of the FR1-NTN and/or FR2-NTN Frequency Range definition(s).

Referring to the existing NTN specifications, the following Frequency Ranges were defined so far:

| FR1-NTN (NOTE 1) | 410 – 7125 |
| --- | --- |
| FR2-NTN (NOTE 2) | 17300 – 30000 |

When comparing FR1-NTN, FR2-NTN and all the considered Ku ranges (1a, 1b, 2a, 2b), one can notice that the proposed Ku bands are not covered by FR1-NTN, nor by FR2-NTN.

At this stage, one can refer to the Rel-16 study item on 7-24 GHz for terrestrial networks, and related TR 38.820 [2]. One of the issues studies under that study was to investigate on the potential boundary between FR1 and FR2. Even if it was not possible to draw conclusion on that matter during Rel-16, there was general understanding that such boarder would be somewhere in between 10 and 15 GHz.

Figure 1: FR1-NTN, FR2-NTN, Ka and Ku bands overview

We draw number of initial observations on the frequency range assignment for the Ku band:

Observation 1: Introduction of Ku band will require to update either FR1-NTN, or FR2-NTN definition.

Observation 2: Ku band shall be fully contained within either FR1-NTN, or FR2-NTN (and not split among them).

Observation 3: For the decision on the FR selection for Ku band, both system parameters, UE RF, as well as SAN aspects shall be considered.

Observation 4: Discussion on FR selection for Ku band is valid for NTN only, with the assumption that it does not affect the current terrestrial network definitions of FR1 nor FR2, and is not supposed to apply to future terrestrial bands defined in the Ku frequency range. As this has been captured in the WID, there is no need for RAN4 to have further discussion on this aspect.

Thinking of possible FR1-NTN extension to cover Ku, there are two approaches to consider:

- Variant a: Extension of FR1-NTN with Ku range only, i.e. 10700-14500 MHz added to the FR1-NTN,

- Variant b: Continuous extension of FR1-NTN to cover Ku range, i.e. addition of the Ku itself as well as the intermediate range among FR1-NTN and Ku (i.e. 7125 – 10700 MHz)

| Frequency range designation | Corresponding frequency range (MHz) |
| --- | --- |
| FR1-NTN: Variant a | 410 – 7125; 10700 – 14500 |
| FR1-NTN: Variant b | 410 – 14500 |

Similar, two variants in the case of FR2-NTN extension:

| Frequency range designation | Corresponding frequency range (MHz) |
| --- | --- |
| FR2-NTN: Variant a | 10700 – 14500; 17300 – 30000 |
| FR2-NTN: Variant b | 10700 – 30000 |

Formally speaking we should focus on the Ku range only. Additionally, it would be also good to further verify NTN regulatory aspects of those intermediate ranges (i.e. 7125 – 10700 MHz in case of FR1 extension, or 14500 – 17300 MHz in case of FR1 extension).

On the other hand, it is seen to be more pragmatic to simply extend FR1 or FR2 in a continuous fashion, while exact frequency allocation for new bands would be addressed by the new NTN bands (and not FR1/FR2).

Proposal 1: Regardless of the decision on FR1-NTN, or FR2-NTN extension for Ku range introduction, FR extension shall be done in a continuous fashion based on Variant b, i.e. 410 – 14500 MHz in case of FR1-NTN, or 10700 – 30000 MHz in case of FR2-NTN extension.

Before we draw final conclusion on the incorporation of the Ku into FR1-NTN, or FR2-NTN, we need to look at the requested channel bandwidths first – see section 2.4.

## Operating bands consideration

According to the WID [1], the Ku band is to be introduced in RAN4 specifications as FDD bands:

- For ITU regions 1 and 3:

  - Downlink 10.70 – 12.75 GHz

  - Uplink 13.75-14.5 GHz

- For ITU regions 2 excluding US:

  - Downlink 10.70 – 12.70 GHz

  - Uplink 13.75-14.5 GHz

- For ITU regions 1 and 3

  - Downlink 10.70 – 12.75 GHz

  - Uplink 12.75 - 13.25 GHz

- For ITU regions 2 excluding US

  - Downlink 10.70 – 12.70 GHz

  - Uplink 12.70 - 13.25 GHz

The above potential bands were derived based on the following principles:

- ITU regulation has defined slightly different DL Ku range (i.e. differing by 50 MHz on the upper edge of the DL Ku block) for Regions 1/3 and for Region 2.

- There are two UL blocks, which require to define at least two new FDD bands to cover Ku range,

- There is ongoing regulatory study for US driven by FCC (i.e. consideration of the 12.7 GHz – 13.25 GHz frequency range for terrestrial services), which is not concluded at this point of time.

Due to bullet 3 above, and to avoid unnecessary work delays, WID splits the work into Priority 1 and Priority 2 objectives, as extracted below:

For sake of progress, we suggest to clarify Priority 1 / Priority 2 aspects even further, to avoid parallel discussions and to have more focused scope, and step-wise approach:

Proposal 2: Priority 2 objectives of the WID are put on hold until Priority 1 objectives are agreed. Further work on Priority 2 objectives to leverage previous agreements on Priority 1 package, wherever possible.

Figure 2: Graphical representation of Ku ranges captured in the WID [1]

Observation 5: DL operation in frequency range from 10.7 GHz up to 12.7 GHz is allowed in all ITU regions.

When comparing all four frequency ranges on Figure 1, one can notice that DL range only differs at its upper edge (12.7 vs. 12.75 GHz), depending on the ITU region. There are two potential approaches to address this in RAN4 specification when defining new Ku band:

- Approach 1: define two separate new bands, e.g.

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| Ku#1 (Note 1) | 13750 – 14500 MHz | 10700 – 12750 MHz | FDD |
| Ku#2 (Note 2) | 13750 – 14500 MHz | 10700 – 12700 MHz | FDD |
| NOTE 1: This band is applicable in ITU Region 1 and Region 3. NOTE 2: This band is applicable in ITU Region 2. |  |  |  |

Such approach would be simplest from standardization point of view, while some may say that we can try to optimize it as the covered frequency ranges are very similar, and the “Ku#2” is fully contained within the “Ku#1” band. Furthermore, when considering Priority 2 objectives (as well as potential FCC outcomes), this approach may lead to a situation where Ku band will require at least 4 new bands to be introduced in RAN4 – this does not sound as optimal solution.

- Approach 2: define one global Ku band, with clarifications provided for ITU Region 2.

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| Ku (Note 1) | 13750 – 14500 MHz | 10700 – 12750 MHz | FDD |
| NOTE 1: In ITU Region 2, DL is limited to 10700 – 12700 MHz range. |  |  |  |

This approach would require additional standardisation effort (additional signalling, scheduling restrictions, etc.) to reassure that 12700 MHz to 12750 MHz is not used in Region 2 for NTN.

With the two approaches seen as doable in RAN4, the Approach 1 is seen as simpler, while it would be also acceptable to further investigate Approach 2.

Proposal 3: As a starting point, define two separate Ku bands for #1a and #1b (i.e. Priority 1 bands).

Example implementation depicted in table below:

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| Ku#1 (Note 1) | 13750 – 14500 MHz | 10700 – 12750 MHz | FDD |
| Ku#2 (Note 2) | 13750 – 14500 MHz | 10700 – 12700 MHz | FDD |
| NOTE 1: This band is applicable in ITU Region 1 and Region 3. NOTE 2: This band is applicable in ITU Region 2. |  |  |  |

## Channel bandwidths

Referring to the WID [1], it has clearly captured that support for Carrier Aggregation is not in the scope of the current Rel-19 WID. This clarification was added to address high work-load issue of the NTN package, and the Ku band specifically.

Initial proposal to add CA into the WID was to address 125 MHz (UL) and 250 MHz (DL) channel bandwidths for Ku band by reusing CA in NTN deployment, instead of introducing brand new NR channel bandwidths. Introduction on new NR channel bandwidths would trigger not only additional work on system parameters, but it would also require related implementation work.

Carrier Aggregation was not defined for NTN so far. While there is still a chance to introduce CA for NTN Ku band at later stage of the Rel-19 (i.e. December 2024 RAN Plenary checkpoint for RAN4), it is proposed to focus for now on the reuse of the existing NR channel bandwidths for Ku band, and refrain from introducing new channel bandwidths (at least not until February 2025 meeting).

Observation 6: Current WID allows only single carrier operation in Ku band.

Proposal 4: RAN4 to focus on the reuse of the existing NR channel bandwidths for Ku band, and postpone discussion on 125 MHz (UL) and 250 MHz (DL) new channel bandwidths for Ku band at least until February 2025 meeting.

For Priority 1 objectives, two possible approaches (not precluding others) were identified in WID for the Ku band and related channel bandwidths introduction:

In order to benefit from the similarities with the Ka band specification work in Release 18, the following approach for the Ku band is considered:

- Solution 1: Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing

- Solution 2: Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing

Below we present initial analysis of the pros and cons for both solutions:

|  | Pros | Cons |
| --- | --- | --- |
| Solution 1 (FR1-based) | - CHBW: Allows to reuse the already existing FR1 channel bandwidths from TN (more specifically 35MHz, as requested by some companies). - Standard: Allows to benefit from FR1 system parameters work reuse. - Deployment: ability to benefit from wider range of exiting FR1 baseband products and components. - CA: the potential future CA scheme would be able to support 125MHz and 250MHz combinations. | - Standard: FR1-based solution will not be able to benefit from Ka (FR2-based) band specification work (e.g. different numerologies); - UE: as the WID is focused on VSAT devices, the envisioned Ka- and Ku-capable devised would need to support both FR1-based and FR2-based numerologies; this is expected to raise the price tag of the terminal. - CA: there is underlying assumption, that CA will be supported for NTN to achieve 125/250MHz channel bandwidths. This cannot be granted at this point of time. |
| Solution 2 (FR2-based) | - Standard: Allows to benefit from Ka band specification work (see WID motivation) - UE: Easier and cheaper FR2-based VSAT devices integration for Ka- and Ku-capabilities. - CHBW: with definition of brand new FR2 channel bandwidths (e.g. 20, 35, 70, 125, 250MHz) there is no need to rely on CA (which is still not in the scope of Rel-19 WI). | - System parameters: new set of system parameters would be needed to support 20, 35, and 70 MHz channel bandwidths. - UE: FR2-based products availability is expected to be later than in case of FR1-based. - CA: it would not be possible to support 125 MHz combination with the CA scheme based on the existing FR2 channel bandwidths (unless considering solutions from Irregular channel bandwidth work would be considered in future). |

As this is the first meeting of the Ku WI, we suggest not to rush it and to collect feedback before taking decision among Solution 1 and Solution 2:

Proposal 5: Companies to provide further feedback on the selection of Solution 1 vs. Solution 2, based on a set of pre-defined metrics (to be decided during the meeting), e.g. standardisation effort, competitive Ku VSAT, device cost, Ka/Ku components integration, etc.

# Conclusions

Based on the discussion above, the following proposals were formulated:

Proposal 1: Regardless of the decision on FR1-NTN, or FR2-NTN extension for Ku range introduction, FR extension shall be done in a continuous fashion based on Variant b, i.e. 410 – 14500 MHz in case of FR1-NTN, or 10700 – 30000 MHz in case of FR2-NTN extension.

Proposal 2: Priority 2 objectives of the WID are put on hold until Priority 1 objectives are agreed. Further work on Priority 2 objectives to leverage previous agreements on Priority 1 package, wherever possible.

Proposal 3: As a starting point, define two separate Ku bands for #1a and #1b (i.e. Priority 1 bands).

Proposal 4: RAN4 to focus on the reuse of the existing NR channel bandwidths for Ku band, and postpone discussion on 125 MHz (UL) and 250 MHz (DL) new channel bandwidths for Ku band at least until February 2025 meeting.

Proposal 5: Companies to provide further feedback on the selection of Solution 1 vs. Solution 2, based on a set of pre-defined metrics (to be decided during the meeting), e.g. standardisation effort, competitive Ku VSAT, device cost, Ka/Ku components integration, etc.

# References

[1]		RP-241690	Introduction of Ku Band for NR NTN, RAN#104

[2]		3GPP TR 38.820	7 - 24 GHz frequency range