# 3GPP TSG-RAN WG4 Meeting #112	R4-2413248

# Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	(NR_RAIL_EU_900MHz-Core) Analysis of the updated ECC/DEC/(20)02 decision

Agenda Item:	4.3

Source: 	Huawei, HiSilicon

Document for:	Discussion

# Introduction

ECC Decision ECC/DEC/(20)02 on harmonised use of the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR) was amended 28-06-2024 [1].

In this contribution we provide analysis of recent ECC/DEC/(20)02 modifications, with related proposal on their implementation.

# Discussion

ECC Decision ECC/DEC/(20)02 was amended 28 June 2024 [1], introducing updates to the previous version of ECC Decision ECC/DEC/(20)02, dated 10-06-2022. Below, we present delta, depicting modifications introduced by 28-06-2024 version, on top of the previous 10-06-2022 version of ECC/DEC/(20)02.

Technical conditions for RMR BS using wideband technologies The least restrictive technical conditions (LRTC) defined in this section are in the form of a block-edge mask (BEM) applicable to wideband RMR BS. The technical conditions defined in this section are valid for a single RMR carrier using wideband technologies. The BEM is developed on the basis that detailed coordination and cooperation agreements would not be required to be in place prior to network deployment. Only non-AAS BS are considered. For radio access technologies other than GSM-R, the following parameters apply: The lower edge of the lowest Resource Block shall be 919.6 MHz. Table 2: General in-block requirement not mandatory RMR channel BW; Maximum e.i.r.p.. Any of the channel BW; The following value may be used by an administration in case an upper bound is desired: = Min {65 dBm/channel , Maximum e.i.r.p. specific to the channel BW}. Table 3: Specific in-block requirements for 5.6 MHz and 5 MHz channels mandatory for uncoordinated deployment RMR channel BW; Maximum e.i.r.p.. 5.6 MHz; = 62 dBm/5.6MHz. 5 MHz; = 64.5 dBm/5 MHz + (fDL – 922.1)×40/3 dB (Note 1). fDL is the centre frequency in MHz. NB-IoT in-band operation mode without power boost is allowed. NB-IoT guard-band operation mode and in-band operation mode with power boost are not allowed. Note 1: The calculation has been based on 25 Resource Blocks. The formula equally applies to 20 Resource Blocks..

Table 4: Specific in-block requirements for 3 MHz channels and channels ≤ 200 kHz mandatory for uncoordinated deployment RMR channel BW; Maximum e.i.r.p.. 3 MHz; = 61 dBm/3 MHz + (fDL – 921.4)×40/3 dB (Note 1). 200 kHz (Note 2); = 70.5 dBm/200kHz + (fDL – 921)×40/3 dB (Note 3). Note: fDL is the centre frequency in MHz. Note 1: Formula applicable to fDL 922.5 MHz. No specific e.i.r.p. restriction above. The calculation has been based on 15 Resource Blocks. It equally applies to 12 Resource Blocks. Note 2: Applicable to narrowband technologies. Note 3: Formula applicable to fDL 921.0 MHz. No specific e.i.r.p. restriction above..

To keep consistency with the corrected ECC decision, the following is proposed:

Proposal 1: Reflect updates to ECC/DEC/(20)02 in the TR 38.853.

# Conclusion

Based on the above discussion, the following proposals were formulated:

Proposal 1: Reflect updates to ECC/DEC/(20)02 in the TR 38.853.

# References

[1]	ECC/DEC/(20)02	ECC/DEC/(20)02 of 20 November 2020 on harmonised use of the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR), https://docdb.cept.org/document/16736

AAS: Active Antenna System