# 3GPP TSG-RAN WG4 Meeting #112	R4-2413258

# Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	Initial discussion on general aspects for introduction of NR bands n87 and n88

Agenda Item:	7.15.1

Source: 	Huawei, HiSilicon

Document for:	Discussion

# Introduction

During RAN#104 meeting, new WID on introduction of NR bands n87 and n88 was approved in [1], to allow refarming of E-UTRA Bands 87 and 88.

In this contribution we provide initial analysis of general and regulatory aspects for introduction of NR bands n87 and n88.

# Discussion

The following objectives were defined for the introduction on new NR operating bands n87 and n88 in [1]:

The core objectives of this WI are: - Standardization of two new NR FDD bands n87 and n88 in the 410 MHz range based on - E-UTRA band 87 (UL 410 - 415, DL 420 – 425 MHz) - E-UTRA band 88 (UL 412 - 417, DL 422 – 427 MHz) - During the work evaluate potential impacts of NR. If any impact on performance or any changes w.r.t. existing requirements of band 87 and 88. are found, address those during the work. - Specify band numbering (i.e. n87 and n88) system parameters and RF characteristics for 3 and 5 MHz channel bandwidth of the new bands. - 15 kHz SCS for 3MHz and 5MHz channel bandwidth - Only symmetric bandwidths are supported. - Address potential BS and UE co-existence issues, if any. - Specify the UE RF requirements for the new bands based on 2 Rx operation. - Update the related technical specifications to include support for the new band Note: These 2 new NR bands will be introduced in a REL-independent way starting from REL-15. The objectives of the Performance part work item are to - Update the related 3GPP NR technical specifications to include support for the new band e.g. BS conformance testing and RRM test cases.

New NR operating are designated for PPDR and PMR/PAMR operation in Europe. Legacy E-UTRA bands 87 and 88 were introduced in Rel-16 under LTE410_Europe_PPDR-Core WI. Related TR 36.762 [2] captures related studies for E-UTRA.

Proposal 1: Consider TR 36.762 as baseline for further NR-related analyses.

## 2.1	Regulatory overview

Referring to the ECO Documentation Database (https://docdb.cept.org/home), the following can be found on the frequency range in question:

The following regulatory decisions related to the PPDR in 410 – 430 MHz spectrum were identified:

- ECC/Dec/(16)02 [3] on Harmonised technical conditions and frequency bands for the implementation of Broadband Public Protection and Disaster Relief (BB-PPDR) systems

- ECC/Dec/(19)02 [4] on Land mobile systems in the frequency ranges 68-87.5 MHz, 146-174 MHz, 406.1-410 MHz, 410-430 MHz, 440-450 MHz and 450-470 MHz

ECC Decision (16)02 as well as ECC Decision (19)02 decisions were referred in the TR 36.762 [2]. It is proposed to double-check latest versions of the above mentioned ECC decisions before introduction of new NR bands, as both were last amended 08-03-2019.

Proposal 2: Consider ECC/Dec/(16)02 and ECC/Dec/(19)02 regulatory decisions as baseline for evaluation, together with underlying ECC Report 283 on compatibility studies.

ECC/Dec/(16)02 [3] decision has introduced additional spectrum for BB-PPDR in parts of the 400 MHz range, for 1.4 MHz, 3 MHz or 5 MHz channels, within the following paired frequency ranges:

410.0-415.0 MHz (uplink) / 420.0-425.0 MHz (downlink)

412.0-417.0 MHz (uplink) / 422.0-427.0 MHz (downlink)

ECC/Dec/(19)02 [4] decision is on Land mobile systems in the frequency ranges 68-87.5 MHz, 146-174 MHz, 406.1-410 MHz, 410-430 MHz, 440-450 MHz and 450-470 MHz. Land mobile systems in these frequency bands are used for PMR/PAMR (Private (Professional) Mobile Radio / Public Access Mobile Radio) applications. For the frequency ranges 410-430 MHz, this ECC Decision includes harmonised technical conditions to be applied for land mobile systems with channel bandwidth of 3 MHz or 5 MHz, which are of interest in this particular WI. This ECC decision further refers to the ECC Report 283 [5], which captures background of the technical requirements derivation.

ECC/Dec/(19)02 [4] captures decisions on the use of land mobile systems with channel bandwidths of 3 MHz and 5 MHz, also covering NB-IoT technologies:

that CEPT administrations wishing to introduce spectrum for land mobile systems with channel bandwidth of 1.25 MHz, 1.4 MHz, 3 MHz and 5 MHz in parts of the 400 MHz range shall apply technical conditions within the following paired frequency arrangements: - 410-415 MHz (uplink) / 420-425 MHz (downlink) those specified in Annex 2; - 411-416 MHz (uplink) / 421-426 MHz (downlink) those specified in Annex 2; - 412-417 MHz (uplink) / 422-427 MHz (downlink) those specified in Annex 2; - 451-456 MHz (uplink) / 461-466 MHz (downlink) those specified in Annex 3; - 452.5-457.5 MHz (uplink) / 462.5-467.5 MHz (downlink) those specified in Annex 3; that CEPT administrations wishing to introduce land mobile systems in the 410-430 MHz or 450-470 MHz frequency ranges based on NB-IoT and LPWAN technologies shall apply the technical requirements specified in Annex 4;

While the ECC/Dec/(19)02 [4] captures decision on the NB-IoT technologies operation in 410-430 MHz frequency range, the NB-IoT was not explicitly captured in the WID [1]. On the other hand, NB-IoT operation is also captured in 38-series NR specifications. Therefore, the following clarification is proposed:

Proposal 3: During n87/n88 investigations, cover analysis of regulatory requirements for NB-IoT operation in 410-430 MHz frequency range.

## 2.5	TR considerations

Referring to the WID [1], no new TR was considered for this WI. During Rel-16 work on E-UTRA bands under LTE410_Europe_PPDR-Core WI, TR 36.762 [2] was created. While we are still to review its content, it is expected that majority of findings in TR 36.762 would be also applicable to Rel-19 work on NR bands. Therefore, it would be most pragmatic if it would be allowed to capture any NR-specific findings also in the legacy 36-series TR. However, this is not strictly allowed by 3GPP rules. It is suggested to check offline with MCC how to approach this issue in the most pragmatic way, e.g. discuss if it would be allowed to at least capture additional NR-specific aspects as informative Notes in the TR 36.762; WI code of related CR may be “LTE410_Europe_PPDR-Core” under maintenance agenda.

Proposal 4: As the ECC/Dec/(19)02 and ECC/Dec/(19)02 regulations were already analysed in existing TR 36.762, stick to the WI agreements and not introduce another TR for n87/n88 introduction.

Proposal 5: Further check with MCC if it would be allowed to reuse the existing TR 36.762 for the purpose of NR-related updated for bands n87/n88.

## 2.5	WI timeline

Referring to the Excel spreadsheet in the WID [1], both Core and Perf parts were allocated to this RAN4#112 meeting.

However, according to TU spreadsheet compiled by RAN4 Chair, new spectrum items were captured with the focus on the Core part only, as expected:

Regardless of the above, we would like to highlight one issue related to both Core and Performance parts of the WI, i.e. introduction of co-existence/co-location requirements for new Rel-19 spectrum items.

There were multiple offline discussions ongoing on the potential optimisation of the approach, to avoid the need to have 10-20 CR for every new operating band introduction, just for sake of updating co-ex/co-location tables. This WID lists 14 specifications which needs to be update co-ex/co-location tables.

Before RAN4 invests time to work on those co-ex/co-location table updates for Rel-19 spectrum items, it is suggested to coordinate Rel-19 plans among the interested companies, so that work-load reduction can be achieved. One possibility is to aim for a new cleanup WI during December 2024 checkpoint for new RAN4 proposals.

To trigger offline follow-up discussion, we formulate a formal proposal:

Proposal 6: Before RAN4 initiates work on co-ex/co-location table updates for n87/n88 introduction, follow-up on work-load reduction framework during remaining Rel-19 timeframe.

# Conclusion

Based on the above discussion, the following proposals were formulated:

Proposal 1: Consider TR 36.762 as baseline for further NR-related analyses.

Proposal 2: Consider ECC/Dec/(16)02 and ECC/Dec/(19)02 regulatory decisions as baseline for evaluation, together with underlying ECC Report 283 on compatibility studies.

Proposal 3: During n87/n88 investigations, cover analysis of regulatory requirements for NB-IoT operation in 410-430 MHz frequency range.

Proposal 4: As the ECC/Dec/(19)02 and ECC/Dec/(19)02 regulations were already analysed in existing TR 36.762, stick to the WI agreements and not introduce another TR for n87/n88 introduction.

Proposal 5: Further check with MCC if it would be allowed to reuse the existing TR 36.762 for the purpose of NR-related updated for bands n87/n88.

Proposal 6: Before RAN4 initiates work on co-ex/co-location table updates for n87/n88 introduction, follow-up on work-load reduction framework during remaining Rel-19 timeframe.

# References

[1]	RP-241663		Introduction of NR bands n87 and n88, WID, RAN#104

[2]	TR 36.762		410 – 430 MHz Evolved Universal Terrestrial Radio Access (E-UTRA) Frequency Division Duplex (FDD) Band(s) for LTE Public Protection and Disaster Relief (PPDR) and Professional Mobile Radio (PMR) / Public Access Mobile Radio (PAMR) in Europe

[3]	ECC/Dec/(16)02 	Harmonised technical conditions and frequency bands for the implementation of Broadband Public Protection and Disaster Relief (BB-PPDR) systems, https://docdb.cept.org/document/941

[4]	ECC/Dec/(19)02 	Land mobile systems in the frequency ranges 68-87.5 MHz, 146-174 MHz, 406.1-410 MHz, 410-430 MHz, 440-450 MHz and 450-470 MHz, https://docdb.cept.org/document/9680

[5]	ECC Report 283 	Compatibility and sharing studies related to the introduction of broadband and narrowband systems in the bands 410-430 MHz and 450-470 MHz, https://docdb.cept.org/document/6033