3GPP TSG-RAN WG4 Meeting # 112	R4-2413262

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.10.2.2

Source: 		Qualcomm Incorporated

Title: 	Views on Rel-19 ATG UE RF requirements

Document for:	Approval

# Introduction

In RAN4#111, UE RF requirements for Rel-19 ATG enhancement were discussed and the agreements are captured in WF on Rel-19 ATG UE requirements [1].

# 2. Discussion

## 2.1 Intra-band CA

In RAN4#111, the BCS for DL CA_n79C was discussed and the following agreements were made.

Issue 2-1-1: BCS for DL CA_n79C Agreement: - Reuse the existing BCS table for ATG - Clarify that uplink CA is not supported for ATG UE elsewhere for Rel-19 - FFS on whether BCS#4 and #5 are not supported for ATG UE for Rel-19

Regarding whether BCS4 and BCS5 are supported for ATG UE, technically, BCS4 and BCS5 can be applied for ATG UE since BCS4 and BCS5 have been introduced for NR UE after completing the necessary analysis. While whether to specify BCS4/5 is dependent on operator’s request. In addition, to follow the guidelines for band combination with BCS4/BCS5 in TR38.862, BCS4/BCS5 shall be requested together, but BCS5 can’t be reported together with BCS4.

The following are the rules for applying BCS4/BCS5 for band combination request: - BCS4/BCS5 apply to SUL, NR CA, NR DC and SUL and/or NR CA part of inter-band MR-DC while it does not apply to intra-band MR DC. - BCS4/ BCS5 shall be requested together, but BCS5 can’t be reported together with BCS4. - If needed, traditional BCSs are allowed for all releases. For a new band combination in Rel-17 and onwards, if BCS4/BCS5 are requested, traditional BCSs are allowed pending on the proponents, the network of the proponents of BCS4/BCS5 is demanded to recognize BCS4/BCS5.

Proposal 1: BCS4 and BCS5 can be applied for ATG UE. Whether to specify BCS4 and BCS5 for DL CA_n79C is dependent on operator’s request. BCS4/BCS5 shall be requested together, but BCS5 can’t be reported together with BCS4.

## 2.2 Inter-band CA

For antenna type for each band in inter-band CA, the following agreements were captured in [1].

Issue 3-1: clarify the antenna type for each band for inter-band CA Agreement: - No limitation on antenna types for ATG CA - FFS on whether to assume omni-antenna type can be assumed for both band n3 and n39 in DL CA_n3-n39 - FFS on whether new capability is needed for ATG CA

In Rel-18, there are two types of antenna, i.e., omni-direction antenna and phase array antenna defined via per band capability. For inter-band CA case, it is straightforward to assume the per band capability for antenna type could be applied for the supported band in a band combination. While it is possible that UE antenna type is different from per band capability to the same band in a band combination. For example, for low band (e.g., n8), UE might use omni-direction antenna for single carrier case, and for high band (e.g., n79), UE might use phase array antenna for single carrier case due to higher frequency. But for CA, only one UE type, e.g., omni-direction might be used due to implementation limitations.  In this case, ATG UE should report capability for antenna type, i.e., omnidirectional or array, per band combination. For example, below per band combination, UE antenna type capability can be signaled by UE:

- CA_nA-nB: Band A: {omnidirectional, phase array}, Band B: {omnidirectional, phase array}

To reduce the signal overhead, this signal could be optional which means by default the same UE antenna type per band would apply for that band in a band combination.

Proposal 2: New capability is needed for UE antenna type which is per band combination. The signal is optional that means by default the same UE antenna type per band would apply for that band in a band combination.

For the issue of whether to allow n39 UL for CA_n3A-n39A, the implementation solutions have been extensively investigated for NR UE and the conclusion is that n39 UL is not allowed in CA_n3-n39 [2,3]. Therefore, the same conclusion should be applied and there is no need to extend the discussion in Rel-19 ATG UE RF discussion.

Proposal 3: Not allow n39 UL for CA_n3A-n39A for ATG UE.

For the issue of BCS for DL CA_n3_n39, since only BCS0 is defined in TS38.101-1 v18.6.0, it should naturally specify the same BCS for ATG UE.

| CA_n3A-n39A | n3 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
| --- | --- | --- | --- | --- |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 35, 40 |  |

Proposal 4: To specify BCS0 for CA_n3A-n39A.

## 2.3 UL MIMO

The issues of whether ULFPTx mode for UL-MIMO is supported for ATG UE were discussed in previous meetings.

Issue 4-1-1: ULFPTx mode for UL-MIMO Agreement: - FFS on whether ULFPTx mode for UL-MIMO is supported for ATG UE

The intention of ULFPTx mode is to increase UL power with 2Tx for NR UE. While for ATG, there is no UE power class, and the maximum output power of ATG UE is indicated by signalling. This means that the maximum transmit power of ATG UE can be reported based on actual capabilities, and the upper bound is limited by regulations. In addition, the implementation complexity should also be considered if ULFPTx mode for UL-MIMO is supported. With that, we have the following proposal:

Proposal 5: ULFPTx mode for UL-MIMO is NOT supported for ATG UE.

# 3.	Conclusion

In this paper, we provide our views on UE RF requirements in ATG enhancements. The following observations and proposals are made:

Proposal 1: BCS4 and BCS5 can be applied for ATG UE. Whether to specify BCS4 and BCS5 for DL CA_n79C is dependent on operator’s request. BCS4/BCS5 shall be requested together, but BCS5 can’t be reported together with BCS4.

Proposal 2: New capability is needed for UE antenna type which is per band combination. The signal is optional that means by default the same UE antenna type per band would apply for that band in a band combination.

Proposal 3: Not allow n39 UL for CA_n3A-n39A for ATG UE.

Proposal 4: To specify BCS0 for CA_n3A-n39A.

Proposal 5: ULFPTx mode for UL-MIMO is NOT supported for ATG UE.

# 4.	References

- R4-2410687, WF on Rel-19 ATG UE requirements, CMCC

- R4-2410641, Discussion and TP for TR 38.718-02-01 to introduce CA_n3A-n39A, Huawei

- TS 38.101-1 v18.6.0