3GPP TSG-RAN WG4 Meeting #112									                          R4- 2413264

Maastricht, The Netherlands, Aug. 19 – 23, 2024

Agenda item:			8.10.2.1

Source:	Ericsson

Title:	Discussion on RF requirement of ATG UE intra-band contiguous CA

Document for:	Approval

# Introduction

In 3GPP release 18, RAN4 has completed the standardization work for the air-to-ground (ATG) network WI, the main work focused on the FR1 single carrier scenario. To enhance the throughput to meet growing ATG capacity demand and fully utilize operators’ spectrum, the RAN plenary meeting #103 has approved a new WI on enhancements for the air-to-ground network for NR [1].

# Discussion

In the last meeting, some agreements were made addressing the general concerns and intra-band contiguous CA [2]. Part of the agreements regarding the intra-band contiguous CA are shown below:

  - intra-band contiguous CA

Issue 2-1-1: BCS for DL CA_n79C

Agreement:

- Reuse the existing BCS table for ATG

- Clarify that uplink CA is not supported for ATG UE elsewhere for Rel-19

- FFS on whether BCS#4 and #5 are not supported for ATG UE for Rel-19

Issue 2-1-2: whether/how to reflect operation band information in spec

Agreement:

- The information that only DL CA is supported in Rel-19 for ATG can be reflected in the clause of operating band for ATG

Issue 2-2: ATG Rx requirements applicability

Agreement:

- Following requirements needs to be updated for ATG CA compared with TN CA.

  - Max input level

  - ACS testing case 2

- Following requirements can reuse legacy TN CA requirements

  - Blocking including in-band and out of band blocking, narrow-band blocking

  - REFSENSE

  - Spurious response

  - Intermodulation

- Following requirement is not applicable for ATG CA

  - No CA Rx Spurious emission

  - No CA narrow-band blocking

- Following single carrier requirement of ATG is enough and no need to define ATG CA specific requirements

  - diversity characteristics requirement

Issue 2-3: max input level

Agreement:

Table 3: Maximum input level for ATG with CA_n79C

| Rx Parameter | Units | ATG UE Types |  |
| --- | --- | --- | --- |
|  |  | Omni-directional antenna | Antenna array |
| Power in largest transmission bandwidth configuration CC, Plargest BW | dBm | -422 | -302 |
|  |  | -443 | -323 |
| Power in each other CC | dBm | Plargest BW +10*log{(NRB,c*SCSc)/(NRB,largest BW*SCSlargest BW)} |  |
| The applicable NR CA Bandwidth Class | MHz | C |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 and 7.3F.2-3 for shared spectrum channel access operation with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. |  |  |  |

Issue 2-4: ACS testing parameters for case 2

Agreement:

- the principle of TN contiguous CA could be reused

  - Intra-band Contiguous CA

The following sessions will discuss the RF requirements for ATG UE intra-band contiguous CA.

### BCS for DL CA_n79C

The WF for CA_n79C was to reuse the existing BCS table for ATG UE, BCS 4 and 5 are specified for NR UE in the current spec. Furthermore, there isn’t any obvious technical reason to preclude BCS 4 and 5. To fully enable the bandwidth usage, the BCS 4 and 5 should be supported for ATG UE.

Table 1 Channel bandwidths for NR band n79

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n7910 | 15 |  |  | 10 |  | 20 |  | 30 |  | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  |  | 10 |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 704 | 80 | 90 | 100 |
|  | 60 |  |  | 10 |  | 20 |  | 30 |  | 40 |  | 50 | 60 | 704 | 80 | 90 | 100 |

Table 2: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA_n79C

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations or single uplink carrier5 | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n79C | CA_n79C3 | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |
|  |  | See n79 channel bandwidths in Table 5.3.5-1 for each carrier2 |  |  |  |  | 200 | 4 and 5 |

- To fully enable the bandwidth usage, BCS 0, 4, and 5 should be supported for CA_n79C

### ATG Rx requirements for intra-band CA

#### Adjacent channel selectivity

ACS test parameters for case 1 need to be specified, it could reuse the tables in 7.5A.1

ACS case 2 needs to be updated. Particularly Power in transmission bandwidth configuration, per CC needs to be scaled according to the ATG UE maximum input level. e.g.

- ACS case 2 needs to be updated. Particularly Power in transmission bandwidth configuration, per CC needs to be scaled according to the ATG UE maximum input level.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	To fully enable the bandwidth usage, BCS 0, 4, and 5 should be supported for CA_n79C

Proposal 2	ACS case 2 needs to be updated. Particularly Power in transmission bandwidth configuration, per CC needs to be scaled according to the ATG UE maximum input level.

# References

- RP-240804, New WID on Enhancements for Air-to-ground network for NR.

- R4-2410687, WF on Rel-19 ATG UE requirements