3GPP TSG-RAN WG4 Meeting #112									                          R4- 2413265

Maastricht, The Netherlands, Aug. 19 – 23, 2024

Agenda item:			8.10.2.2

Source:	Ericsson

Title:	Discussion on RF requirement of ATG UE inter-band CA

Document for:	Approval

# Introduction

In 3GPP release 18, RAN4 has completed the standardization work for the air-to-ground (ATG) network WI, the main work focused on the FR1 single carrier scenario. To enhance the throughput to meet growing ATG capacity demand and fully utilize operators’ spectrum, the RAN plenary meeting #103 has approved a new WI on enhancements for the air-to-ground network for NR [1].

# Discussion

In the last meeting, some agreements addressed the general concerns [2]. Part of the agreements regarding the inter-band are shown below:

  - inter-band CA

Issue 3-1: clarify the antenna type for each band for inter-band CA

Agreement:

- No limitation on antenna types for ATG CA

- FFS on whether to assume omni-antenna type can be assumed for both band n3 and n39 in DL CA_n3-n39

- FFS on whether new capability is needed for ATG CA

Issue 3-2-1: whether to allow n39 UL for CA_n3A-n39A

Agreement:

- FFS on whether to allow n39 UL for CA_n3A-n39A

Issue 3-3: BCS for DL CA_n3_n39

Agreement:

- FFS on BCS for DL CA_n3_n39

Issue 3-4-1: ATG Rx requirements applicability

Agreement:

- Following requirements needs further analysis:

  - delta RIB,

  - MSD,

  - Diversity requirements

    - FFS whether current ATG single carrier requirement is enough

- legacy inter-band CA requirements applies for following requirements while each CC shall meet the ATG UE requirement for single CC

  - REFSENSE

  - Max input level

  - ACS

  - in-band blocking

  - out of band blocking

    - FFS whether no OOB exception for CA_n3A-n39A

  - spurious response

  - wide band intermodulation

  - spurious emissions

- Following requirements are not applicable

  - Narrow band blocking

There remain numerous unresolved matters requiring attention, and the upcoming session will delve into the RF requirement for ATG UE inter-band CA.

  - Inter-band CA

### Antenna type for ATG UE supporting inter-band CA.

- FFS on whether to assume omni-antenna type can be assumed for both band n3 and n39 in DL CA_n3-n39

- FFS on whether new capability is needed for ATG CA

The operating band for ATG in Rel-18 includes n1, n3, n34, n39, n41, n78, n79 [3]. The example band combination in the WID is n3-n39, and this is mid band + mid band combination [3].

The choice of the ATG UE antenna type could vary based on the specific band combination. For the band combination like n3+n39, since both bands are low-frequency band, the omni-directional antenna could be used, which is also in line with the simulation study.

Regarding the new capability, it would be helpful to clarify its meaning. Does it imply that the UE signals the antenna type, and is this indication made per band?

- Omni-directional antenna type can be assumed for band n3 and n39 in DL CA_n3-n39.

### BCS for DL CA_n3_n39

The supported BCS for DL CA_n3-n39 should be BCS 0, 4, and 5.

- BCS 0, 4, and 5 should be supported for DL CA_n3-n39

### Whether to allow n39 UL for CA_n3A-n39A

The bands n3 and n39 are defined in the table below.

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n39 | 1880 MHz – 1920 MHz | 1880 MHz – 1920 MHz | TDD |

The CA_n3-n39 is a FDD and TDD inter-band combination, specifically, the upper bound of the n3 DL is next to the lower bound of the n39 without any gap in between. Therefore, the actual deployment scenarios/configuration would impact the potential interference. There was a discussion during the #110 meeting and a WF was reached as shown below [4]:

<Way forward 1>:

Single UL only in band n3 for DL CA_n3A-n39A is assumed.

Essentially, for the inter-band CA, PCell can operate either in FDD band or TDD band. To mitigate the potential interference, if only one UL transmission is assumed in band n3 for DL CA_n3-n39, then FDD band n3 could serve as the PCell and TDD band n39 serve as SCell while n39 is configured as DL to avoid interference with the n3 DL.

- FDD band n3 serves as PCell and TDD band n39 could serve as SCell while n39 is configured as DL to avoid interference with the n3 DL.

### Out-of-band blocking

FFS whether no OOB exception for CA_n3A-n39A

Exceptions to the requirement for inter-band CA are allowed when the second order intermodulation product of the lower frequency band UL carrier and the CW interfering signal fully or partially overlaps with the higher frequency DL carrier.

In the case of CA_n3A-n39A, the second order intermodulation products (IMD) would be 210 MHz, 95 MHz, 3590 MHz, and 3705 MHz which do not overlap with the higher frequency DL carrier. Therefore, the following was proposed:

- No OOB exception for CA_n3A-n39A

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Omni-directional antenna type can be assumed for band n3 and n39 in DL CA_n3-n39.

Proposal 2	BCS 0, 4, and 5 should be supported for DL CA_n3-n39

Proposal 3	FDD band n3 serves as PCell and TDD band n39 could serve as SCell while n39 is configured as DL to avoid interference with the n3 DL.

Proposal 4	No OOB exception for CA_n3A-n39A

# References

- RP-240804, New WID on Enhancements for Air-to-ground network for NR.

- R4-2410687, WF on Rel-19 ATG UE requirements

- TS 38.101-1, V18.5.0

- R4-2403719, WF on CA_n3A-n39A MSD and architecture.