3GPP TSG-RAN WG4 Meeting #112									                          R4- 2413266

Maastricht, The Netherlands, Aug. 19 – 23, 2024

Agenda item:			8.10.2.3

Source:	Ericsson

Title:	Discussion on RF requirement of ATG UE UL MIMO

Document for:	Approval

# Introduction

In 3GPP release 18, RAN4 has completed the standardization work for the air-to-ground (ATG) network WI, the main work focused on the FR1 single carrier scenario. To enhance the throughput to meet growing ATG capacity demand and fully utilize operators’ spectrum, the RAN plenary meeting #103 has approved a new WI on enhancements for the air-to-ground network for NR [1].

# Discussion

In the last meeting, some agreements were made addressing the general concerns [2]. Part of the agreements regarding the UL MIMO are shown below:

## UL MIMO

Issue 4-1-1: ULFPTx mode for UL-MIMO

Agreement:

- FFS on whether ULFPTx mode for UL-MIMO is supported for ATG UE

Issue 4-2-1: how to modify the NR UL MIMO requirement with ATG capability antennaArrayType-r18

Agreement:

- if the legacy requirement for UL MIMO is defined as sum of measurements of all UE transmit antenna connectors, then for ATG UE with capability antennaArrayType-r18, the requirement should be defined as sum of all TAB connector including both polarization.

- If the legacy requirement for UL MIMO is defined as per antenna port or per layer, FFS

Issue 4-2-2: maximum output power

Agreement:

- Reuse the rated maximum output power reported by the ATG UE capability maxOutputPowerATG-r18, and consider the existing NR UL-MIMO requirement with some adaption in requirement description specific to ATG UE.

  - Detailed modification of NR UL MIMO requirement with ATG capability antennaArrayType is based on conclusion of issue 4-2-1

Issue 4-2-3: configured transmitted power

Agreement

- Configured transmitted power for UL MIMO is for each UE

- Reuse ATG single carrier tolerance requirement

- modify the NR UL MIMO requirement with ATG capability antennaArrayType-r18, detailed based on issue 4-2-1

Issue 4-2-4: output power dynamic

Agreement:

- Legacy NR UL MIMO requirements is baseline

  - modify the NR UL MIMO requirement with ATG capability antennaArrayType-r18, detailed based on issue 4-2-1

Issue 4-2-5: transmit signal quality

Agreement:

- Legacy NR UL MIMO requirements is baseline

  - modify the NR UL MIMO requirement with ATG capability antennaArrayType-r18, detailed based on issue 4-2-1

Issue 4-2-6: whether to apply coherent UL MIMO requirement for ATG UE

Agreement:

- FFS on whether to apply coherent UL MIMO requirement for ATG UE

Issue 4-2-7: Output RF spectrum emissions

Agreement:

- Legacy NR UL MIMO requirements is baseline

- modify the NR UL MIMO requirement with ATG capability antennaArrayType-r18, detailed based on issue 4-2-1

Issue 4-3: receiver requirements

Agreement:

- FFS on whether to have Rx requirements for UL-MIMO for ATG UE

There are still many open issues that need to be addressed and the following session will discuss the RF requirement for ATG UE UL MIMO.

## RF requirements for ATG UE UL MIMO

### ULFPTx mode for UL-MIMO

For ATG UE UL-MIMO, in the case of one antenna port being configured (single layer UL MIMO operation) but with two Tx antenna connectors, the uplink full power transmission (ULFPTx) mode is applicable. From Rel-16 onwards, with the support of ULFPTx mode, one can transmit at full power either by combining two connectors using a coherent precoder or by supporting a full-power PA in full power mode. This is to ensure that UE can transmit at full power, i.e., if the UE is configured with only a single antenna port, it should always transmit at full power.

- ULFPTx mode should be supported for ATG UL-MIMO

### Output power dynamic, Transmit signal quality, and output RF spectrum emissions

As stated in the previous session, The ATG UE UL-MIMO is possible to be scheduled for a single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for codebook-based transmission with precoding matrix W=1. Therefore, from the completeness perspective, the single antenna requirement still needs to be specified for ATG UE UL MIMO.

It is suggested to re-use the existing NR UL-MIMO requirement for the following spec.

- Output power dynamic.

- Transmit signal quality.

- Output RF spectrum emissions.

- Reuse the existing NR UL-MIMO requirement for output power dynamic, transmit signal quality, and output RF spectrum emissions, and do not exclude the single antenna port requirement.

### Coherent UL MIMO requirement for ATG UE

FFS on whether to apply coherent UL MIMO requirement for ATG UE

UE capabilities have been defined for full coherence, partial coherence, and non-coherence transmission since Rel-15 NR. These modes correspond to where all transmit chains, partial transmit chains or none of the transmit chains have sufficiently well controlled relative phase for the codebook-based operation. A UE supporting fully coherent transmission can also support partial and non-coherent transmission.

As coherent UL MIMO is an optional feature for the UE, it is generally desirable to allow as much implementation freedom as possible. Therefore, coherent UL MIMO requirement should be specified for ATG UE UL MIMO as well.

- Apply the coherent UL MIMO requirement for ATG UE.

### Rx requirements for supporting UL-MIMO

Adopting a comparable approach in defining UL MIMO requirements for the TN UE, it is proposed that the following requirements could reuse the clauses defined for ATG UE.

- Reference sensitivity for UL MIMO

- Maximum input level for UL MIMO

- Adjacent channel selectivity for UL MIMO

- Blocking characteristics for UL MIMO

- Spurious response for UL MIMO

- Intermodulation characteristics for UL MIMO.

- Reuse the requirements defined in ATG UE for the following requirements: Reference sensitivity for UL MIMO, Maximum input level for UL MIMO, Adjacent channel selectivity for UL MIMO, Blocking characteristics for UL MIMO, Spurious response for UL MIMO, Intermodulation characteristics for UL MIMO.

# Conclusion

In the previous sections, we made the following observations:

Based on the discussion in the previous sections we propose the following:

Proposal 1	ULFPTx mode should be supported for ATG UL-MIMO

Proposal 2	Reuse the existing NR UL-MIMO requirement for output power dynamic, transmit signal quality, and output RF spectrum emissions, and do not exclude the single antenna port requirement.

Proposal 3	Apply the coherent UL MIMO requirement for ATG UE.

Proposal 4	Reuse the requirements defined in ATG UE for the following requirements: Reference sensitivity for UL MIMO, Maximum input level for UL MIMO, Adjacent channel selectivity for UL MIMO, Blocking characteristics for UL MIMO, Spurious response for UL MIMO, Intermodulation characteristics for UL MIMO.

# References

- RP-240804, New WID on Enhancements for Air-to-ground network for NR.

- R4-2410687, WF on Rel-19 ATG UE requirements

- TS. 38.822, NR UE feature list