3GPP TSG-RAN WG4 Meeting #112									                          R4-2413270

Maastricht, The Netherlands, Aug. 19 – 23, 2024

Agenda item:			8.6.2

Source:	Ericsson

Title:	Discussion on UE Rx chains of Fragmented Carriers

Document for:	Discussion

# Introduction

Fragmented carriers in the same band consuming multiple Rx chains may limit the UE CA capabilities. A new RAN4 study item was approved in the RAN #104 meeting with the aim of investigating solutions to address this.

The objectives of the SI are as follows [1]:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

NOTE 1:	No RAN1 impact is foreseen

NOTE 2:	RAN2 work, if necessary, will be triggered by RAN4 LS

NOTE 3:	This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers.

- Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

The following sections will discuss some issues of the number of UE Rx chains of fragmented carriers.

# Discussions

## Scenarios for fragmented spectrum study

When studying fragmented spectrum scenarios, it is crucial to consider various deployment situation and their impact on RF requirements. In the inter-operator co-located scenario of the SID, co-location between wanted carriers is assumed, and the collaborating operator/s has full control over power levels and scheduling across all carrier components. The operators may or may not operate using a shared RAN and accept UE Rx degradation due to a non-collocated operator in the gap.

However, a relaxation to the requirements may not be acceptable for all deployments of non-contiguous intra-band carrier aggregation (CA) when a non-collocated operator is present in the gap. Hence supported band combinations with a reduced number of Rx chains and reduced selectivity performance would have to be advertised with modified UE band-combination (BC) capability. Unless this is restricted to complex BC, there would be nothing to prevent a UE vendor from advertising BCs with reduced interference rejection capability even for band combinations where the number of Rx chains is not a problem, which implies degraded selectivity in general as support of any BC is always optional.

We observe that

- Supported band combinations with a reduced number of Rx chains, and consequently reduced selectivity performance, would have to be advertised with modified UE band-combination capabilities. There is nothing to prevent a UE vendor from advertising BCs with reduced interference rejection capability even for band combinations where the number of Rx chains is not a problem, which implies degraded selectivity in general as support of any BC is always optional.

Reduced performance is normally not advertised as a “capability”. The UE can still implement collocated non-contiguous carriers with a single Rx chain as long as the existing requirements are met, the UE implementation is not declared in the tests and should not be.

For UEs supporting non-collocated carriers separate UE receive chains are assumed. The UE receiver requirements are primarily driven by scenarios where there is a non-coordinated or non-co-located operation in these gaps.

## Co-located conditions in practical deployment

The co-location definition refers to a scenario where two or more gNB are physically located at the same site or in very close proximity to each other. The gNB is a logical node typically implemented within a BS deployed at a site [2]. This collocation deployment scenario should meet the conditions outlined in MRTD as specified in TS38.133 clause 7.6.4 and the power spectra density imbalance specified in TS38.101 [3][4].

Table 7.6.4-1: Maximum receive timing difference requirement for intra-band non-contiguous NR carrier aggregation

| Frequency Range | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 31 |
| FR2-1 | 0.26 |
| Note 1: In the case of different SCS on different CCs, if the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot. |  |

The current power imbalance requirement for the intra-band CA is shown below:

NOTE 6: For a UE not indicating intraBandNR-CA-non-collocated-r18, the minimum requirements for intra-band non-contiguous CA apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. For a UE indicating intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided and UE is configured with maxMIMO-Layers with value less than or equal to 2, the power imbalance requirement defined in subclause 7.10A apply. For a UE indicating intraBandNR-CA-non-collocated-r18 and when nonCollocatedTypeNR-CA-r18 is provided, the minimum requirements for intra-band non-contiguous CA apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. For these UEs, the power spectral density imbalance condition also applies for these carriers when applicable intra-band non-contiguous NR CA configuration is a subset of a higher order NR CA configuration.

It is important to note that the 6 dB PSD imbalance for intra-band collocated CA is only the minimum core requirement that can be accessed during the conformance testing. However, in real-world deployment, a 6 dB power imbalance between two non-contiguous CCs cannot always be guaranteed in real deployment, it is not possible for the network(s) to maintain a 6 dB PSD imbalance in the field at all times.

- The collocation deployment scenario should meet the conditions outlined in MRTD as specified in TS38.133 clause 7.6.4 and the power spectra density imbalance specified in TS38.101.

# Conclusion

In the previous sections, we made the following observations:

Observation 1	Supported band combinations with a reduced number of Rx chains, and consequently reduced selectivity performance, would have to be advertised with modified UE band-combination capabilities. There is nothing to prevent a UE vendor from advertising BCs with reduced interference rejection capability even for band combinations where the number of Rx chains is not a problem, which implies degraded selectivity in general as support of any BC is always optional.

Observation 2	The collocation deployment scenario should meet the conditions outlined in MRTD as specified in TS38.133 clause 7.6.4 and the power spectra density imbalance specified in TS38.101.

# References

- RP-241669, Study on NR FR1 DL Fragmented Carriers

- RP-180557, LS response on work to support IMT-2020/5G in the Transport Network

- TS. 38.133 v18.5.0

- TS. 38.101-1 v18.5.0

- RP-240503, Fragmented carriers in the DL