3GPP TSG-RAN WG4 #112			R4-2413272

Maastricht, Netherlands, August 19th – 23rd, 2024

Source:	Keysight Technologies

Title:	On Rel-19 spatial channel model for demodulation performance requirements

Agenda item:	8.14.2

Document for:	Discussion

# 1. Introduction

In RAN#104 a new study item on spatial channel model for demodulation performance requirements was approved ([1]).

The main goal of this study item is to study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements for both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1:

- Including the limitations of the channel models used up to Rel-18 regarding UE MIMO performance

- Considering both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches (using the tuned repeatable spatial channel model of TR38.827 as the starting point for CDL approach).

- Evaluating test methodology feasibility and achievable results uncertainty while test complexity is not significantly increased.

This document provides evaluation on limitations of the channel models used up to Rel-18 for multiple UE MIMO demodulation performance scenarios and how these limitations could be overcome with CDL-based channel modelling. It also provides initial analysis on feasibility of CDL-based channel models.

# 2. Channel models used up to Rel-18 for demodulation requirements and their limitations

Doing a retrospective on the work that 3GPP has carried out in terms of the channel model for 5G technology and focusing on UE demodulation requirements, it could be seen that:

- In Rel-14, two study items were progressed carried out: the first one analyzing how to define a channel model representing the channel for frequencies up to 100 GHz [2], whose outcome was documented in 3GPP TR 38.900 [3]), and the second one aiming to develop an NR access technology to meet a broad range of use cases including enhanced mobile broadband, massive MTC, critical MTC [4], including the channel model definition that was finally documented in TR 38.901 [5]. 3GPP TR 38.901 included a general framework including Tapped Delay Line models (TDL) and Clustered Delay Line models (CDL).

- In Rel-15, when demodulation performance requirements were defined as part of the New Radio Access Technology Work item [6], TDL channel models were chosen for both FR1 and FR2, as shown in Annex B.2.1 in 3GPP TS 38.101-4 [7].

- From Rel-16 towards Rel-18, new combinations of channel model parameters have been added to 3GPP TS 38.101-4 [7] but always using TDL channel models.

Latest version of 3GPP TS 38.101-4 [7] includes the following combinations of channel model parameters:

Observation 1: For UE demodulation performance requirements up to Rel-18, channel models used are based on Tapped Delay Line model.

However, as described in R4-2402277 [10], RAN4 demodulation session is finding more often in the situation where non-spatial channel models neither show the performance observed in RAN1 nor the one expected in actual deployments. To be noticed here that RAN1 is using CDL models for running link-level simulations while RAN4 keeps using TDL channel models to define demodulation requirements (refer to R4-2402277 for further details on this).

Observation 2: Tapped Delay Line models don’t show the performance observed in RAN1 nor the one expected in actual deployments.

# 3. Alternative to channel models used for demodulation requirements

The alternative to TDL channel models used within 3GPP TR 38.901 [5] framework is CDL channel models.

R4-2402277 [10] already referred in section 2.2 the efforts 3GPP RAN4 made to try to achieve alignment on RAN4 performance requirements with spatial channel modelling without success during Rel-15 as the framework defined in 3GPP TR 38.901 included lots of random (but performance impacting) starting values, that made results alignment between companies too challenging within Rel-15 timeframe. Hence time pressure made TDL channel model become the working assumption for UE demodulation performance requirements at that time.

CDL channel model result alignment was finally achieved during the Rel-16 study item on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR UEs [11], which used the channel models defined in 3GPP TR 38.901 [5] as well as the associated aspects related to channel modelling in 3GPP TR 38.810 [12] as the basis of the emulated propagation environment to finally converge in the channel models defined in clause 7 in 3GPP TR 38.827 [8], which are being used to defined MIMO OTA requirements defined in 3GPP TS 38.151 [13]:

Observation 3: An alternative to Tapped Delay Line models used in UE demodulation performance requirements is Cluster Delay Line models, already in use in MIMO OTA requirements defined in 3GPP TS 38.151.

# 4. Differences between CDL and TDL channel models impacting UE MIMO demodulation performance

In an attempt to justify why non-spatial channel models used up to Rel-18 for UE demodulation requirements don`t show the expected performance in actual deployments, some key differences between TDL and CDL channel models that could make a difference when exercising UE MIMO demodulation performance have been analysed:

- Phase difference between channel coefficients on adjacent BS antennas on a single cluster/tap:

Figure 1 shows the phase difference between channel coefficients on adjacent BS antennas (half wavelength spacing) on a single cluster/tap.

Figure 1. Phase difference between channel coefficients on adjacent BS antennas (half wavelength spacing) on a single cluster/tap

From Figure 1, it can be observed that in the TDL model the phase varies widely between ±180 deg as expected while in the CDL model the phase is more stable as the cluster centre is in a specific angle causing an average phase offset between the antennas. With the CDL, the largest deviation of the phase difference from the mean value occurs when either or both channel coefficient are in a deep fade (not visible in this figure). The blue TDL curve looks rather random, but it is not, since there is temporal correlation between successive samples, though it is difficult to read from this figure.

Observation 4: Phase difference between channel coefficients on adjacent BS antennas in CDL channel models is more realistic than in the case of TDL models.

- Temporal correlation function for a single cluster/tap :

The temporal correlation function for a single cluster/tap case is shown in Figure 2.

Figure 2. Temporal correlation function for a single cluster/tap case

It shows that the correlation decreases more rapidly with the TDL model than with the CDL one. This is caused by the difference in Doppler spreads. With the TDL model each tap has Jakes/Clarke Doppler power spectrum while, with the CDL, a single cluster has a more limited set of Doppler frequency components and hence smaller Doppler spread.

Observation 5: Temporal correlation function of a CDL model is more realistic than the one of TDL model.

- Angular power distribution:

Figure 3 illustrates the angular power distribution comparison between CDL and TDL models. This was estimated by sweeping the steering angle of X X X X antenna array from -90 to +90 degrees of azimuth (Bartlett beamforming). There are 1000 curves for each sub-figure indicating 1000 time samples of generated MIMO channels. Vertical axes are squared magnitudes in linear units (not dB). It can be seen that with the CDL model there are clearly certain angles where the power is concentrated. With the TDL model the power distribution is close to uniform.

Figure 3. Angular power distribution

Observation 6: With a TDL model there are no specific angles of arrival or departure in the channel. Moreover, the phasing in antenna arrays (indicates angles) varies wildly over time in a TDL model. This phenomenon does impact both single and multi-user cases.

# 5. Feasibility, achievable results and test complexity of CDL channel models

As described in section 3 in this discussion paper, CDL channel models are already used in 5G NR MIMO OTA requirements [13], which demonstrates that they are feasible including acceptable results uncertainty while associated test system complexity is not significantly increased.

Observation 7: CDL channel models are feasible including acceptable results uncertainty while associated test system complexity is not significantly increased.

# 6. Conclusion

This document provides evaluation on limitations of the channel models used up to Rel-18 for multiple UE MIMO demodulation performance scenarios and how these limitations could be overcome with CDL-based channel modelling. It also provides initial analysis on feasibility of CDL-based channel models.

The following observations were made:

Observation 1: For UE demodulation performance requirements up to Rel-18, channel models used are based on Tapped Delay Line model.

Observation 2: Tapped Delay Line models don’t show the performance observed in RAN1 nor the one expected in actual deployments.

Observation 3: An alternative to Tapped Delay Line models used in UE demodulation performance requirements is Cluster Delay Line models, already in use in MIMO OTA requirements defined in 3GPP TS 38.151.

Observation 4: Phase difference between channel coefficients on adjacent BS antennas in CDL channel models is more realistic than in the case of TDL models.

Observation 5: Temporal correlation function of a CDL model is more realistic than the one of TDL model.

Observation 6: With a TDL model there are no specific angles of arrival or departure in the channel. Moreover, the phasing in antenna arrays (indicates angles) varies wildly over time in a TDL model. This phenomenon does impact both single and multi-user cases.

Observation 7: CDL channel models are feasible including acceptable results uncertainty while associated test system complexity is not significantly increased.

References

- RP-241610, “New SID: Study on spatial channel model for demodulation performance requirements”, Nokia, RAN#104, June 2024.

- RP-160210, “SID: Study on channel model for frequency spectrum above 6 GHz”, RAN#71, Samsung, Nokia, March 2016

- 3GPP TR 38.900, “Study on channel model for frequency spectrum above 6 GHz” (Rel-14)

- RP-170379, “SID: Study on New Radio Access Technology”, NTT DOCOMO INC., RAN#75, March 2017

- 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz” (Rel-14)

- RP-190213, “WID on New Radio Access Technology”, NTT DOCOMO INC., RAN#83, March 2019

- 3GPP TS 38.101-4, “NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements”

- 3GPP TR 38.827, “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”, Rel-16

- 3GPP TS 38.151, “NR; User Equipment (UE) Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements”, Rel-17

- R4-2402277, “Discussion on 8Rx general demodulation aspects and spatial channel models”, Nokia, Nokia Shanghai Bell, BT, RAN4#110, February 2024

- RP-201069, “SID: Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR UEs”

- 3GPP TR 38.810, “NR; Study on test methods”

- 3GPP 38.151, “NR; User Equipment (UE) Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements”