3GPP RAN WG4 Meeting #112		R4-2413273

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	Huawei, HiSilicon

Title: 	Redcap UE capability for enhanced channel raster

Agenda Item:	5.3

Document for:	Approval

1 Introduction

In this contribution, we provide our view on the remaining issue of UE capability for enhanced channel raster.

2 Discussion

As discussed in last meeting the following shows the agreement made in R4-2406680 at RAN4#110bis and no update was made at RAN4#111 in R4-2410747.

It is noted that the only remaining issue is FFS for (e)RedCap. At RAN4#110 meeting interested operators provided inputs on the request of mandatory bands for enhanced channel raster [2-4]. And Redcap is the major justification for the mandatory feature proposal. The following agreement was made and in the feature list it was agreed as “Mandatory with capability signaling for all Rel-18 UEs for certain bands as defined in 38.101-1 and 38.101-5”. In our view here “all Rel-18 UEs” may be interpreted as it applies to both normal UEs and Rel-18 Redcap UEs.

At RAN4#110bis meeting, the following WF was agreed

For Rel-18, at least support the enhanced channel raster as mandatory feature for (e)Redcap UEs in the same set of NR operating bands for eMBB UEs. FFS on other bands.

Change RedCap to (e)RedCap in the “mandatory/optional” column.

And for early release, there are a large number of Rel-17 commercial Redcap UEs in the field sine Rel-17 are frozen in 2022. Hence it is not possible to assume this new feature can be mandate for the Rel-17 commercial Redcap UEs. Hence it is unfortunately that it has to be an optional feature for Rel-17. Hence we propose that the channel raster enhancement feature is optional for Rel-17 RedCap UE, .i.e. the same as normal UEs. Hence one of the proposals is to define the same UE capability framework as normal UEs.

Option 1: it is proposed to remove FFS for RedCap UE in the RAN4 feature list, i.e. the same UE capability framework as MBB UEs is applicable for Redcap UEs.

The odd/even number of PRB is the major issue for Redcap. The maximum channel bandwidth for Redcap UE is 20 MHz which corresponds to 106 PRB. If the system bandwidth is 25 MHz with the 133 PRBs transmission bandwidth configuration. One of them can not be on the 100 kHz channel raster. As discussed above it can only be optional for release Rel-17. Meanwhile for Rel-18, it comes feasible that the enhanced channel raster is mandatory for RedCap UE from Rel-18.

Option 2: the enhanced channel raster is mandatory for RedCap UE from Rel-18 and it is optional for Rel-17.

Proposal: it is proposed to further discuss the two options and make a decision at RAN4#112

Option 1: it is proposed to remove FFS for RedCap UE in the RAN4 feature list, i.e. the same UE capability framework as MBB UEs is applicable for Redcap UEs.

Option 2: the enhanced channel raster is mandatory for RedCap UE from Rel-18 and it is optional for Rel-17.

3 Conclusion

Proposal: it is proposed to further discuss the two options and make a decision at RAN4#112

Option 1: it is proposed to remove FFS for RedCap UE in the RAN4 feature list, i.e. the same UE capability framework as MBB UEs is applicable for Redcap UEs.

Option 2: the enhanced channel raster is mandatory for RedCap UE from Rel-18 and it is optional for Rel-17.

References

[1] R4-2400647,	Frequency bands with mandatory support of the new channel raster, Orange UK

[2] R4-2400723,	Mandatory enhanced raster for NR bands n2, n5, and n66, AT&T

[3] R4-2402472,	Request for mandatory enhanced raster for n25, n66, n71 and n85, T-Mobile USA

[4] R4-2406680, RAN4 Rel-18 feature list, CMCC

[5] R4-2410747, Rel-18 RAN4 feature list, CMCC