3GPP TSG-RAN WG4 Meeting #112 	R4-2413276

Maastricht, Netherlands, Aug 19 – 23, 2024

Source: 	Huawei, HiSilicon

Title: 	Expected EIRP conformance – Test Vectors, MU budget and Conformance test method

Agenda Item:	8.11.4

Document for:	Approval

1 Introduction

In [1] we discussed the statistical nature of the EERIP conditions and how these may be translated into a fixed test vector which can reasonably used for conformance testing.

In this paper we expand upon this idea with some simulation results for various antennas and test vectors.

1 Conformance test - Test vectors

1.1	Beam steering profiles

The EEIRP requirement states that the beam is pointed in different directions over its beam steering range and that the directions have a uniform statistical angular distribution over this range (as opposed to a uniform UE location on the ground). During the regulatory analysis a number of different sub-sets of beam steering directions were proposed and some initial analysis was done to see if they were suitable but nothing was agreed. As the description of the EEIRP requirement incudes a statistical factor (the direction of the beam(s)) and an averaging when looking at potential fixed test vectors it is important we get a reasonable method to judge their accuracy.

During conformance testing it is important that the test results are repeatable, as such a random test procedure is not desirable, particularly if a very large number of cycles are required to achieve any level of repeatability. Defining a fixed test vector which is both repeatable and representative is something that can be done by simulation so that the conformance testing of BS can be simplified.

    - Antenna Arrays

In order to quantify the performance of any method it is necessary to ensure it is suitable for a range of arrays, for this purpose we have highlighted 4 different arrays with varying vertical beams widths and sub-array construction as well as varying azimuth beam widths.

Table: 1.1.1-1 Test antennas descriptions

|  |  | Antenna |  |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 4 | 5 |  |
| elevation | No elements/sub-arrays | 16 | 8 | 8 | 16 |  |
|  | elements per sub-array | 1 | 2 | 3 | 2 |  |
|  | element separation | 0.7 | 0.7 | 0.7 | 0.7 | λ |
| Azimuth | No elements | 8 | 8 | 16 | 16 |  |
|  | element separation | 0.5 | 0.5 | 0.5 | 0.5 | λ |
| Directivity (zero steer) |  | 27.7 | 27.7 | 32.4 | 33.9 | dBi |
| vertical BW |  | 4.5 | 4.5 | 3 | 2.3 | deg |
| horizontal BW |  | 12.7 | 12.7 | 6.4 | 6.4 | deg |

Antenna 2 is a sub-array impanation of the Antenna 1 and Antenna 4 is a sub-array implementation. Antenna 5 has a very narrow vertical beam width and uses small sub arrays. We believe that this covers a range of implementations which are likely in the 6-7GHz band.

The steering range of the beams also need to be considered, as we have included sub-array models which have limited steering performance the analysis was done over a suitable range of the worst performing antenna. The same steering range was used for all tests so antennas could be compared directly. The vertical steering range as +/- 5deg (based on ANT2 capability.

1.1.2	Reference pattern

In order to access if a sub-set of beam steering directions (or test vectors) are suitable we must first know what the ideal or correct result is? The requirement describes a statistical average based on random beam locations as such the “correct” result, this can never be described 100% as beam directions are random and as no upper limit is defined essentially there are potentially an infinite number of them. As close as can be achieved we must estimate the “correct” answer by averaging a very large number of samples until the error function is minimised.

An alternative method to achieve a reasonable approximation at a reference result is a fixed distribution of many beam directions distributed evenly over the steering range. One such profile was suggested in [2]. Whilst there are some small differences in results between this set of test vectors and a set of 10k random directions they are small and the advantage of having fixed profile as a reference which can be simulated in a reasonable time is useful, particularly when potentially comparing results between different companies.

- Table 1.1.2-1: Difference between fixed 325 direction test vector and 10k random samples.

| Test vector type | Beam width |  | Elevation Angle range (deg) |  |  |  |  |  |  | average/abs | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | az | el | 60-90 | 30-60 | 20-30 | 15-20 | 10-15 | 5-10 | 0-5 |  |  |
| ANT1 | 12.6 | 4.5 | -0.18 | 0.19 | 0.29 | 0.36 | 0.22 | 0.21 | 0.19 | 0.23 | dB |
| ANT2 | 12.6 | 4.5 | -0.17 | -0.09 | 0.31 | 0.26 | -0.15 | -0.02 | -0.27 | 0.18 | dB |
| ANT4 | 6.3 | 3.0 | -0.13 | 0.25 | -0.08 | -0.08 | -0.11 | -0.15 | 0.56 | 0.19 | dB |
| ANT5 | 6.3 | 2.3 | -0.54 | 0.22 | 0.29 | 0.34 | -0.07 | -0.28 | 0.12 | 0.27 | dB |

A 0.5deg grid was used for this analysis due to the narrow beam width of ANT5, generally we use a 1deg grid. Simulating a full 3d grid with 0.5deg step size over 10k directions takes a very long time as such is not ideal for even a simulation task, however it can be seen the difference between this and the fixed 325 point pattern are not great (0.25dB average error).

Observation1: We have used 10k random directions to form a stable reference spatial power distribution pattern as this most closely matched the requirement definition.

1.1.3	Test vectors

For conformance testing it is important that the number of directions tested is not to large so that the test time is not excessive as such targeting a small number of directions is preferable.

As such we have focused on a number of simple test vector patterns 3x3, 4x4 and 5x5 as well as some test patterns previously suggested in some papers, as shown in figure 1.1.3-1

Figure 1.1.3-1: Test vectors weights are equal unless stated otherwise.

The results are presented as the error between the reference condition and the results from the test vector. The intention of the exercise is not to identify compliance against the requirement but to identify how accurately the test vector simulates the reference condition.

Table 1.1.3-1: Test vector Errors (10k samples, Mech=8°, elect tilt = +/-5°, Az = +/-60° )

| ANT1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test vector type | Elevation Angle range (deg) |  |  |  |  |  |  | rms | Unit |
|  | 60-90 | 30-60 | 20-30 | 15-20 | 10-15 | 5-10 | 0-5 |  |  |
| typeA 3x3 | -0.33 | -0.17 | 0.12 | 0.17 | 0.25 | 0.38 | 1.15 | 0.50 | dB |
| typeA 3x4 | 0.25 | -0.11 | 0.07 | 0.16 | 0.25 | 0.41 | 1.51 | 0.61 | dB |
| typeA 4x4 | 0.23 | -0.10 | 0.06 | 0.13 | 0.20 | 0.33 | 1.26 | 0.51 | dB |
| typeA 5x5 | 0.00 | -0.07 | 0.05 | 0.11 | 0.18 | 0.26 | 1.10 | 0.43 | dB |
| 5x5(EB) | 0.78 | 0.77 | 0.26 | 0.12 | 0.06 | -0.19 | -1.20 | 0.63 | dB |
| HW9 | -0.05 | 0.36 | -0.13 | 0.01 | 0.31 | 0.57 | 1.38 | 0.60 | dB |
| E9 | -3.35 | -0.92 | -0.15 | -0.16 | -0.13 | 0.32 | 1.15 | 1.39 | dB |
| N6 | -1.55 | -0.04 | 1.05 | 0.58 | 0.33 | 0.56 | 1.28 | 0.92 | dB |
| ANT2 |  |  |  |  |  |  |  |  |  |
| Test vector type | Elevation Angle range (deg) |  |  |  |  |  |  | rms | Unit |
|  | 60-90 | 30-60 | 20-30 | 15-20 | 10-15 | 5-10 | 0-5 |  |  |
| typeA 3x3 | -0.12 | 0.18 | 1.16 | 0.15 | 0.17 | 0.30 | 1.01 | 0.60 | dB |
| typeA 3x4 | 0.49 | 0.15 | 1.08 | 0.13 | 0.17 | 0.32 | 1.36 | 0.70 | dB |
| typeA 4x4 | 0.35 | 0.13 | 0.85 | 0.12 | 0.14 | 0.25 | 1.12 | 0.56 | dB |
| typeA 5x5 | 0.09 | 0.16 | 0.72 | 0.10 | 0.12 | 0.19 | 0.97 | 0.47 | dB |
| 5x5(EB) | 0.55 | 0.02 | -0.81 | -0.11 | -0.09 | -0.27 | -1.30 | 0.63 | dB |
| HW9 | 0.05 | 2.42 | 1.27 | 0.11 | 0.23 | 0.45 | 1.22 | 1.15 | dB |
| E9 | -3.22 | 0.91 | 1.02 | 0.09 | -0.05 | 0.31 | 1.06 | 1.39 | dB |
| N6 | -1.42 | 0.77 | 1.51 | 0.41 | 0.14 | 0.38 | 1.10 | 0.96 | dB |
| ANT4 |  |  |  |  |  |  |  |  |  |
| Test vector type | Elevation Angle range (deg) |  |  |  |  |  |  | rms | Unit |
|  | 60-90 | 30-60 | 20-30 | 15-20 | 10-15 | 5-10 | 0-5 |  |  |
| typeA 3x3 | -0.07 | 0.67 | 1.32 | -0.02 | 2.57 | 0.86 | -0.03 | 1.17 | dB |
| typeA 3x4 | 1.12 | 0.49 | 1.09 | -0.10 | 2.48 | 0.76 | -0.23 | 1.16 | dB |
| typeA 4x4 | 1.05 | 0.52 | 0.92 | 0.06 | 1.74 | 0.34 | 0.34 | 0.89 | dB |
| typeA 5x5 | -1.91 | 0.39 | 0.72 | 0.06 | 1.47 | 0.27 | 0.51 | 0.99 | dB |
| 5x5(EB) | -0.39 | 0.08 | -1.23 | -0.90 | -1.62 | -0.48 | -0.45 | 0.89 | dB |
| HW9 | 0.07 | 1.50 | -1.23 | 3.25 | 3.00 | 0.96 | 0.48 | 1.87 | dB |
| E9 | -3.82 | -0.54 | -1.28 | 2.36 | 2.03 | 0.64 | 0.52 | 1.96 | dB |
| N6 | -2.81 | 1.10 | -0.40 | -0.78 | 1.88 | 0.03 | -0.09 | 1.38 | dB |
| ANT5 |  |  |  |  |  |  |  |  |  |
| Test vector type | Elevation Angle range (deg) |  |  |  |  |  |  | rms | Unit |
|  | 60-90 | 30-60 | 20-30 | 15-20 | 10-15 | 5-10 | 0-5 |  |  |
| typeA 3x3 | -0.39 | 0.68 | 0.92 | 0.09 | 0.19 | 0.26 | 1.13 | 0.64 | dB |
| typeA 3x4 | 0.40 | 0.51 | 0.88 | 0.10 | 0.09 | 0.20 | 0.79 | 0.52 | dB |
| typeA 4x4 | 0.24 | 0.36 | 1.09 | -0.34 | -0.23 | -0.04 | 0.42 | 0.50 | dB |
| typeA 5x5 | 0.00 | 0.20 | 0.88 | -0.01 | 0.03 | 0.00 | 0.45 | 0.38 | dB |
| 5x5(EB) | 1.09 | -0.15 | -1.05 | 0.16 | 0.22 | -0.16 | -0.58 | 0.63 | dB |
| HW9 | 0.61 | 3.91 | 1.09 | 0.56 | 0.68 | 0.61 | 1.01 | 1.65 | dB |
| E9 | -2.96 | 1.79 | 0.40 | 0.38 | 0.09 | 0.52 | 0.92 | 1.38 | dB |
| N6 | -1.29 | 1.94 | 0.93 | 0.51 | 0.10 | 0.29 | 0.40 | 0.99 | dB |

The rms errors are then collated across the antenna types to give a single performance figure in table 1.1.3-2.

Table 1.1.3-2: Test vector Errors summary

| Test vector type | ANT1 | ANT2 | ANT4 | ANT5 | RMS AVG | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| 3x3 | 0.50 | 0.60 | 1.17 | 0.64 | 0.72 | dB |
| 3x4 | 0.61 | 0.70 | 1.16 | 0.52 | 0.74 | dB |
| 4x4 | 0.51 | 0.56 | 0.89 | 0.50 | 0.61 | dB |
| 5x5 | 0.43 | 0.47 | 0.99 | 0.38 | 0.56 | dB |
| 5x5(EB) | 0.63 | 0.63 | 0.89 | 0.63 | 0.69 | dB |
| HW9 | 0.60 | 1.15 | 1.87 | 1.65 | 1.31 | dB |
| E9 | 1.39 | 1.39 | 1.96 | 1.38 | 1.53 | dB |
| N6 | 0.92 | 0.96 | 1.38 | 0.99 | 1.06 | dB |

When considering how to select an appropriate test vector we need to trade off potential error (between the ideal power distribution as defined in the requirement and the power distribution of the test vector) against text time and complexity.

The lowest errors appear to be associated with the simple non-weighted test vectors (3x3, 4x4, 5x5 etc). These also follow a pretty clear pattern in that the error reduced as the number of test points increases.

To estimate what an acceptable error maybe we can look at the measurement uncertainty analysis, the TRP requirement has an error figure associated with the grid step size. The value of this error is also based on a trade between measurement points. As such we can use this as somewhat of a precedent and set the acceptable error at a similar level. As such an error of 0.75dB is acceptable.

The 3x3 test vector has the fewest number of points which passes this threshold hence offers the best balance between accuracy and test time.

Observation 2: Test Vector 3x3 offers the best balance between accuracy and test time.

    - Further Work

It should be noted that all of the analysis has been based on beam peak directions sets which are defined as rectangular.

The intention of the peak direction set declarations was always to not restrict the nature of the coverage area as can be seen by the declarations definitions for example:

1)	The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction.

Also the examples given in TR 38.842 are deliberately not rectangular to highlight this

Figure F-4: Example of a directions diagram indicating possible settings
for the beam pointing direction

Observation 3: The OTA peak directions set is not necessarily rectangular

If a beam peak directions set is based on and EIRP contour then it is more likely that the peak directions set will be more elliptical. Analysis of beam steering directions in a cell are shown in figure 1.1.4-1 and it can be seen that the elliptical distribution is also the more probable in a functioning cell.

Figure 1.1.4-1: Simulated beam directions distribution patterns in a cell

Observation 4: A rectangular OTA peak direction set does not represent likely BS usage.

The EEIRP requirement is specified over the BS beam steering range which is effectively the OTA peak directions set, as such the test vectors should be suitable for any OTA peak directions set not just a rectangular distribution.

Proposal 1: The Test vector should also consider for any OTA peak directions set not just rectangular

2	MU budget and grid step size

2.1	Test procedure

We have proposed a test methodology in section 3 which has many similarities to the existing TR procedure in a anechoic chamber. In both cases emission are measured on a measurement grid of points with a calibrated path between the DUT and the measurement antenna. The DUT is rotated on a positioner in order to change the measurement direction.

Traditionally BS spatial requirements have been referenced to the BS enclosure as such the measurement grid was referenced to the BS enclosure. In a test chamber this is probably the most sensible approach. As EEIRP is referenced to the horizon the declared electrical tilt needs to be considered during test. However this can be accommodated by either

- Offset the measurement grid points by the required angle

- Use the normal measurement grid and mathematically “tilt” the results after the measurement.

Neither of these should impact the measurement uncertainty.

The EERIP MU budget therefore will be very similar to the TRP MU budget – however there are a few points to consider:

- The EEIRP is not measuring the main lobe but the side lobes, these will have different spatial characteristics (lobe width for example) and be of a much lower amplitude than the main lobe

  - As this main lobe is present in the chamber then the chamber reflection level and isolation will be important

  - Measurement accuracy of the power measurement equipment may be lower as power levels are lower.

  - As side lobes may have different lobe width to the main lobe the measurement step size recommendations and accuracy impact may need to be reconsidered.

In addition the latest version of 38.141-2 has, for most requirements, had the upper frequency range extended form 6GHz to 7.125 GHz which covers the n104 band to which EEIRP requirement applies. The MU budget in TR 37.941 which shows the calculations of which the final MU in the test requirement are based however has not been updated and covers only up to 6GHz, as the MU for 7.125GHz is the same as that for 6GHz we can assume that the upper range has just been extended in these calculations as well (this should be updated at some point).

  - MU budgets

    - TRP measurement uncertainty

The TRP budget for the Indoor Anechoic Chamber (IAC) and the CATR (Compact Antenna Test Range) are given n TR 37.841, bot:

| Table 11.2.2.3-1: IAC MU value derivation for EIRP measurement of OTA BS output power |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |
|  |  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤6 GHz |  |  |  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤6 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |
| A1-1 | Positioning misalignment between the BS and the reference antenna | 0.03 | 0.03 | 0.03 | Rectangular | 1.73 | 1 | 0.02 | 0.02 | 0.02 |
| A1-2 | Pointing misalignment between the BS and the receiving antenna | 0.3 | 0.3 | 0.3 | Rectangular | 1.73 | 1 | 0.17 | 0.17 | 0.17 |
| A1-3 | Quality of quiet zone | 0.1 | 0.1 | 0.1 | Gaussian | 1.00 | 1 | 0.10 | 0.10 | 0.10 |
| A1-4a | Polarization mismatch between the BS and the receiving antenna | 0.01 | 0.01 | 0.01 | Rectangular | 1.73 | 1 | 0.01 | 0.01 | 0.01 |
| A1-5a | Mutual coupling between the BS and the receiving antenna | 0 | 0 | 0 | Rectangular | 1.73 | 1 | 0.00 | 0.00 | 0.00 |
| A1-6 | Phase curvature | 0.05 | 0.05 | 0.05 | Gaussian | 1.00 | 1 | 0.05 | 0.05 | 0.05 |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | 0.26 | Gaussian | 1.00 | 1 | 0.14 | 0.26 | 0.26 |
| A1-7 | Impedance mismatch in the receiving chain | 0.14 | 0.33 | 0.33 | U-shaped | 1.41 | 1 | 0.10 | 0.23 | 0.23 |
| A1-8 | Random uncertainty | 0.1 | 0.1 | 0.1 | Rectangular | 1.73 | 1 | 0.06 | 0.06 | 0.06 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| A1-9 | Impedance mismatch between the receiving antenna and the network analyzer | 0.05 | 0.05 | 0.05 | U-shaped | 1.41 | 1 | 0.04 | 0.04 | 0.04 |
| A1-10 | Positioning and pointing misalignment between the reference antenna and the receiving antenna | 0.01 | 0.01 | 0.01 | Rectangular | 1.73 | 1 | 0.01 | 0.01 | 0.01 |
| A1-11 | Impedance mismatch between the reference antenna and the network analyzer | 0.05 | 0.05 | 0.05 | U-shaped | 1.41 | 1 | 0.04 | 0.04 | 0.04 |
| A1-3 | Quality of quiet zone (normal test conditions) | 0.10 | 0.10 | 0.10 | Gaussian | 1.00 | 1 | 0.10 | 0.10 | 0.10 |
| A1-4b | Polarization mismatch between the reference antenna and the receiving antenna | 0.01 | 0.01 | 0.01 | Rectangular | 1.73 | 1 | 0.01 | 0.01 | 0.01 |
| A1-5b | Mutual coupling between the reference antenna and the receiving antenna | 0.00 | 0.00 | 0.00 | Rectangular | 1.73 | 1 | 0.00 | 0.00 | 0.00 |
| A1-6 | Phase curvature | 0.05 | 0.05 | 0.05 | Gaussian | 1.00 | 1 | 0.05 | 0.05 | 0.05 |
| C1-3 | Uncertainty of the network analyzer | 0.13 | 0.20 | 0.20 | Gaussian | 1.00 | 1 | 0.13 | 0.20 | 0.20 |
| A1-12 | Influence of the reference antenna feed cable | 0.05 | 0.05 | 0.05 | Rectangular | 1.73 | 1 | 0.03 | 0.03 | 0.03 |
| A1-13 | Reference antenna feed cable loss measurement uncertainty | 0.06 | 0.06 | 0.06 | Gaussian | 1.00 | 1 | 0.06 | 0.06 | 0.06 |
| A1-14 | Influence of the receiving antenna feed cable | 0.05 | 0.05 | 0.05 | Rectangular | 1.73 | 1 | 0.03 | 0.03 | 0.03 |
| C1-4 | Uncertainty of the absolute gain of the reference antenna | 0.50 | 0.43 | 0.43 | Rectangular | 1.73 | 1 | 0.29 | 0.25 | 0.25 |
| A1-15 | Uncertainty of the absolute gain of the receiving antenna | 0.00 | 0.00 | 0.00 | Rectangular | 1.73 | 1 | 0.00 | 0.00 | 0.00 |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  | 0.44 | 0.54 | 0.54 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  | 0.87 | 1.06 | 1.06 |
| TRP Summation error |  |  |  |  |  |  |  | 0.75 | 0.75 | 0.75 |
| Total MU |  |  |  |  |  |  |  | 1.15 | 1.30 | 1.30 |

There are some uncertainties already defined in 37.941 used for different requirements which may be applicable.

    - Dynamic Range

When measuring spurious emissions it is necessary to measure low levels of emissions in the presence of the larger wanted signal, albeit at a different frequency. In the case of EIRP it is necessary to measure the sidelobes at the same frequency as the main lobe, as such it is the directional isolation of the chamber we rely on to attenuate the main lobe sufficiently so that the side lobes can be measured. Both the size of the chamber (path loss) and the quality of the RAM (Radio Assortative material) to supress reflections impact the attenuation of any reflections. It is also possible in some circumstances to deliberately place a block in the path of the main beam.

Chamber dynamic range is already defined for the CATR chamber in 37.941.

A2-17 Measurement system dynamic range uncertainty

Uncertainty associated with the addition of each of the directional power measurements to calculate the TRP due to the limited dynamic range of the OTA test system causing an overestimation.

The limited dynamic range described refers to the measurement of the noise floor (noise being any unwanted signal) being higher than the signal in the measured direction. The measurement floor of the chamber in this case is related to a number of things:

- The noise floor of the measurement equipment (translated back to the DUT)

- The limited isolation in the chamber resulting in reflections of the main beam interfering with the measurement.

For TRP this is the same mechanism we are considering as for EEIRP, however the impact is less in the case of TRP, for example in the examples studied so far we have considered a worst case system with 30dBm/MHz output power and a maximum antenna gain (Antenna 5) of 33.7dBi so an EIRP of 63.7dBm/MHz.

The lowest EEIRP requirement is -15dBm/MHz

Currently the dynamic range contribution is 0.51dB, in order to maintain that level of accuracy in the above scenario an isolation of approximately 88dB would be required:

Pint=10*log101063.7-5810+101510=15.5dBm/MHz

To maintain the same error for TRP a chamber isolation of ~60dB would be sufficient, this is a reasonable value for an anechoic chamber

Observation 5: A chamber isolation of approx. 60dB is required to provide a dynamic range measurement uncertainty of 0.51dB (the value currently used for FR1 CATR)

    - Low power measurement

As the chamber must be relied upon to remove the large main beam the test equipment dynamic range is not an issue (as it is perhaps in an ACLR type measurement) so the issue is if the measurement equipment can measure the required power level to the desired accuracy.

OBUE requirements have a -15dBm/MHz TRP requirement level which is similar to the lowest EEIRP requirement, and can be use as a guide for the measurement equipment accuracy, however the RF measurement equipment uncertainty for OBUE is the same as that for TRP requirements (0.26dB @ 6GHz) as such we can assume the measurement accuracy is not affected by these levels

Observation 6: RF power level is sufficient that RF power measurement accuracy is not affected, we can use existing values.

    - TRP/EEIRP Summation Error

The TRP summation error is defines as:

TRP summation error: the relative difference between theoretical TRP and numerically estimated TRP

In 37.941sub-clause it derivation is expanded upon:

6.3.6	TRP summation error

In practice, discrete samples of EIRP or power density are measured at different directions (θ,ϕ) over the entire sphere, which are used to numerically approximate the surface TRPReference integral. The obtained value, TRPEstimate, is an approximation of TRPReference and the difference between them is defined as the TRP summation error (SE) which is

SE = | TRPEstimate – TRPreference | (in dB)

The SE is the error caused by the numerical integration of measured values on the grid to obtain TRPEstimate, given that the number of test points correspond to the reference angular steps, which are in turn dependent on antenna size and frequency, as described in clause 6.3.4. A reasonable trade-off between accuracy and sampling time is achieved when the SE = 0.75 dB.

Whilst EEIRP is not exactly the same measurement as TRP as angular bins are separated rather than measuring the entire sphere the relationship between measurement step size and uncertainty is similar.

One major difference is that minimum step size for TRP is defined by the main beam - beam width. This is correct as the power of the main beam far exceeds the power in all of the side lobes. As such only the error in measuring main beam (or possibly major grating lobes which are related to main beam) will affect the accuracy of the TRP.

For EEIRP in elevation the requirement is not interested in the main lobe but the power in the sidelobes alone.

Looking at ANT4 from we can see:

Figure 2.1.4-1: ANT4 elevation plot

The beam width of the main beam is 3.2° however the widest sidelobe is 5.2° and the narrowest sidelobe is 1.7°.

ANT2 has a main beam – beam width of 4.6° with widest sidelobe of 14.5° and narrowest of 2.6°.

There is perhaps a weak relationship between the main beam - beam width and the max and min side lobe beam widths but clearly the measurement step size rules for TRP are not suitable for all the side lobes.

If we look at the error in EEIRP (total average error from all ANTS and all angular bins we see:

Figure 2.1.4-2: EEIRP average Error vs grid step size and % beam width – Test vector E325

The left hand graph shows error with respect to step size in deg the right hand graph shows the same data with respect to step size as a percentage of main beam - beam width. It’s clear that the error is not directly related to the main beam width, in fact all antennas have a very similar pattern with respect to the step size despite their beam width varying from 2.3° to 4.5°.

Observation 7: Step size accuracy is not related to main beam – beam width

As there seems to be no link between the declared information regarding the beam (i.e. main beam – beam width) and the EEIRP accuracy with step size we should agree a reasonable maximum step size for accuracy, considering the range of antennas and sub-arrays likely to be used. In this case a step size of 1° offers a reasonable trade off between accuracy and test time.

Proposal 2: Elevation step size should be 1°.

In azimuth the entire range of angles is measured and as such the main lobe of the azimuth main beam is always included, the minimum step size therefore is similar to the TRP in that it can be directly related to the main beam - beam width (in azimuth). Sweeping the azimuth step size (with an elevation step size of 1deg) we see the results in figure 1.1.4-3.

Figure 2.1.4-3: EEIRP average Error vs azimuth grid step size

In this case the accuracy is related to the azimuth beam width (as with TRP), accuracy is held with the step size up to 1x the beam width, as such the existing TRP rule that step size is at least half the beam width can be maintained.

Observation 8: the EEIRP error is much more dependent on elevation step size than azimuth step size.

Proposal 3: Azimuth step size should be at least half the main beam azimuth beam width

As with TRP the step size offers a trade off between accuracy and test time, when assessing acceptable levels of error the TRP summation error vale of 0.75dB was considered. This would seem to a be reasonable bench mark for accuracy so we should use the same summation error value for EEIRP.

Proposal 4: EEIRP summation Error is 0.75dB

    - Preliminary EEIRP MU budget

Taking into account the uncertainties highlighted we have for CATR:

| Table 11.2.3.3-1: CATR MU value derivation for OTA BS output power measurement, FR1 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |
|  |  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤7.125 GHz |  |  |  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤7.125 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |
| A2-18a | Misalignment and pointing error of BS (for TRP) | 0.30 | 0.30 | 0.30 | Rectangular | 1.73 | 1 | 0.173 | 0.173 | 0.173 |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | 0.26 | Gaussian | 1.00 | 1 | 0.140 | 0.260 | 0.260 |
| A2-2a | Standing wave between BS and test range antenna | 0.21 | 0.21 | 0.21 | U-shaped | 1.41 | 1 | 0.148 | 0.148 | 0.148 |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.00 | 0.00 | 0.00 | Gaussian | 1.00 | 1 | 0.001 | 0.001 | 0.001 |
| A2-4a | QZ ripple experienced by BS | 0.09 | 0.09 | 0.09 | Gaussian | 1.00 | 1 | 0.093 | 0.093 | 0.093 |
| A2-12 | Frequency flatness of test system | 0.25 | 0.25 | 0.25 | Gaussian | 1.00 | 1 | 0.250 | 0.250 | 0.250 |
| A2-17 | Measurement system dynamic range uncertainty | 0.51 | 0.51 | 0.51 | Gaussian | 1 | 1 | 0.51 | 0.51 | 0.51 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Uncertainty of the network analyzer | 0.13 | 0.20 | 0.20 | Gaussian | 1.00 | 1 | 0.13 | 0.20 | 0.20 |
| A2-5a | Mismatch of receiver chain between receiving antenna and measurement receiver | 0.13 | 0.33 | 0.33 | U-shaped | 1.41 | 1 | 0.09 | 0.23 | 0.23 |
| A2-6 | Insertion loss of receiver chain | 0.18 | 0.18 | 0.18 | Rectangular | 1.73 | 1 | 0.10 | 0.10 | 0.10 |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.00 | 0.00 | 0.00 | Gaussian | 1.00 | 1 | 0.00 | 0.00 | 0.00 |
| A2-7 | Influence of the calibration antenna feed cable | 0.02 | 0.02 | 0.02 | U-shaped | 1.41 | 1 | 0.02 | 0.02 | 0.02 |
| C1-4 | Uncertainty of the absolute gain of the reference antenna | 0.50 | 0.43 | 0.43 | Rectangular | 1.73 | 1 | 0.29 | 0.25 | 0.25 |
| A2-8 | Misalignment positioning system | 0.00 | 0.00 | 0.00 | Exp. normal | 2.00 | 1 | 0.00 | 0.00 | 0.00 |
| A2-18b | Misalignment and pointing error of calibration antenna (for TRP) | 0.50 | 0.50 | 0.50 | Exp. normal | 2.00 | 1 | 0.25 | 0.25 | 0.25 |
| A2-9 | Rotary joints | 0.05 | 0.05 | 0.05 | U-shaped | 1.41 | 1 | 0.03 | 0.03 | 0.03 |
| A2-2b | Standing wave between calibration antenna and test range antenna | 0.09 | 0.09 | 0.09 | U-shaped | 1.41 | 1 | 0.06 | 0.06 | 0.06 |
| A2-4b | QZ ripple experienced by calibration antenna (normal test conditions) | 0.01 | 0.01 | 0.01 | Gaussian | 1.00 | 1 | 0.01 | 0.01 | 0.01 |
| A2-11 | Switching uncertainty | 0.26 | 0.26 | 0.26 | Rectangular | 1.73 | 1 | 0.15 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  | 0.78 | 0.84 | 0.84 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  | 1.53 | 1.65 | 1.65 |
| EEIRP Summation error |  |  |  |  |  |  |  | 0.75 | 0.75 | 0.75 |
| Total MU |  |  |  |  |  |  |  | 1.71 | 1.81 | 1.81 |

* note there may be an error in A2-5a, the value in the SEM TRP budget is larger – check this value

3	Conformance test method

Clearly there are some differences between TRP and EEIRP

- EEIRP defines average levels in a number of elevation ranges whereas TRP averages the entire sphere.

- EEIRP requires the beam direction to be swept over the steering range (This is not excluded for TRP measurement but not required)

- EEIRP requirement levels are lower

- EEIRP requirement includes consideration of mechanical tilt.

3.1	Spherical sampling points

Expected EIRP requires measurements averaged over +/-180deg in azimuth and 0-90deg elevation, this half sphere is very similar to the spherical grid measurement we use for TRP

Figure 3.1-1: Spherical equal angle sampling grid

The equal angle sampling grid has a concentration of points near the poles which are somewhat unnecessary, methods to reduce the number of points are detailed in annex I of 38.141-2, however the equal spaced grid is simplest to describe so is useful as a primary method.

Existing directional measurements are made with respect to the antenna coordinate system rather than the horizon, mechanical tilt will offset the Expected EIRP requirements with respect to the antenna co-ordinate system. The radiation wrt the antenna only needs to be measured once and the results with respect to the horizon for different mechanical tilts can be calculated by mathematically transposing the measurement points if necessary.

Only half the sphere need be measured however if mechanical tilts are considered this should be extended to include potential mechanical tilt ranges.

Observation 9: Mechanical tilts can be applied mathematically to the measured points to achieve Expected EIRP results if necessary.

3.2	Measurement conditions

Measurement conditions cover the environmental conditions, the frequencies to be tested, and the directions to be tested.

Looking at each of these:

Environmental conditions:

Currently only a very few requirements are measured over extreme environmental conditions, this is due to the time and complexity of carrying out such measurements. Using existing methods for thermal testing it would not be possible to apply them to the EEIRP requirement. As such we do not recommend extreme testing for the EEIRP at this time.

Proposal 5: EEIRP is test under ambient conditions.

Frequencies to be tested:

The EEIRP frequency range 6425 – 7075 MHz is approximately 10%. Applying this frequency range to ANT4 for example makes a 0.5dBm variation in antenna gain from bottom to top frequency.

Looking at the EEIRP results over a 10% frequency differential we see the following errors:

Table 3.2-1: EEIRP deltas over frequency

| Test vector type | Beamwidth |  | Elevation Angle range (deg) |  |  |  |  |  |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | az | el | 60-90 | 30-60 | 20-30 | 15-20 | 10-15 | 5-10 | 0-5 |  |
| E325 (mech=8, elec=-5 to +5) - 1/2 deg step - 95%λ (low freq) |  |  |  |  |  |  |  |  |  |  |
| ANT1 | 12.6 | 4.5 | -0.73 | -0.20 | 0.03 | 0.12 | 0.15 | 0.15 | 0.10 | dB |
| ANT2 | 12.6 | 4.5 | -3.83 | -0.33 | 0.08 | -1.43 | 1.13 | -0.54 | 0.10 | dB |
| ANT4 | 6.3 | 3.0 | 4.02 | -0.23 | -0.03 | -0.19 | -1.34 | 0.07 | 0.12 | dB |
| ANT5 | 6.3 | 2.3 | -0.22 | -0.69 | -1.01 | 0.06 | 0.13 | 0.15 | 0.07 | dB |
| E325 (mech=8, elec=-5 to +5) - 1/2 deg step - 105%λ (high freq) |  |  |  |  |  |  |  |  |  |  |
| ANT1 | 12.6 | 4.5 | 0.84 | 0.28 | -0.06 | -0.11 | -0.13 | -0.03 | 0.05 | dB |
| ANT2 | 12.6 | 4.5 | 0.86 | 0.14 | 0.23 | 0.92 | -1.01 | 0.35 | 0.13 | dB |
| ANT4 | 6.3 | 3.0 | 6.23 | 0.16 | 0.30 | -0.74 | 1.38 | 0.06 | 0.03 | dB |
| ANT5 | 6.3 | 2.3 | 0.59 | -0.07 | 2.49 | -0.05 | -0.13 | 0.00 | 0.09 | dB |

As with step size the trade of between test coverage and test time is important, as the differences between the channels are in most case very small it seems testing a single channel is sufficient. We propose to test middle

It should also be noted that as the EEIRP requirement does not apply to the entire n104 band (6425 – 7125) as such the definition of B,M,T are not correct as they apply to the operating band not the band to which EEIRP applies. This should be clarified when defining the test frequencies.

In addition as the EEIRP requirement only applies to part of a single band as such the multi-band test conditions do not apply.

Proposal 6: EEIRP is tested M of the appropriate range of frequencies to which EEIRP is defined.

Finally the text directions to be tested will be decided by the beam steering test vectors with are discussed in section 1.

3.3	Measurement procedure

The TRP test procedure (in an anechoic chamber) where the DUT is rotated in 3 dimensions with a fixed calibrated measurement path is very similar to that required for Expected EIRP.

Figure 3.3-1: Principle test environment

The procedure steps in the conformance requirements are not very detailed, however the existing TRP measurement steps can be used as a starting point for the Expected EIRP measurement steps.

The TRP method makes all measurement relative to the BS measurement grid (as this is how all requirements were specified prior to EEIRP). EEIRP is related to the horizon and hence the relationship between the BS coordinate system and the Horizon (i.e. the mechanical tilt) is important. As we are measuring in an anechoic chamber over a sphere (or hemisphere) the mechanical tilt can be applied in post processing, as would seem to be advantageous as if necessary any number of mechanical tilts could be applied to the same data set. It is also possible to offset the measurement grid by the mechanical tilt and effectively tilt the BS on the positioner but it is not clear why this would be necessary. The procedure should allow for both approaches however.

Starting with the TRP procedure a number of changes are of course required:

- The alternative reverberation chamber measurements are not possible for EEIRP

- The references to Annex I whilst many of the concepts may be applicable to EEIRP ate TRP specific. A EEIRP specific annex should be added – Annex N

- An additional step needs to be added instructing the sweeping of the beam over the steering direction test vectors. This is included in the “repeat steps…” instruction

- The calculation step includes instruction to add the mechanical tilt if necessary.

- The note discussing the order of the measurement grid sweep, frequency sweep (if necessary for EEIRP?) has the beam direction sweep added to it.

  - It is likely the most efficient way of measurement will be to vary the beam steering direction at each of the directional measurement points as it is faster to change the beam steering direction (which should be a electrical steering) than to change the measurement direction (which requires moving the BS on the positioner), however there is no need for the procedure to implicitly specify this.

Applying these modifications to the TRP procedure we have:

The following procedure for measuring EEIRP is based on directional power measurements as described in annex M. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber if so follow steps 1, 3, 4, 5, 7 and 10.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	Measurements shall use a measurement bandwidth in accordance to the conditions in clause x.x.x.x.

4)	The measurement device characteristics shall be:

-	Detection mode: True RMS.

-	The emission power should be averaged over an appropriate time duration to ensure the measurement is within the measurement uncertainty in Table 4.1.2.2-1 for FR1 and Table 4.1.2.2-2 for FR2.

5)	Set the BS to transmit:

-	For RIB declared to be capable of single carrier operation only, set the RIB to transmit a signal according to the applicable test configuration in clause 4.8 using the corresponding test model in clause 4.9.2 (i.e. NR-FR1-TM1.1 for BS type 1-O and NR-FR2-TM1.1 for BS type 2-O), at manufacturer's declared rated output power Prated,c,TRP.

-	For a RIB declared to be capable of multi-carrier and/or CA operation, set the RIB to transmit according to the corresponding test model in clause 4.9.2 on all carriers configured using the applicable test configuration and corresponding power setting specified in clause 4.7.2 and 4.8.

6)	Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna such that measurements to determine EEIRP can be performed (see annex M).

7)	Sweep the beam direction over the directions required by the steering test vector (TVxx)

8)	Measure the emission at the specified frequencies with specified measurement bandwidth.

9)	Repeat step 6-8 for all measurement directions in the appropriated EEIRP measurement grid (see annex M), and all beam steering directions in the steering test vector.

NOTE 1:	The EEIRP measurement grid may not be the same for all measurement frequencies.

NOTE 2:	The frequency sweep, the EEIRP measurement grid sweep or the beam steering vector sweep may be done in any order.

10)	Calculate EEIRP for each elevation angle range, applying the mechanical tilt offsets if necessary at each specified frequency using the directional measurements.

This procedure is used in the draft CR in the annex of this paper.

3.4	Test requirements

Expected EIRP requirements are regulatory emission requirements, as such we do not apply an TT to them so the test requirements are the same as the core requirements

Proposal 7: As EEIRP requirements are regulatory emissions requirements TT=0.

4	Summary

4.1	Test Vectors

A number of test vectors both identified internally and from other papers have been examined with respect to a reference of 10k random directions. A range of antennas have been used with varied beam width and sub-array capability.  We note

Observation1: We have used 10k random directions to form a stable reference spatial power distribution pattern as this most closely matched the requirement definition.

The average error of each test vector is then compared to a level of 0.75dB, and it has been identified that:

Observation 2: Test Vector 3x3 offers the best balance between accuracy and test time.

We have also highlighted that further work is required based on the likely BS declarations for OTA peak directions sets

Observation 3: The OTA peak directions set is not necessarily rectangular

Observation 4: A rectangular OTA peak direction set does not represent likely BS usage.

Proposal 1: The Test vector should be suitable for any OTA peak directions set not just rectangular

4.2	MU budgets

We have looked at the difference between the equally spaced grid TRP measurement and the new EEIRP measurement. An additional uncertainty related to the chamber dynamic range has been added and, the following observation are made:

Observation 5: A chamber isolation of approx. 60dB is required to provide a dynamic range measurement uncertainty of 0.51dB (the value currently used for FR1 CATR)

Observation 6: RF power level is sufficient that RF power measurement accuracy is not affected, we can use existing values.

Uncertainty with respect to measurement grid size has been studied with the following observations and proposals.

Observation 7: Step size accuracy is not related to main beam – beam width

Observation 8: the EEIRP error is much more dependent on elevation step size than azimuth step size.

Proposal 2: Elevation step size should be 1°.

Proposal 3: Azimuth step size should be at least half the main bema azimuth beam width

Proposal 4: EEIRP summation Error is 0.75dB

Finally a preliminary MU budget for the IAC and the CATR have been presented.

4.3	Conformance test method

In this paper we look at an appropriate test procedure for the Expected EIRP requirement based on the existing TRP procedure for an equally spaced grid. With the following observation:

Observation 9: Mechanical tilts can be applied mathematically to the measured points to achieve Expected EIRP results if necessary.

The test procedure Initially conditions are discussed with the following proposals

Proposal 5: EEIRP is test under ambient conditions.

Proposal 6: EEIRP is tested M of the appropriate range of frequencies to which EEIRP is defined.

A test procedure is suggested (with changes from the TRP procedure highlighted) and finally the test requirements are discussed with the following proposal.

Proposal 7: As EEIRP requirements are regulatory emissions requirements TT=0.

A draft CR to the Base Station (BS) conformance testing, Part 2: Radiated conformance testing, 38.141-2 is provided in Annex A.

References

[1]	R4-2409401 Expected EIRP requirements, Huawei

[2] R4-2409118 On introduction of OTA spatial emission above the horizon requirement for band n104, Ericsson

Annex A – draft CR to 38.141-2 v18.6.0

<start of changes>

Table 4.6-1 Manufacturers declarations for BS type 1-H, BS type 1-O and BS type 2-O radiated test requirements

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BS type 1-H (Note 2) | BS type 1-O | BS type 2-O |
| D.1 | Coordinate system reference point | Location of coordinated system reference point in reference to an identifiable physical feature of the BS enclosure. | x | x | x |
| D.2 | Coordinate system orientation | Orientation of the coordinate system in reference to an identifiable physical feature of the BS enclosure. | x | x | x |
| D.3 | Beam identifier | A unique title to identify a beam, e.g. a, b, c or 1, 2, 3. The vendor may declare any number of beams with unique identifiers. The minimum set to declare for conformance, corresponds to the beams at the reference beam direction with the highest intended EIRP, and covering the properties listed below: 1) A beam with the narrowest intended BeWθ and narrowest intended BeWϕ possible when narrowest intended BeWθ is used. 2) A beam with the narrowest intended BeWϕ and narrowest intended BeWθ possible when narrowest intended BeWϕ is used. 3) A beam with the widest intended BeWθ and widest intended BeWϕ possible when widest intended BeWθ is used. 4) A beam with the widest intended BeWϕ and widest intended BeWθ possible when widest intended BeWϕ is used. 5) A beam which provides the highest intended EIRP of all possible beams. When selecting the above five beam widths for declaration, all beams that the BS is intended to produce shall be considered, including beams that during operation may be identified by any kind of cell or UE specific reference signals, with the exception of any type of beam that is created from a group of transmitters that are not all phase synchronised. For BS required to meet EEIRP requirements in sub-clause 6.9. the following beams must also be declared 6) A beam which meets the EEIRP requirements when minimum mechanical tilt is applied [7) A beam which meets the EEIRP requirements when maximum mechanical tilt is applied] (Note 3) | x | x | x |
| D.4 | Operating bands and frequency ranges | List of NR operating band(s) supported by the BS and if applicable, frequency range(s) within the operating band(s) that the BS can operate in. Supported bands declared for every beam (D.3). (Note 4) | c | x | x |
| D.5 | BS requirements set | Declaration of one of the NR base station requirement's set as defined for BS type 1-H, BS type 1-O, or BS type 2-O. | c | x | x |
| D.6 | BS class | Declared as Wide Area BS, Medium Range BS, or Local Area BS. | c | x | x |
| D.7 | BS channel band width and SCS support | BS supported SCS and channel bandwidth per supported SCS. Declared for each beam (D.3) and each operating band (D.4). | c | x | x |
| D.8 | OTA peak directions set reference beam direction pair | The beam direction pair, describing the reference beam peak direction and the reference beam centre direction. Declared for every beam (D.3). | x | x | x |
| D.9 | OTA peak directions set | The OTA peak directions set for each beam. Declared for every beam (D.3). | x | x | x |
| D.10 | OTA peak directions set maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam (D.3). | x | x | x |

<Next change>

  - 6.9	OTA spatial emission

6.9.1	Definition and applicability

OTA spatial emission requirements are defined to set upper limits on radiated power in specific directions.

6.9.2	Minimum requirement

The minimum requirement for BS type 1-O operation is defined in TS 38.104 [2], clause 9.9.2.

The OTA spatial emission requirement is not applicable for BS type 2-O.

6.9.3	Test purpose

The test purpose is to verify the ability of the transmitter units associated with the RIB under test to radiated power in specified elevation angle ranges with respect to the horizon.

6.9.4	Method of test

6.9.4.1	Initial conditions

Test environment: normal; see annex B.2.

RF channels to be tested for single carrier:	M for the applicable frequency band over which EEIRP requirements apply; see clause 4.9.1.

Base Station RF Bandwidth positions to be tested for multi-carrier:

-	MRFBW in single-band RIB, see clause 4.9.1;

Directions to be tested: For the beam identifiers (D3) which apply to the EEIRP requirement (D3_5, D3_6) and the associated OTA peak directions set reference beam direction pair (D8) and OTA peak directions set maximum steering direction(s) (D10) the beam steering test vector is defined as (θ,φ):

θD8-0.66(θD8-θD10_4), φD8-0.66(φD8-φD10_2θD8-0.66(θD8-θD10_4), φD8θD8-0.66(θD8-θD10_4),φD8-0.66(φD8-φD10_1)θD8, φD8-0.66(φD8-φD10_2)θD8, φD8θD8,φD8-0.66(φD8-φD10_1)θD8-0.66(θD8-θD10_3), φD8-0.66(φD8-φD10_2)θD8-0.66(θD8-θD10_3), φD8θD8-0.66(θD8-θD10_3),φD8-0.66(φD8-φD10_1)

6.9.4.2	Procedure

The following procedure for measuring EEIRP is based on directional power measurements as described in annex M.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	Measurements shall use a measurement bandwidth in accordance to the conditions in clause 6.9.5.

4)	The measurement device characteristics shall be:

-	Detection mode: True RMS.

-	The emission power should be averaged over an appropriate time duration to ensure the measurement is within the measurement uncertainty in Table 4.1.2.2-1 for FR1.

5)	Set the BS to transmit:

-	For RIB declared to be capable of single carrier operation only, set the RIB to transmit a signal according to the applicable test configuration in clause 4.8 using the corresponding test model in clause 4.9.2 (i.e. NR-FR1-TM1.1 for BS type 1-O and NR-FR2-TM1.1 for BS type 2-O), at manufacturer's declared rated output power Prated,c,TRP.

-	For a RIB declared to be capable of multi-carrier and/or CA operation, set the RIB to transmit according to the corresponding test model in clause 4.9.2 on all carriers configured using the applicable test configuration and corresponding power setting specified in clause 4.7.2 and 4.8.

6)	Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna such that measurements to determine EEIRP can be performed (see annex M).

7)	Sweep the beam direction over the directions required by the steering test vector in sub-clause 6.9.4.1.

8)	Measure the emission at the specified frequencies with specified measurement bandwidth.

9)	Repeat step 6-8 for all measurement directions in the appropriated EEIRP measurement grid (see annex M), and all beam steering directions in the steering test vector.

NOTE 1:	The EEIRP measurement grid may not be the same for all measurement frequencies.

NOTE 2:	The frequency sweep, the EEIRP measurement grid sweep or the beam steering vector sweep may be done in any order.

10)	Calculate EEIRP for each elevation angular range, applying the mechanical tilt offsets if necessary at each specified frequency using the directional measurements.

6.9.5	Test requirements

6.9.5.1	Requirement for BS type 1-O

For BS type 1-H and BS type 1-O operating in band n104, the Expected EIRP (EEIRP) in the frequency range 6425 – 7075 MHz , the power measured in step 10 shall not exceed the values specified in table 6.9.5.1-1.

Table 6.9.5.1-1: EEIRP limits as function of elevation above horizon

| Elevation angular range [θL ≤ θ < θH] (Degrees) | EEIRP limit (dBm/MHz) |
| --- | --- |
| 0<<5 | 27 |
| 5<<10 | 23 |
| 10<<15 | 19 |
| 15<<20 | 18 |
| 20<<30 | 16 |
| 30<<60 | 15 |
| 60<<90 | 15 |
| Note 2: The requirement shall apply to all supported mechanical tilts. |  |

<Next change>

Annex M (normative):
EEIRP measurement procedures

M.1	General

The annex describes various procedures for BS OTA EEIRP measurments.

When making TRP measurements the alignment between EUT and measurement antenna is important to achieve expected measurement uncertainty:

1.	The measurement antenna needs to be aligned tangential to the measurement surface forming a sphere around the EUT, in order to correctly measure the EEIRP properly.

M.2	Spherical equal angle grid

M.2.1	General

For an equally spaced grid EEIRPEstimate is calculated as:

EEIRP=12πsinθH-sinθLθLθH-ππP1θ,φcosθdφdθ

Where P1 is the average power in each measurement direction when the beam steering direction is applied over the OTA peak directions set with a uniform angular distribution

So

P1θ0φ0=n=1NwnPθ0φ0;αnβn

Where

θ0 and φ0  the measurement grid angle

αn, βn are the beamforming directions

N is the number of beamforming directions required for accurate averaging

Wn is the weight of the beam in the average (1/N)

So

As a discrete sum for the equally spaced grid

EEIRPEsitimaeθL,θH=12πsinθH-sinθLθHθH-τππP1θ,φcosθ∆φ∆θ

where:

dφ≈∆φ=2πM, 	M is the number of azimuth points

∆θ can be calculated a few ways but they should all give the same answer:

For a specific elevation angle range :	dθ≈∆θ=θH-θLN , N is No of elevation points in elevation angle range

For a half sphere	:   dθ≈∆θ=π2N	, N is No of elevation points in hemisphere (π/2)

For a full sphere:	   dθ≈∆θ=πN	, N is No of elevation points in sphere (π)

M.2.1	Rotating an equal angle grid

{ equations for how to rotate the grid can be given here}

<end of changes>