3GPP RAN WG4 Meeting #112		R4-2413277

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	Huawei, HiSilicon

Title: 	TX IMD requirements for high frequency band in FR1

Agenda Item:	8.11.3

Document for:	Approval

1 Introduction

There have been a number of proposals to update the co-location test procedure, issues with the current methodology have included:

- Problems sourcing suitable CLTA for all bands

- Problems generating sufficient Tx power for the interference measurements

- The current specification not covering latest implementation

The paper [R4-2409402] has investigated the background of the OTA co-location requirements and how the current CLTA method was approved. In general, most of the reasons why the CLTA was defined seem to be the same today and hence are useful to reiterate.

In this paper we look at the background of the TX IMD requirements and see if it is still be needed for high frequency band in FR1.

2 Discussion

As discussed in 7-24 GHz SI [TR 38.820], TX IMD is specified under the assumption that the BS is co-located with another BS of the same class, output power and antenna configuration. The reverse interferer specified in the TX IMD requirement is therefore based on the BS output power (as interfering BS is assumed to be the same power) and the isolation between the 2 systems.

- For BS type 1-C the isolation between the victim and aggressor antenna ports is 30 dB.

- For BS type 1-H an intra system requirement is defined.

- For BS type 1-O co-location a co-location reference antenna is specified and placed physically at a distance of 10cm from AAS BS and the full BS power is fed into it as an interferer.

- For FR2 the OTA scenario is the same however it can be shown that the isolation is such that the requirement becomes unnecessary.

The necessity of the TX IMD requirement can be analyzed by understanding the isolation, the aggressor Pout and the emissions requirements (notably ACLR), as frequency increases the ACLR requirement is smaller, achievable Pout is lower and isolation increases.

TX IMD products are calculated as follows:

PIMD=Pint-2IP3-Pwanted dBm

When the 2 tones are not equal one product is larger than the other – there may be some phase dependence which can change the power distribution but the larger product represents a worst case.

The power of the wanted is known, the power of the interferer is based on the co-located system output power and the isolation (CL). In order to pass the requirement, the power of the IMD product must be below any emission requirements. The toughest of which is assumed to be the ACLR requirement.

What is unknown is the IP3 of the BS final stage, however a number of worst-case assumptions can be made to make a safe estimate.

- The amplifier must not clip the modulated signal so the P1dB must be at least the average power plus the PAR.

- A common assumption is IP3 is 10dB above P1dB – assuming IP3 = P1dB is a safe assumption.

Hence,

PIMD=Pint-2IP3-Pwanted =Pwanted-CL-2Pwanted+PAR-Pwanted=Pwanted-CL-2*PAR

To find out what the minimum isolation requirement needed to eliminate the TX IMD requirement, assume to be inconsequential the IMD product is 10dB lower than the specification, and PAR=8dB.

PIMD=(Pwanted-CL-2*PAR)≤ (Pwanted-ACLR-10)

CL≥ACLR+10-2*PAR=ACLR-6

So for 45 dB ACLR, the required coupling loss (CL) is 45-6 = 39 dB and for band n104 the required coupling loss is 32 dB. In above evaluation pretty safe assumptions are made, i.e. setting reverse IP3 the same as P1dB.

Observation 1: A min isolation of approx. 40dB is probably enough to remove the reverse IMD requirement. For band n104 even with 30dB isolation the reverse IMD requirement can probably be removed.

CLTA

AAS

d =10 cm

The following we did some measurement on a 3.5 GHz AAS product and a CLTA antenna. The measured center frequency is 3500 MHz with 80 MHz channel bandwidth. The coupling loss from CLTA to each RX channel is shown in Table 2-1. It can be found the coupling loss is in the range of 42 dB ~ 64 dB.

Table 2-1: Co-location test for 3.5G AAS

| CH24 | CH8 | CH28 | CH12 | CH31 | CH15 |
| --- | --- | --- | --- | --- | --- |
| -60.74 | -63.91 | -53.94 | -54.28 | -42.18 | -45.99 |
| -63.14 | -61 | -54.02 | -55.58 | -49.12 | -47.97 |
| CH0 | CH16 | CH4 | CH20 | CH7 | CH23 |
| -57.81 | -61.95 | -55.79 | -58.07 | -51.06 | -60.3 |
| -54.3 | -59.61 | -52.43 | -52.98 | -44.03 | -47.51 |

Proposal: It is proposed to remove TX IMD requirements for high FR1 band, e.g. frequency bands above 4.2 GHz.

3 Conclusion

In this paper we look at the background of the TX IMD requirements and evaluate the need high frequency band in FR1.

Proposal: It is proposed to remove TX IMD requirements for high FR1 band, e.g. frequency bands above 4.2 GHz.

References

[1]	TR 37.843	Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS) Radiated requirements

[2]	TSGR4#8(99)631: "Antenna-to-Antenna Isolation Measurements", Allgon

[3] R4-1703900	Co-location unwanted emission requirements,	Huawei

[4] R4-1704323	WF on co-location requirements (RAN4#82bis)

[5]	R4- 1706133	Way forward on co-location (RAN4#83)