3GPP RAN WG4 Meeting #112		R4-2413278

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	Huawei, HiSilicon

Title: 	TP for IMT technology related parameters for 4400 to 4800 MHz

Agenda Item:	8.2.2

Document for:	Approval

- Introduction

The contribution provides a TP for TR 38.922 to correct the applicable channel bandwidth(s) for ACLR for 4400 to 4800 MHz.

References

[1]	R4-2410763,	TR 38.922 version 0.1.0

- TP to TR 38.922

<Start of TP>

4.2.1.3	ACLR

From TS 38.104, subclause 6.6.3 the ACLR limit applicable for band n79 is listed in Table 4.2.1.3-1.

Table 4.2.1.3-1: Base station ACLR limit

| BS channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 10, 20, 30, 40, 50, 60, 70, 80, 90,100 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 45 dB |
|  | 2 x BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 45 dB |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). |  |  |  |  |

<End of TP>