3GPP RAN WG4 Meeting #112		R4-2413279

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	Huawei, HiSilicon

Title: 	Remaining issues on IMT parameters for range 7125 to 8400 MHz

Agenda Item:	8.2.3

Document for:	Approval

- Introduction

In the last plenary (RAN#103) the Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved [1]. For frequency range 7125 to 8400 MHz and 14800 to 15350 MHz, the WF was agreed in [2] at RAN#110bis. And at RAN#111, further agreements were made in [3].

This paper looks at the remaining issues and provides the proposals for the range 7125 to 8400 MHz.

- Discussion

2.1 IMT technology related parameters

Issue 2-3: Typical channel bandwidth

Agreement:

- Mention 100MHz as typical bandwidth in the LS

  - FFS whether to mention wider bandwidths may be possible in future in the LS

- Describe other possibilities for bandwidth in the TR, mentioning that additional specification effort needed for  >100MHz

In our understanding, wider channel bandwidth or multiple 100 MHz carriers CA would not impact to WP5D sharing study. And since there is some uncertainty for 200 MHz, it is proposed to not to mention wider channel bandwidths in the reply LS.

Proposal 1: It is proposed to not to mention wider channel bandwidths in the reply LS.

The supported channel bandwidths are defined per operating band. For the band operation in the range 7125 to 8400 MHz, the total band range is 1275 MHz which is larger than that of existing FR1 bands. 4% the relative channel bandwidth is 310 MHz. We think 200 MHz could be max channel bandwidth for the band. In existing FR1 specifications, the max channel bandwidth is 100 MHz and introduction of wider channel bandwidth would need the study on the spectrum utilization of the new channel bandwidth and corresponding BS and UE transmitter and receiver requirements. Furthermore, carrier aggregation will be defined for the operator hold wide spectrum, e.g. 400~800 MHz bandwidth.

Proposal 2: Describe 200 MHz as a potential wider channel bandwidth in the TR

Issue 2-4: Typical signal bandwidth

Outcome of discussion (Not agreed):

Option 1: 273 RB, 30k SCS as 38.104

Option 2: Quote formula of RBs * SCS without number of RBs

Option 3: Directly quote the signal bandwidth in MHz

The transmission bandwidth configuration defined in BS/UE core spec can be used to derive the signal bandwidth. The signal bandwidth is calculated based on the NR spectrum utilization for each SCS:

Signal bandwidth = NRB x SCS x 12

with NRB: Number of Resource block for 100 MHz channel bandwidth and 30kHz SCS, as specified in TS 38.104.

Proposal 3: Option 2 is adopted: quote formula of RBs * SCS without number of RBs

BS Transmitter characteristics

Issue 2-7: Emissions mask

Agreement:

- AAS BS:

  - Use n104 unwanted emissions mask

  - [Aim to use 100MHz for delta_obue, conclude feasibility until RAN4#112]

- Non-AAS BS:

  - [n104 unwanted emissions mask for IMT response]

    - Double check on whether there are any concerns towards sharing by RAN4#112 and in case concerns are identified, consider n96 if needed to resolve concerns

The frequency range 7125 to 8400 MHz is just adjacent to existing NR band n104. It is proposed that existing spectral mask for band n104 is applicable for the range. The definition of ΔfOBUE and ΔfOOB is related to the band frequency range. In current stage it is not clear how many bands will be defined for the range 7125 to 8400 MHz. It is foreseen the max band range would be similar as band n104. Hence it is feasible to use the same emission mask as n104 for the frequency range 7125 to 8400 MHz. the feasibility can refer to the study for the SI on 6.425-7.125GHz [4]

Proposal 4: n104 unwanted emissions including ΔfOBUE definition are used for the frequency range 7125 to 8400 MHz.

UE Transmitter characteristics

ACLR:

  - Option 1:  26dB, 27dB (study) for PC3

  - Option 2:  30dB (n104) for PC3, 31dB (n104) for PC2

From the SI for 6 GHz and 10 GHz, it was concluded that 26 dB and 24 dB are applied respectively. Although 30 dB is reused for n104 due to MPR is also reused from other FR1 bands, we prefer to reply WP5D with 26 dB for PC3, which can achieve lower MPR and be beneficial for UL coverage.

Proposal 5: 26 dB ACLR for PC3 UE

Issue 2-15: Maximum output power

Tentative agreement:

- Capture 23dBm as typical maximum output power in the LS to ITU.

  - Other output powers are not precluded and refer to TR

- Capture 20dBm, 23dBm, 26dBm, and higher power classes in TR

Both PC3 and PC2 are defined for 6 GHz band n104. For the purpose of co-existence analysis, the UE maximum/typical output power for the considered frequency ranges could be 23 dBm.

Proposal 6: For the purpose of co-existence analysis, the UE maximum/typical output power for the considered frequency ranges could be 23 dBm.

Receiver characteristics

Noise figure:

  - Option 1: Follow n104 noise figure (12dB)

  - Option 2: Be consistent with information sent previously IMT-2020 28GHz, e.g. 10dB

  - Option 3: Be consistent with Previous LS to ITU-R on 6, 10GHz, NF was 9-13dB

The frequency range 7125 to 8400 MHz is just adjacent to existing NR band n104. It is proposed that existing NF for band n104 is applicable for the range, i.e.12 dB

Proposal 7: Follow n104 noise figure (12dB)

Sensitivity:

The sensitivity is not a critical parameter for sharing and compatibility studies. It was agreed to not mention any value for this parameter.

Blocking response

Proposal 8: The blocking characteristic specified in clause 7.6 of TS 38.101-1 [4] for frequency larger than 3300 MHz could be applied for the range.

2.2 Antenna characteristics

Issue 2-14: BS antenna array parameters

Agreement:

|  |  | Rural macro (If it’s available) Parameters proposed by ZTE | Suburban macro | Urban macro | Urban small cell (outdoor)/Micro cell | Indoor (small cell) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Base station antenna characteristics |  |  |  |  |  |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) | 8 × 8 elements | Option 1: 8 × 16 (Nokia, Ericsson, Huawei, Spark) Option 2: 8 x 8 (ZTE, Samsung) | Option 1: 8 × 16 (Nokia, Ericsson, Huawei, Cable Labs) Option 2: 8 x 8 (ZTE, Samsung) | 8 × 8 | 4 × 4 |
| 1.7a | Number of element rows in sub-array, Msub |  | Option 1: 4 (Nokia, Spark, Cable Labs, Charter) Option 2: 3 (Ericsson, Huawei, Samsung) | Option 1: 4 (Nokia, Spark, Cable Labs, Charter) Option 2: 3 (Ericsson, Huawei, Samsung) | N/A | N/A |

Agreement:

- Remove the rural macro scenario from the table.

- non-AAS can be used for Urban small cell/Micro cell scenario

- non-AAS can be used for indoor scenario

- FFS on where to capture the above two bullets

For the frequency range, comparable coverage as 3.5 GHz should be considered. For this consideration twice the carrier frequency 4 times antenna elements size is proposed. For Macro BS in 1710 to 4990 MHz, antenna array configuration is 4x8 with 3 vertical element in sub-array.  Hence Option 1 (8 × 16) antenna array configuration and Option 2 (3) element rows in sub-array should be suitable.

For indoor small cell, beam forming is not suitable for indoor deployment. The indoor installation is mainly ceiling-mounted. The main requirement is wide beam width on the hemispherical surface, so beam forming does not provide benefits. It is proposed not provide Beamforming antenna characteristics for indoor small cell. Non-AAS would be used for indoor small cell.

The antenna characteristics for IMT in 7125 to 8400 MHz is proposed in following Table 2.2-1.

Table 2.2-1: Beamforming antenna characteristics for IMT in 7125 to 8400 MHz

|  |  | Macro suburban | Macro urban | Small cell outdoor/ Micro urban |
| --- | --- | --- | --- | --- |
| 1 | Base station Antenna Characteristics |  |  |  |
| 1.1 | Antenna pattern | Table 3 |  | ITU-R M.2101 |
| 1.2 | Element gain (dBi) (Note 2) | 6.4 | 6.4 | 6.4 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V | 90º for H 65º for V | 90º for H 65º for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V |
| 1.5 | Antenna polarization | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array |
| 1.6 | Antenna sub-array configuration (Row × Column) (Note 4) | 8 × 16 sub-arrays/elements | 8 × 16 sub-arrays/elements | 8 × 8 elements Or 4 x 4 |
| 1.7 | Horizontal/Vertical radiating sub-array spacing (Note 5) | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 0.7 of wavelength for V |
| 1.7a | Number of element rows in sub-array | 3 | 3 | N/A or 4(3) |
| 1.7b | Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength for V | 0.7 of wavelength for V | N/A |
| 1.7c | Pre-set sub-array down-tilt (degrees) (Note 6) | 3 | 3 | N/A |
| 1.8 | Array Ohmic loss (dB) (Note 2) | 2 | 2 | ## 2 |
| 1.9 | Conducted power (before Ohmic loss) per sub-array (dBm) (Note 3) | 22 | 22 | 16 |
| 1.10 | Base station horizontal coverage range (degrees) | +/-60 | +/-60 | +/-60 |
| 1.11 | Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-100 | 90-120 |
| 1.12 | Mechanical down-tilt (degrees) | 6 | 6 | N/A |
| 1.13 | Maximum base station output power/sector (e.i.r.p.) (dBm) (Note 7) | 78.3 | 78.3 | 61.5 |

Proposal 9: The antenna characteristics for IMT in 7125 to 8400 MHz is proposed in following Table 2.2-1.

- Conclusion

The contribution provides an overview on IMT technology related parameters for frequency range 7125 to 8400 MHz.

Proposal 1: It is proposed to not to mention wider channel bandwidths in the reply LS.

Proposal 2: Describe 200 MHz as a potential wider channel bandwidth in the TR

Proposal 3: Option 2 is adopted: quote formula of RBs * SCS without number of RBs

Proposal 4: n104 unwanted emissions including ΔfOBUE definition are used for the frequency range 7125 to 8400 MHz.

Proposal 5: 26 dB ACLR for PC3 UE

Proposal 6: For the purpose of co-existence analysis, the UE maximum/typical output power for the considered frequency ranges could be 23 dBm.

Proposal 7: Follow n104 noise figure (12dB)

Proposal 8: The blocking characteristic specified in clause 7.6 of TS 38.101-1 [4] for frequency larger than 3300 MHz could be applied for the range.

Proposal 9: The antenna characteristics for IMT in 7125 to 8400 MHz is proposed in following Table 2.2-1.

References

[1]	RP-240787,	New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz, 	Ericsson

[2] R4-2406615, “WF on IMT parameters for other frequency ranges”, CATT

[3] R4-2410741, “WF on IMT parameter study”, Ericsson

[4] R4-2101496, “TP to TR 38.921: remaining BS parameters”, Huawei