3GPP RAN WG4 Meeting #112		R4-2413280

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	Huawei, HiSilicon

Title: 	TP for BS IMT parameters for range 7125 to 8400 MHz

Agenda Item:	8.2.3

Document for:	Approval

- Introduction

In the last plenary (RAN#103) the Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved [1]. For frequency range 7125 to 8400 MHz and 14800 to 15350 MHz, the WF was agreed in [2] at RAN#110bis. And at RAN#111, further agreements were made in [3].

This paper provides TP for TR 38.922 on BS IMT parameters for the range 7125 to 8400 MHz.

References

[1]	RP-240787,	New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz, 	Ericsson

[2] R4-2406615, “WF on IMT parameters for other frequency ranges”, CATT

[3] R4-2410741, “WF on IMT parameter study”, Ericsson

[4] R4-2101496, “TP to TR 38.921: remaining BS parameters”, Huawei

- TP to TR 38.922

<Start of TP>

5.2	BS parameters

5.2.1	Transmitter characteristics

5.2.1.1	Power dynamic range

There is no power control in downlink and fixed power per resource block is assumed during the study phase. Hence 0 dB power dynamic range was agreed for the LS reply.

5.2.1.2	Spectral mask

The frequency range 7125 to 8400 MHz is just adjacent to existing NR band n104. It is proposed that existing spectral mask for band n104 in TS 38.104 subclause 6.6.4 is applicable for the range.

Table 5.2.1.2-1: Wide Area BS operating band unwanted emission limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | -7dBm-750foffsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 100 MHz f fmax | 100.5 MHz f_offset < f_offsetmax | -15 dBm (Note 3) | 1MHz |

5.2.1.3	ACLR

The frequency range 7125 to 8400 MHz, it is agreed to n104 ACLR. The ACLR should be higher thatn the value specified in Table 5.2.1.3-1.

Table 6.6.3.2-1: Base station ACLR limit

| BS channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90,100 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 38 dB |
|  | 2 x BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 38 dB |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). |  |  |  |  |

5.2.1.4	Spurious emissions

The general spurious emissions for band n104 are applicable for the frequency range 7125 to 8400 MHz.

Table 5.2.1.4-1: BS spurious emission limits

| Spurious frequency range | Basic limit | Measurement bandwidth |
| --- | --- | --- |
| 9 kHz – 150 kHz | -36 dBm | 1 kHz |
| 150 kHz – 30 MHz |  | 10 kHz |
| 30 MHz – 1 GHz |  | 100 kHz |
| 1 GHz – 26 GHz | -30 dBm | 1 MHz |

5.2.1.5	Maximum output power

The maximum output power will be provided in the antenna parameter table. It was agreed to be aligned with antenna characteristics.

The Total Radiated Power for two polarizations was agreed as shown in Table 5.2.1.5-1 below.

Table 4.2.1.5-1: The Total Radiated Power

| Parameter | Macro Sub-urban | Macro Urban | Micro Urban |
| --- | --- | --- | --- |
| Total Radiated Power for two polarizations (dBm) | 46 | 46 | 37 |

5.2.1.6	Average output power

It was agreed the average output power won’t be mentioned in the reply LS.

5.2.2	Receiver characteristics

5.2.2.1	Noise figure

For the frequency range 7125 to 8400 MHz, the typical Noise Figure for a Wide Area BS operating was agreed to be 6 dB (11 dB for Medium Range BS and 14 dB for Local Area BS).

5.2.2.2	Sensitivity

The BS reference sensitivity relevant 7125 to 8400 MHz is listed in Figure 5.2.2.2-1, Figure 5.2.2.2-2 and Figure 5.2.2.2-3.

5.2.2.3	Blocking response

The in-band blocking requirement should apply from FUL,low - ΔfOOB to FUL,high + ΔfOOB, excluding the downlink frequency range of the FDD operating band. It is agreed to adopt ΔfOOB = 100 MHz for 7125 to 8400 MHz. The in-band blocking levels are reused from existing FR1 requirements.

Table 5.2.2.3-1: Base station general blocking requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | PREFSENS + 6 dB | Wide Area BS: -43 Medium Range BS: -38 Local Area BS: -35 | ±30 | 20 MHz DFT-s-OFDM NR signal 15 kHz SCS, 100 RBs |
| NOTE: PREFSENS depends on the RAT. |  |  |  |  |

The out-of-band blocking requirement apply from 1 MHz to FUL,low - ΔfOOB and from FUL,high + ΔfOOB up to 12750 MHz for FR1 bands. -15 dBm CW interfering signal is reused for out-of-band blocking for 7125 to 8400 MHz.

5.2.2.4	ACS

It is agreed to specify 42 dB ACS for the frequency range 7125 to 8400 MHz.

<End of TP>